| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O030 Filename: JVT-O030.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Improvement of the update step in JSVM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yihua Chen1, Jizheng Xu2, Feng Wu2, Hongkai Xiong1 1 Shanghai Jiaotong University, China #1954, Huashan Rd, Xuhui Dist Shanghai 200030, P.R.China 2 Microsoft Research Asia #49, Zhichun Rd, Haidian Dist Beijing 100080, P.R.China | Tel: Email: Tel: Email: Tel: Email: | +86-21-62932458 jameschen@sjtu.edu.cn +86-10-62617711 t-jzhxu@microsoft.com +86-10-62617711 fengwu@microsoft.com |
| Source: | Shanghai Jiaotong University, Microsoft Research Asia |  |  |

_____________________________

- Overview

This document presents an alternative update operator, aka. Energy Distributed Update [3], for JSVM [1]. Update operator plays an important role in the lifting implementation of MCTF and can improve the coding performance significantly. Most update schemes proposed up to now focus on the inversion of the motion field obtained in the prediction step. Given the fact that not all motion vectors in the update step can be correctly derived from those in the prediction step, the inherent mismatch in these schemes may lead to potential loss of coding performance. The one adopted by JSVM 1.0 [2] follows the same principle by inversing the motion vector with maximum cover ratio for each 4x4 block. An adaptive weight is also applied to reduce potential visual artifacts and improve objective quality for low frame rate sub-sequences. However, this update algorithm requires two steps: first, the motion field obtained in the prediction step is scanned to derive the inverse motion field; then, the update component is obtained by motion compensation. Moreover, the application of adaptive weighting requires additional computation.

The update operator proposed in this document circumvents the step of inversing the motion field and enables deriving the update component by one scan of the motion field used in the prediction step. For the same purpose with applying adaptive weighting, simple thresholds are used instead. The implementation issues are addressed in section 2 and section 3 gives experimental results.

- Implementation of Energy Distributed Update in JSVM

The motivation of Energy Distributed Update is introduced in [3]; here we only describe the corresponding algorithm implemented in JSVM 1.0. We denote the pixel value at position x in the ith frame of decomposition level j as

Xi(j)(x),  i=1,⋯,2n,  n=2−j−1m,   j=0,⋯,log2m−1,

where m is the GOP size. Then the prediction step for Haar wavelet (unidirectional prediction) can be expressed as

Hk(j)(x)=X2k−1(j)(x)−X2k−2(j)(x+mv(x)),

or

Hk(j)(x)=X2k−1(j)(x)−X2k(j)(x+mv(x)).

For 5/3 wavelet (bidirectional prediction), we have

Hk(j)(x)=X2k−1(j)(x)−12X2k−2(j)(x+mv(x))+X2k(j)(x+mv(x)),

where mv(x) and mv(x) are the forward and backward motion vectors at positionx, respectively. As an extension of H.264, JSVM uses the same spatial interpolation filters. Therefore, the prediction step for Haar wavelet can be further expressed as

Hk(j)(x)=X2k−1(j)(x)−y∈Ω2k−2αb,2k−1(x,y)X2k−2(j)(y),

or

Hk(j)(x)=X2k−1(j)(x)−y∈Ω2kαf,2k−1(x,y)X2k(j)(y),

while for 5/3 wavelet, we have

Hk(j)(x)=X2k−1(j)(x)−12y∈Ω2k−2αb,2k−1(x,y)X2k−2(j)(y)+y∈Ω2kαf,2k−1(x,y)X2k(j)(y),

where Ωi represents all the pixels at integer sampling positions in the ith frame. Here αb,i(x,y) and αf,i(x,y) are the corresponding interpolation coefficients determined by mv(x) and mv(x). Actually, these coefficients are non-zero only at a small neighborhood of x+mvb,i(x) and x+mvf,i(x), respectively.

Using Energy Distributed Update, the update operator is formulated as follows,

Lk(j)(y)=X2k(j)(y)+14x∈Ω2k−1αf,2k−1(x,y)Hk(j)(x)+x∈Ω2k+1αb,2k+1(x,y)Hk+1(j)(x).

Here we use a simple example to shed light on the above equation. Assuming simple bilinear interpolation filter is used, according to Fig. 1, we have

Hk(2)=X2k−1(2)−1212X2k−2(2)+12X2k−2(3)+14X2k(1)+34X2k(2).

Then from the viewpoint of Energy Distributed Update, 18Hk(2), 18Hk(2), 116Hk(2) and 316Hk(2) will be added to X2k−2(2), X2k−2(3), X2k(1) and X2k(2) respectively as part of their update components.

Figure 1. An example of proposed update operator.

For the proposed update operator, the update component is

Uk(j)(y)=14x∈Ω2k−1αf,2k−1(x,y)Hk(j)(x)+x∈Ω2k+1αb,2k+1(x,y)Hk+1(j)(x).

In order to reduce potential visual artifacts and improve objective quality for low frame rate sub-sequences, a positive decomposition level adaptive threshold THj is applied on the update component. Therefore, the actual update component is

Uk(y)=−THj,                   Uk(j)(y)≤−THjUk(j)(y),          −THj<Uk(j)(y)<THjTHj,                      Uk(j)(y)≥THj,

and the low-pass frames are ultimately obtained by

Lk(j)(y)=X2k(j)(y)+Uk(y).

The above derivation of update component can be easily extended to the situation of multiple references.

In our implementation, those non-zero coefficients αb,i(x,y) and αf,i(x,y) are obtained by a small look-up table derived from the 6-tap filter used in H.264, while the thresholds can be transmitted as sequence or picture parameters with negligible influence on the overall bit rate. Note that other simple interpolation filter (e.g. bilinear filter) can also be used to get the look-up table, which may further reduce the computational complexity.

- Experimental Results

We modified JSVM 1.0 and ran most of the Palma test points [4] using the configuration files provided along with the reference software. The experimental results are demonstrated in the accompanying datasheet (JVT-O030.xls).

The preliminary results clearly show that the proposed update operator, which is of less complexity than the current one in JSVM 1.0, can achieve small improvement on the quality of the luminance component of many test sequences. Among the four CIF sequences, “Bus” and “Mobile” enjoy constant gain, which reaches 0.1 – 0.15 dB at high bit rate; the performance on the other two is almost the same with the current update operator. In addition, from the per-frame PSNR curves of “Bus” and “Mobile” (also included in the datasheet), we observe that the proposed update operator significantly reduces quality fluctuation, which is apparently more appealing to human visual system. However, despite the gain achieved on the luminance component, we also notice that the quality of the chrominance components of the two 4CIF sequences suffers from minor loss. This may be due to the relatively high thresholds used in testing and requires further investigation.

- Conclusion

In this document, an alternative yet effective update operator is presented. In comparison with the  current update operator in JSVM 1.0, the complexity of the proposed one is just equal to the motion compensation step in the current scheme, while that of collecting connection data for inversing motion field (which requires additional memory for storing connection data and motion vectors) and computing adaptive weight is totally eliminated. The preliminary experimental results, especially those obtained from the four CIF sequences, demonstrate the prospect of the proposed update operator by showing that with reduced computational complexity, the proposed update operator is still able to achieve higher or at least similar performance on most sequences, while at the same time, reduce quality fluctuation.

- References

- “JSVM 1.0 Software,” ISO/IEC JTC1/SC29/WG11/N6900, Jan. 2005, Hong Kong, China.

- “Joint Scalable Video Model (JSVM) 1.0 Reference Encoding Algorithm Description,” ISO/IEC JTC1/SC29/WG11/N6899, Jan. 2005, Hong Kong, China.

- Bo Feng, et al., “Energy distributed update step (EDU) in lifting based motion compensated video coding,” ICIP’04, Oct. 2004.

- “Description of Core Experiments in MPEG-21 Scalable Video Coding,” ISO/IEC JTC1/SC29/WG11/N6521, July 2004, Redmond, USA.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: