| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, Korea 16-22 Apr., 2005 | Document: JVT-O038 Filename: JVT-O038.doc |
| --- | --- |

| Title: | Report on Core experiment 9 : Non Dyadic Spatial Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Francois, J. Viéron, G. Marquant, N. Burdin, P. Lopez THOMSON R&D France, Video Compression Lab, Corporate Research Rennes, 1, Avenue Belle Fontaine – CS17617, 35 576 Cesson Sevigne Cedex – France | Tel: Email: | +33 2 99 27 32 62 edouard.francois@thomson.net |
| Source: | Thomson |  |  |

_____________________________

# Introduction

This document reports the results of CE9 related to Spatial scalable coding using cropped areas. The main goal of this core experiment is to address the cropping feature, as described by the spatial scalability requirement, [N6880]: “enlarging the spatial region coded from one spatial layer to its enhancement, for example by encoding additional bordering areas”.

Coordinator: Thomson

| Proponent | Partner | Contact | Email |
| --- | --- | --- | --- |
| Thomson | VisioWave | E.Francois | edouard.francois@thomson.net |

# CE9 description

For this CE, the following test conditions have been considered:

The cropping area is defined as follows:

- upper left pixel located at pixel (line 16, column 32) of high layer pictures

- window size in high layer pictures = 512 lines x 640 columns

The base layer pictures are a downsampled version of the high layer cropped window, with dimensions =  256 lines x 320 columns.

Bitrates for each layer are defined as follows.

| bitrates (kbps) | Low layer 320*256 | High layer 704*576 |
| --- | --- | --- |
| City | 416 | 1024 |
| Crew, Harbour, Soccer | 640 | 1500 |

Table 1 : tests conditions

In order to check that the tool is functional that inter-layer prediction still brings quality improvements, the two following sets of simulations have been performed:

- test1 : cropping with full inter layer prediction (proposal [JVT-O040])

- test2 : cropping without inter layer motion and texture residue prediction

# Results

Full results can be found in the following file :

The following table and figure summarize these results:

|  | real bitrate | psnr Y | psnr U | psnr V | delta bitrate | real bitrate | psnr Y | psnr U | psnr V | delta bitrate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CITY | low layer – 416 kbps |  |  |  |  | high layer – 1024 kbps |  |  |  |  |
| with interlayer pred | 417,42 | 37,48 | 45,09 | 46,74 | 0,34% | 1033,54 | 34,28 | 42,79 | 45,03 | 0,93% |
| without interlayer pred | 417,42 | 37,48 | 45,09 | 46,74 | 0,34% | 1039,50 | 33,49 | 41,97 | 44,07 | 1,51% |

| CREW | low layer – 640 kbps |  |  |  |  | high layer – 1500 kbps |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| with interlayer pred | 629,61 | 36,06 | 40,50 | 39,38 | -1,62% | 1490,15 | 35,24 | 40,39 | 40,50 | -0,66% |
| without interlayer pred | 629,61 | 36,06 | 40,50 | 39,38 | -1,62% | 1523,62 | 33,81 | 39,61 | 39,33 | 1,57% |

| HARBOUR | low layer – 640 kbps |  |  |  |  | high layer – 1500 kbps |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| with interlayer pred | 629,25 | 32,71 | 42,50 | 44,14 | -1,68% | 1522,69 | 31,44 | 41,60 | 43,65 | 1,51% |
| without interlayer pred | 629,25 | 32,71 | 42,50 | 44,14 | -1,68% | 1509,95 | 29,82 | 40,31 | 41,94 | 0,66% |

| SOCCER | low layer – 640 kbps |  |  |  |  | high layer – 1500 kbps |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| with interlayer pred | 636,13 | 36,88 | 43,24 | 45,07 | -0,60% | 1494,76 | 34,98 | 42,74 | 44,75 | -0,35% |
| without interlayer pred | 636,13 | 36,88 | 43,24 | 45,07 | -0,60% | 1503,27 | 33,62 | 41,97 | 43,61 | 0,22% |

Table 2 : PSNR results

Figure 1 : PSNR results

Visually, a significant quality improvement can be observed when using motion and residue interlayer prediction.

# Conclusions and recommendations

The proposed tool has been proved to fulfill the cropping functionality. Consequently we recommend to adopt this tool in the JSVM.

# References

[N6880]	ISO/IEC JTC1/SC29/WG11/N6880, Hong Kong, January 2005, Applications and Requirements for Scalable Video Coding

[JVT-O040]	E. Francois, J. Viéron, G. Marquant, N. Burdin, P. Lopez, “Spatial scalable coding using cropped areas”, Joint Video Team of ISO/MPEG and ITU-T VCEG, Doc. JVT- O040, Busan, Korea, April 05.