| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O045 Filename: JVT-O045.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Modification of the calculation of Scaling Factors in the J-SVM |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JVT] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Isabelle AMONOU Nathalie CAMMAS Sylvain KERVADEC Stéphane PATEUX France Télécom 4 Rue du Clos Courtel 35510 Cesson-Sévigné France | Tel: +33 2 99 12 44 77 Email: isabelle.amonou@francetelecom.com nathalie.cammas@francetelecom.com sylvain.kervadec@francetelecom.com stephane.pateux@francetelecom.com |  |
| Source: | [France Télécom] |  |  |

_____________________________

# Introduction

This document presents a modification in the calculation of the adaptive quantizer settings applied to the temporal subbands in the J-SVM. The modification applies on the scaling factor used for the spatial base layer coding when no update is done.

2 Modification of the scaling factor

In the current version of the JSVM, the quantizer settings for the temporal subbands are adjusted according to the fractions of uni- and bi-directional connections  and their respective scaling factors ( [1], section 4.5). In the JSVM software, these scaling factors are the followings:

#define FACTOR_22_HP  0.70710678118654752440084436210485  //sqrt( 1.0/ 2.0)

#define FACTOR_22_LP  1.4142135623730950488016887242097   //sqrt( 2.0/ 1.0)

#define FACTOR_53_HP  0.847791247890659  //sqrt( 23.0/ 32.0)

#define FACTOR_53_LP  1.22474487139159   //sqrt(3.0/2.0)

These scaling factors are used to calculate the scaling factors applied to each temporal subband for all spatial layers. But, in the spatial base layer case (QCIF), no update step is performed to maintain an H264 MPEG-AVC compatibility. For this lower layer, the 2/2 and 5/3 resulting filters applied on the QCIF frames are different from the 2/2 and 5/3 filters applied on the frames for other spatial layers.

In the table below we give the resulting 2/2 and 5/3 filters applied when the update step is performed and when no update step is performed:

| Filter | Impulse Response | Energy |
| --- | --- | --- |
| 22 HP | ½ ½ | ½ |
| 22 LP | 1 1 | 2 |
| 22 HP (no update) | 1 | 1 |
| 53 HP | -1/8 -1/4 ¾ -1/4 -1/8 | 23/32 |
| 53 LP | -1/2 1 -1/2 | 3/2 |
| 53 HP (no update) | 1 | 1 |

In consequence, as the quantizer step size for a given subband should be chosen to be inversely proportional to the square-root of the energy of the impulse response of the synthesis filter, we propose to modify the scaling factors for the 2/2 and 5/3 filters in the case when no update step is performed in the JSVM software as follows:

#define FACTOR_53_HP_BL 1.0 //Scaling Factor for Base Layer (when no update is done)

#define FACTOR_22_HP_BL 1.0 //Scaling Factor for Base Layer (when no update is done)

# 3 Results

PSNR results for each temporal resolution of each spatial scale can be found in the attached Excel file.

For this set of tests, we use the configuration files provided with the J-SVM 1.0 software and we use the J-SVM 1.0 software.

For this set of tests, we modified the scaling factors of the base layer temporal subbands only. It can be seen that the modification leads to an improved PSNR. A PSNR gains of up to 0.25dB can be observed.

Gain is less important at 4CIF resolution than at CIF or at QCIF. The modification essentially impacts on the coding of the QCIF resolution. The improvement on QCIF leads to an improvement on CIF and 4CIF as these resolutions are predictively coded. But, the proportion of QCIF used at 4CIF is lower than at CIF and so limits the improvement at 4CIF.

We can note that for the modified J-SVM, all bitrates can not be reached, especially the lower bitrates. This is due to the choice of the initial QP used for coding. As we used the same configuration files, we did not modify the initial QP. But, the modification of the calculation of the scaling factor changes the amplitude of the values to code and so, the initial QP used may be no more appropriated. It would be necessary to find the new initial QP values which allow to reach the low bitrates.

Rate/distortion curves for the sequence Bus

Rate/distortion curves for the sequence Football

Rate/distortion curves for the sequence Foreman

Rate/distortion curves for the sequence Mobile

Rate/distortion curves for the sequence City

Rate/distortion curves for the sequence Crew

Rate/distortion curves for the sequence Harbour

Rate/distortion curves for the sequence Soccer

PSNR Gain

The tables below show the PSNR gain obtained between the new calculation of the scaling factor and the original J-SVM 1.0. The bitrates given are the bitrates generated by the original J-SVM 1.0 (ORIG) and the bitrates generated by the J-SVM 1.0 modified with the calculation of the scaling factor (HP_BL).

The PSNR gain have been obtained by computing the difference between the PSNR given by the J-SVM 1.0 and the modified J-SVM 1.0 for each bitrate. When the difference between the generated bitrates is more than 1%, no PSNR gain has been calculated.

PSNR gains for the sequence Bus

| ORIG-Bitrates | HP_BL-Bitrates | PSNR Gain |
| --- | --- | --- |
| QCIF7.5 | QCIF7.5 |  |
| 69,4626 | 79,8111 | rate too different |
| 79,9689 | 79,8111 | 0,2496 |
| 95,9684 | 95,9668 | 0,0463 |
| 101,2295 | 99,0063 | rate too different |
| 101,2295 | 99,0063 | rate too different |
| QCIF15 | QCIF15 |  |
| 95,9968 | 102,5856 | rate too different |
| 111,9504 | 111,9408 | 0,094 |
| 127,9936 | 127,9936 | -0,0197 |
| 127,9936 | 127,9936 | -0,0197 |
| 127,9936 | 127,9936 | -0,0197 |
| CIF7.5 | CIF7.5 |  |
| 143,6037 | 153,5779 | rate too different |
| 159,9947 | 159,9963 | 0,0366 |
| 191,9716 | 191,9653 | 0,0446 |
| 223,9721 | 223,9658 | 0,0664 |
| 255,9742 | 255,9695 | 0,0718 |
| CIF15 | CIF15 |  |
| 191,9952 | 192,4832 | -0,0206 |
| 223,64 | 223,9136 | 0,0184 |
| 255,9408 | 255,936 | 0,0383 |
| 319,9424 | 319,944 | 0,0419 |
| 383,9344 | 383,9376 | 0,0579 |
| CIF30 | CIF30 |  |
| 255,9872 | 255,9856 | 0,0144 |
| 319,8784 | 319,8704 | 0,0427 |
| 383,8848 | 383,8784 | 0,0441 |
| 447,88 | 447,88 | 0,0523 |
| 511,984 | 511,9824 | 0,0452 |

PSNR gains for the sequence Football

| ORIG-Bitrates | HP_BL- Bitrates | PSNR gain |
| --- | --- | --- |
| QCIF7.5 | QCIF7.5 |  |
| 138,4422 | 157,32 | rate too different |
| 157,4732 | 157,32 | 0,199 |
| 176,532 | 171,1338 | rate too different |
| 176,532 | 171,1338 | rate too different |
| 176,532 | 171,1338 | rate too different |
| QCIF15 | QCIF15 |  |
| 199,1649 | 234,4791 | rate too different |
| 223,9458 | 234,4791 | rate too different |
| 255,9942 | 255,9942 | 0,1154 |
| 255,9942 | 255,9942 | 0,1154 |
| 255,9942 | 255,9942 | 0,1154 |
| CIF7.5 | CIF7.5 |  |
| 279,1542 | 295,6745 | rate too different |
| 319,9422 | 319,9652 | 0,0812 |
| 383,9705 | 383,9714 | 0,1111 |
| 447,9748 | 447,9692 | 0,1435 |
| 511,9717 | 511,9698 | 0,1219 |
| CIF15 | CIF15 |  |
| 394,6135 | 422,7535 | rate too different |
| 447,9711 | 447,8372 | 0,0384 |
| 511,9348 | 511,9375 | 0,0896 |
| 639,9378 | 639,9378 | 0,114 |
| 767,94 | 767,9446 | 0,1077 |
| CIF30 | CIF30 |  |
| 524,9732 | 553,0449 | rate too different |
| 639,8548 | 639,8585 | 0,0658 |
| 767,8791 | 767,8745 | 0,0914 |
| 895,8803 | 895,8785 | 0,0878 |
| 1023,984 | 1023,9831 | 0,0745 |

PSNR gains for the sequence Foreman

| ORIG-Bitrates | HP_BL-Bitrates | PSNR gain |
| --- | --- | --- |
| QCIF7.5 | QCIF7.5 |  |
| 34,1728 | 39,0656 | rate too different |
| 39,9368 | 39,8304 | 0,1438 |
| 47,9384 | 47,9296 | 0,0288 |
| 55,9664 | 55,9456 | -0,0837 |
| 63,9672 | 63,9688 | -0,0139 |
| QCIF15 | QCIF15 |  |
| 44,288 | 50,0712 | rate too different |
| 55,7656 | 55,768 | 0,0669 |
| 63,8376 | 63,8504 | -0,0565 |
| 79,9928 | 79,9928 | -0,0648 |
| 79,9928 | 79,9928 | -0,0648 |
| CIF7.5 | CIF7.5 |  |
| 70,6384 | 74,7872 | rate too different |
| 79,996 | 79,9968 | 0,1335 |
| 95,992 | 95,928 | 0,1145 |
| 111,9096 | 111,9528 | 0,1124 |
| 127,9544 | 127,9616 | 0,1309 |
| CIF15 | CIF15 |  |
| 95,996 | 99,6216 | rate too different |
| 111,996 | 111,996 | 0,2088 |
| 127,8552 | 127,9 | 0,0707 |
| 159,8864 | 159,9064 | 0,1179 |
| 191,9416 | 191,9392 | 0,1149 |
| CIF30 | CIF30 |  |
| 127,9952 | 127,9952 | 0,111 |
| 159,9232 | 159,4616 | 0,059 |
| 191,5312 | 191,8768 | 0,1059 |
| 223,892 | 223,892 | 0,1036 |
| 255,9824 | 255,9832 | 0,0802 |

PSNR gains for the sequence Mobile

| ORIG-Bitrates | HP_BL-Bitrates | PSNR Gain |
| --- | --- | --- |
| QCIF7.5 | QCIF7.5 |  |
| 51,8152 | 57,2624 | rate too different |
| 55,968 | 57,2624 | rate too different |
| 63,9696 | 63,9688 | 0,0536 |
| 79,968 | 79,9736 | -0,0645 |
| 84,0312 | 81,5496 | rate too different |
| QCIF15 | QCIF15 |  |
| 59,7936 | 67,332 | rate too different |
| 79,9424 | 79,9448 | -0,046 |
| 95,9928 | 95,992 | -0,1224 |
| 95,9928 | 95,992 | -0,1224 |
| 95,9928 | 95,992 | -0,1224 |
| CIF7.5 | CIF7.5 |  |
| 120,3568 | 124,4744 | rate too different |
| 120,3568 | 124,4744 | rate too different |
| 127,996 | 127,996 | -0,0057 |
| 159,9704 | 159,968 | 0,0538 |
| 191,972 | 191,9704 | 0,0875 |
| CIF15 | CIF15 |  |
| 143,1552 | 148,4192 | rate too different |
| 159,9952 | 159,9952 | 0,0272 |
| 191,9256 | 191,936 | 0,0168 |
| 223,9368 | 223,9424 | 0,0365 |
| 255,9392 | 255,94 | 0,0457 |
| CIF30 | CIF30 |  |
| 191,9856 | 191,9856 | -0,0127 |
| 223,5288 | 223,5952 | 0,0088 |
| 255,8672 | 255,864 | 0,019 |
| 319,884 | 319,8824 | 0,0458 |
| 383,9848 | 383,984 | -0,0082 |

PSNR gains for the sequence City

| ORIG-Bitrates | HP_BL-Bitrates | PSNR Gain |
| --- | --- | --- |
| QCIF15 | QCIF15 |  |
| 65,2456 | 71,2656 | rate too different |
| 79,94 | 79,9384 | 0,0214 |
| 95,9384 | 95,9408 | -0,0238 |
| 111,8456 | 111,9448 | -0,4234 |
| 127,9936 | 127,9928 | 0,271 |
| CIF7.5 | CIF7.5 |  |
| 135,3312 | 138,5776 | rate too different |
| 159,9952 | 159,996 | -0,0231 |
| 191,7296 | 191,7712 | 0,066 |
| 223,9688 | 223,9672 | 0,0613 |
| 255,972 | 255,9704 | 0,0812 |
| CIF15 | CIF15 |  |
| 191,9968 | 191,996 | 0,002 |
| 223,8312 | 223,6024 | 0,0575 |
| 255,9432 | 255,9352 | 0,0511 |
| 319,9424 | 319,9384 | 0,0842 |
| 383,944 | 383,9352 | 0,0232 |
| CIF30 | CIF30 |  |
| 255,1144 | 255,0496 | 0,0419 |
| 319,8776 | 319,8752 | 0,0535 |
| 383,0224 | 382,992 | 0,0203 |
| 447,8808 | 447,8768 | 0,0243 |
| 498,124 | 498,3544 | 0,0246 |
| 4CIF15 | 4CIF15 |  |
| 593,8784 | 598,0328 | 0,0336 |
| 639,9928 | 639,7592 | -0,0017 |
| 767,996 | 767,9968 | 0,0197 |
| 895,9688 | 895,9648 | 0,0019 |
| 1023,9352 | 1023,9368 | 0,002 |
| 4CIF30 | 4CIF30 |  |
| 767,9696 | 767,9664 | -0,0034 |
| 895,996 | 895,996 | 0,0216 |
| 1023,944 | 1023,9392 | 0,002 |
| 1279,8808 | 1279,8784 | 0,0023 |
| 1535,872 | 1535,876 | 0,0024 |
| 4CIF60 | 4CIF60 |  |
| 1023,9952 | 1023,996 | 0,0181 |
| 1279,7656 | 1279,764 | 0,0016 |
| 1535,7568 | 1535,76 | 0,0011 |
| 1791,6232 | 1791,7104 | 0,0004 |
| 2047,9688 | 2047,9688 | 0,0017 |

PSNR gains for the sequence Crew

| ORIG-Bitrates | HP_BL-Bitrates | PSNR Gain |
| --- | --- | --- |
| QCIF15 | QCIF15 |  |
| 98,3608 | 109,9544 | rate too different |
| 111,9216 | 111,5896 | 0,0726 |
| 127,9376 | 127,9408 | 0,0157 |
| 159,9376 | 159,9392 | -0,0424 |
| 191,9952 | 191,4784 | 0,0166 |
| CIF7.5 | CIF7.5 |  |
| 191,9832 | 191,984 | 0,0032 |
| 223,9856 | 223,9848 | 0,1127 |
| 255,9712 | 255,9656 | -0,0005 |
| 319,9696 | 319,972 | 0,0075 |
| 383,9696 | 383,9696 | 0,0263 |
| CIF15 | CIF15 |  |
| 255,9824 | 255,9848 | 0,0673 |
| 319,9296 | 319,8904 | 0,0137 |
| 383,9416 | 383,9432 | 0,0145 |
| 447,936 | 447,936 | 0,0243 |
| 511,94 | 511,9384 | 0,0194 |
| CIF30 | CIF30 |  |
| 383,4568 | 383,9656 | 0,0075 |
| 447,8776 | 447,8768 | 0,0151 |
| 511,8752 | 511,8736 | 0,0229 |
| 639,8832 | 639,8792 | 0,0131 |
| 748,8304 | 745,7224 | -0,0276 |
| 4CIF15 | 4CIF15 |  |
| 811,4952 | 827,7704 | rate too different |
| 895,972 | 895,9712 | 0,0069 |
| 1023,9832 | 1023,9864 | 0 |
| 1279,94 | 1279,9408 | 0,0086 |
| 1535,9408 | 1535,9376 | 0,0092 |
| 4CIF30 | 4CIF30 |  |
| 1076,2536 | 1092,6552 | rate too different |
| 1279,9856 | 1279,9824 | -0,003 |
| 1535,9392 | 1535,936 | -0,0046 |
| 1791,876 | 1791,8792 | 0,0053 |
| 2047,8848 | 2047,8896 | 0,0055 |
| 4CIF60 | 4CIF60 |  |
| 1535,9832 | 1543,8376 | 0,0805 |
| 1779,9856 | 1779,9872 | 0,02 |
| 2046,9712 | 2046,8248 | 0,0058 |
| 2559,7648 | 2559,768 | 0,0057 |
| 3071,9688 | 3071,9688 | 0,0018 |

PSNR gains for the sequence Harbour

| ORIG-Bitrates | HP_BL-Bitrates | PSNR Gain |
| --- | --- | --- |
| QCIF15 | QCIF15 |  |
| 96,0784 | 117,6792 | rate too different |
| 111,9328 | 117,6792 | rate too different |
| 127,936 | 127,9368 | 0,1085 |
| 159,9328 | 159,9376 | -0,0473 |
| 191,9936 | 191,9928 | -0,3535 |
| CIF7.5 | CIF7.5 |  |
| 209,9192 | 230,8848 | rate too different |
| 223,9968 | 230,8848 | rate too different |
| 255,992 | 255,9928 | 0,0788 |
| 319,9712 | 319,9704 | -0,0167 |
| 383,9712 | 383,9712 | -0,012 |
| CIF15 | CIF15 |  |
| 255,9952 | 274,4576 | rate too different |
| 319,9888 | 319,984 | 0,0255 |
| 383,9416 | 383,9384 | -0,044 |
| 447,9464 | 447,9384 | -0,0392 |
| 511,9384 | 511,9432 | -0,029 |
| CIF30 | CIF30 |  |
| 383,9224 | 383,6064 | -0,0465 |
| 447,9424 | 447,9432 | -0,036 |
| 511,9432 | 511,9408 | -0,0257 |
| 639,6928 | 639,9696 | -0,0257 |
| 686,3704 | 683,2728 | -0,0391 |
| 4CIF15 | 4CIF15 |  |
| 907,9784 | 936,964 | rate too different |
| 907,9784 | 936,964 | rate too different |
| 1023,9968 | 1023,996 | -0,1074 |
| 1279,984 | 1279,9848 | -0,0107 |
| 1535,94 | 1535,9352 | -0,0035 |
| 4CIF30 | 4CIF30 |  |
| 1127,8576 | 1156,7736 | rate too different |
| 1279,996 | 1279,996 | -0,0809 |
| 1535,9688 | 1535,9712 | -0,0149 |
| 1791,8752 | 1791,8824 | -0,0007 |
| 2047,8752 | 2047,8872 | -0,0047 |
| 4CIF60 | 4CIF60 |  |
| 1535,996 | 1535,996 | -0,0888 |
| 1779,8272 | 1779,9688 | 0,001 |
| 2047,7496 | 2047,7528 | -0,0015 |
| 2559,7504 | 2559,756 | -0,0038 |
| 3071,9688 | 3071,9696 | -0,0031 |

PSNR gains for the sequence Soccer

| ORIG-Bitrates | HP_BL-Bitrates | PSNR Gain |
| --- | --- | --- |
| QCIF15 | QCIF15 |  |
| 99,5312 | 114,5912 | rate too different |
| 102,2272 | 114,5912 | rate too different |
| 102,2272 | 114,5912 | rate too different |
| 102,2272 | 114,5912 | rate too different |
| 102,2272 | 114,5912 | rate too different |
| CIF7.5 | CIF7.5 |  |
| 234,2 | 243,9992 | rate too different |
| 234,2 | 243,9992 | rate too different |
| 255,9912 | 255,9928 | 0,1061 |
| 319,9832 | 319,9856 | 0,048 |
| 383,984 | 383,984 | 0,0572 |
| CIF15 | CIF15 |  |
| 304,3944 | 319,8576 | rate too different |
| 319,9944 | 319,8664 | 0,0433 |
| 383,9688 | 383,9696 | 0,0699 |
| 447,9856 | 447,984 | 0,0366 |
| 494,5504 | 497,0528 | 0,0558 |
| CIF30 | CIF30 |  |
| 383,9904 | 396,7448 | rate too different |
| 447,9848 | 447,9848 | 0,085 |
| 511,988 | 511,984 | 0,0218 |
| 571,4504 | 573,94 | 0,0421 |
| 571,4504 | 573,94 | 0,0421 |
| 4CIF15 | 4CIF15 |  |
| 1090,756 | 1103,6864 | rate too different |
| 1090,756 | 1103,6864 | rate too different |
| 1090,756 | 1103,6864 | rate too different |
| 1279,9904 | 1279,9912 | 0,0259 |
| 1535,9368 | 1535,9432 | 0,0038 |
| 4CIF30 | 4CIF30 |  |
| 1345,8368 | 1358,7976 | 0,2591 |
| 1345,8368 | 1358,7976 | 0,2591 |
| 1535,9928 | 1535,9928 | 0,0233 |
| 1791,8792 | 1791,8736 | 0,0036 |
| 2047,8808 | 2047,8856 | 0,0023 |
| 4CIF60 | 4CIF60 |  |
| 1642,796 | 1655,6472 | 0,2417 |
| 1779,9928 | 1779,9928 | 0,0212 |
| 2046,7632 | 2046,9784 | 0,0007 |
| 2559,7592 | 2559,7672 | 0,0006 |
| 3071,9696 | 3071,9696 | 0,0056 |

References

[1] "Joint Scalable Video Model JSVM 1", Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6), 14th meeting: Hong-Kong, CN, 17-21 January, 2005, document JVT-N021.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: