| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O066 Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Rate-distortion constrained estimation of quantization offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information / Proposal |  |  |
| Author(s) or Contact(s): | Thomas Wedi and Steffen Wittmann Panasonic R&D Center Germany Monzastr. 4c 63225 Langen, Germany | Tel: Email: | +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com Steffen.Wittmann@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Introduction

In this contribution we propose an additional quantization offset parameter in the quantization process of the transform coefficients and address the adaptive adjustment of different quantization offsets. In this context, we distinguish between two different quantization offsets:

- Rounding offset (denoted with f) used in the encoding quantization process only.	
(Constant f already used in JM)

- Additional representative/reconstruction value offset (denoted with ) in the encoding and decoding quantization process. (Proposal)

In previous contributions [1], [2], [3] the quantization-offsets are selected by subjective evaluation and focused the investigations on HD movie sequences containing film grain. The result was a subjective picture quality improvement that is achieved by the introduction of .

Contribution [4] describes a method of adaptive adjustment of a rounding offset f used in the encoding quantization process for scalar quantization (not including ).  The method is based on adjusting the rounding offset to maintain an equal expected value for the input and output of the quantization process for the absolute value of the quantized data. Thus, the adjustment is based on a distortion cost-function and does not consider the rate.

In this contribution we extend the quantization scheme by  and jointly adjust f and  using a rate-distortion constrained cost function. We compare the results with JM as well as with an f-only adjustment. First results indicate rate distortion gains at higher rates.

Although, the results in this contribution are based on JM software we propose to start a core experiment within SVC. We think that the adaptation of the quantization may lead to even bigger improvements for SVC than in JM because of the following reasons:

- In FGS several quantization processes are involved that quantize signals, which have different statistics. The quantization offsets can be adapted to these statistics.

- The images of different MCTF layers have different statistics that can be considered in the adaptive selection of quantization offsets.

# Quantization offsets

## Principle of H.264/AVC Quantization Scheme

The main principle of the H.264/AVC quantization can be expressed with equation (1) that describes the quantization and with equation (2) that describes the inverse quantization.

Z=intW+fΔ⋅sgnW	

W'=Δ⋅Z	

In equation (1), the input signal W is mapped to a quantization level Z. Therefore, the quantization step-size  and the rounding control parameter f is used. The function int() rounds to the nearest integer towards minus infinity and sgn() returns the sign of a signal. The mapping of the quantization level Z to the signal W’ is described in the inverse quantization step that is given by equation (2). Note that in (2), the parameter f is not considered. The following Figure 1 visualizes the quantization and inverse quantization of H.264/AVC for two different values of  f.

Figure 1: Visualization of the quantization and inverse quantization in H.264/AVC for two different parameters f.

The two graphs in Figure 1 show the mapping of the signal W to the representative values W’ for two different f. By modifying f, the positions of the decision level can be controlled. If f=/2, the representative values W’ are located in the center of a quantization interval (see top graph of Figure 1). In the reference model of H.264/AVC the rounding control parameter is f=/3 for Intra and f=/6 for Inter. Therefore, the representative values W’ are not located in the centre of the quantization interval (see bottom graph of Figure 1). This behavior can be utilized to have a quantization that better fits to a non-uniform probability distribution of the W inside a quantization interval. The non-uniform probability distribution inside a quantization interval can be approximated for example by a Laplacian distribution.

Due to this non-symmetric distribution, the expectation value of W inside a quantization interval is not located in the centre of the corresponding interval. In order to reduce the quantization error, this effect is considered by using a rounding control parameter of f</2. Since the probability distribution varies between Intra and Inter mode, different rounding control parameters should be used.

As shown in Figure 1, a parameter f=/6 that is constant for all W has two effects:

- Compared to f=/2, the dead-zone size is increased.

- The representation values W’ for W’0 are located de-centered inside the quantization interval. This leads to a better fit to the non-constant probability distribution of W.

By shifting only the decision level for the first interval, it is possible to reduce the dead-zone size in H.264/AVC and apart from the second interval – keep the match to the non-linear probability distribution. This shift can be realized by using a different f for the first interval. Figure 3 visualizes this effect for different dead-zone sizes.

Figure 2: Visualization of quantization in H.264/AVC with dead-zone sizes. In order to be H.264/AVC compliant only the first decision level is modified

As it can be seen in Figure 3, a H.264/AVC compliant modification of the dead-zone size leads to a deformed second quantization interval that has the following characteristic:

- The second interval has a different size compared to the other intervals.

- The representation value does not match to the probability distribution of W.

This means that it is not possible in H.264/AVC to decouple the control of the dead-zone size and the mapping to the probability distribution of W in the second interval. In order to overcome this problem, we propose to modify the H.264/AVC inverse quantization schemes and use an additional quantization offset parameter. This technique that allows to shift the representation values is described in the following section III.

## Quantization with an offset

### Main Idea

In order to separate the control on the dead-zone size and the mapping to the Laplacian distribution of W, a so-called quantization offset  is introduced. The proposed quantizer using such an offset is described in the following equations.

Z=int|W|+Θ+fΔ⋅sgnW	

W'=Δ⋅Z−Θ⋅sgnZ	

In contrast to the H.264/AVC quantization from (1) and (2) an additional quantization offset  is used. In equation (4) the absolute value of Z is multiplied by the step-size  and the quantization offset  is subtracted. The subtraction of  in the inverse quantization process of (4) is applied, in order to compensate the addition of  in (3). In contrast to this, the addition of the rounding control parameter f in (3) is not compensated in (4). Since the offset is also applied in the inverse quantization it leads to a shift of the decision values W’. Since the offset is used in the encoder and decoder, and since it should be possible to adapt the offset to the signal statistics, it is transmitted in the bitstream.

## Changing the dead-zone size

Using the quantization offset, the dead-zone size can be modified more efficiently. This feature is visualized in the following Figure 4.

Figure 3: Changing the dead-zone size. Comparison between standard H.264/AVC and proposed quantization offset method.

In this figure, quantization with three different dead-zone sizes is shown. The graph for the standard dead-zone size is used in the reference encoder. This graph is used for comparison. The following graphs in Figure 4 show the comparison between the H.264/AVC compliant quantization change and the proposed quantization scheme that uses an additional quantization offset parameter.

As it can be seen in Figure 4, the H.264/AVC compliant modification size leads to a deformed second quantization interval, which leads to a decreased coding efficiency. With the quantization offset , the dead-zone size can be changed, without deforming other quantization intervals. Thus, the picture quality and the coding efficiency can be increased.

# RD constrained cost function for estimation of quantization offsets

Since the modification of the quantization offsets – especially of the  – has a significant influence on the dead-zone i.e. the number of zero coefficients, there is a significant impact on the rate, if  is modified. Thus, a cost function is used for estimation that considers the distortion and the rate. The following figure illustrates the applied RD constrained cost function CRD.

Figure 4: Block diagram to illustrate the rate-constrained cost function.

For the minimization of the cost function in dependence of  and f, we used the downhill simplex method, which is an iterative, numerical method.

# Experimental Results

First results are given below.

Conditions for the experimental results:

- Offsets are adapted for each block

- The bitrate that is required for the transmission of  is not considered, so far

In the plots, B indicates a result that is obtained using a joint optimization of  and f and F indicate a result that is obtained using a f-only optimization. For each B and F evaluation, results for different lambdas shown in the graphs.

# Summary and Proposal

The following improvements were shown:

- In the past (Munich and Redmond meeting), subjective picture quality improvements that are obtained with an additional reconstruction value offset () were already demonstrated [1][2][3].

- In this contribution we present first results that indicate rate distortion gains at higher rates.

Although, the results in this contribution are based on JM software, we propose to start a core experiment within SVC. We think that the adaptation of the quantization offsets may lead to even bigger improvements for SVC than in JM because of the following reasons:

- In FGS several quantization processes are involved that quantize signals, which have different statistics. The quantization offsets can be adapted to these statistics.

- The images of different MCTF layers have different statistics that can be considered in the adaptive selection of quantization offsets.

The following items should be evaluated in this potential core experiment:

- Influence of f-only as well as joint  and f adaptation in SVC scheme

- Transmission of  has to be incorporated

- Estimation process has to be simplified

# References

- T. Wedi and S. Wittmann, “Quantization with an Adaptive Dead Zone Size for H.264/AVC FRExt”, document JVT-K026, JVT of ISO/IEC MPEG & ITU-T VCEG, Munich Meeting, Germany, March 2004.

- T. Wedi and S. Wittmann, “De-quantization offset for H.264/AVC FRExt”, document JVT-L029, JVT of ISO/IEC MPEG & ITU-T VCEG, Redmond Meeting, WA, USA, July 2004.

- A.M. Tourapis, J. Boyce,  “Quantization Offset Matrices for Fidelity Range Extensions”, document JVT-L032r4, JVT of ISO/IEC MPEG & ITU-T VCEG, Redmond Meeting, WA, USA, July 2004.

- G. Sullivan, “Adaptive quantization encoding technique using an equal expected-value rule”, document JVT-N011, JVT of ISO/IEC MPEG & ITU-T VCEG, Hong Kong, China, Jan. 2005.