| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O074 Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Further results on constrained inter-layer prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Heiko Schwarz, Detlev Marpe, and Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-226 / 619 / 617 heiko.schwarz@hhi.fhg.de detlev.marpe@hhi.fhg.de thomas.wiegand@hhi.fhg.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute (HHI) |  |  |

_____________________________

### Summary

The JSVM 1 software does not allow a complete single-loop decoding, since multiple motion-compensation loops are required for the so-called key pictures.  We analyzed the impacts on coding efficiency, when a complete single-loop coding is enabled, and compared it to the JSVM 1 concept as well as to the case of general multiple-loop decoding.

### Analysis of constrained inter-layer prediction suitable for single-loop decoding

In [4][5], we proposed a constrained inter-layer prediction that enables a single-loop inverse MCTF process for the scalable H.264/AVC extension.  This constrained inter-layer prediction was adopted by the JVT, and represents the inter-layer prediction method in the JSVM 1 [2][3]. However, this concept as it is implemented in the JSVM 1 software [3], does not enable a complete single loop decoding.  While the pictures that do not represent key pictures (i.e. the high-pass pictures or pictures between two key pictures) can be decoded using a single motion-compensation loop (or a single inverse MCTF process), multiple motion-compensation loops are required for the decoding of the so-called key pictures.

We have investigated the effects on coding efficiency, when the single-loop decoding option is extended to the key pictures.  This extension is straightforward and requires only minor changes of the JSVM 1 software.  In order to enable the complete single-loop decoding option, the same concepts as for the non-key pictures are applied to the key pictures.  We modified the JSVM 1 software in a way that the single/multiple-loop decoding for key pictures is controlled by the value of constrained_intra_pred_flag of PPS that is used by the base layer key pictures.  This modification is in accordance with the WD [1], and does not require any changes of the WD.

We analyzed the impact on coding efficiency for the 8 common SVC test sequences and the coding conditions of the so-called Munich test points.  The corresponding rate-distortion curves are depicted in Figure 1-Figure 8. In addition to the results for the original JSVM 1 software, and the modified version that enables a complete single-loop decoding, the rate-distortion curves for a multiple-loop decoding are plotted into the diagrams. This codec version (labeled as MLOOP in the diagrams) was enabled by setting the following compiler flag in the JSVM 1 software

#define UNRESTRICTED_INTER_LAYER_PREDICTION   1   // unrestricted inter-layer prediction (not WD / JSVM compatible)

Figure 1: Simulation results for Munich test points and the test sequence “Bus”.

Figure 2: Simulation results for Munich test points and the test sequence “Foreman”.

Figure 3: Simulation results for Munich test points and the test sequence “Football”.

Figure 4: Simulation results for Munich test points and the test sequence “Mobile”.

Figure 5: Simulation results for Munich test points and the test sequence “City”.

Figure 6: Simulation results for Munich test points and the test sequence “Crew”.

Figure 7: Simulation results for Munich test points and the test sequence “Harbour”.

Figure 8: Simulation results for Munich test points and the test sequence “Soccer”.

### References

- J. Reichel, H. Schwarz, M. Wien, eds., “Joint Scalable Video Coding – Working Draft 1,” JVT, Doc. JVT-N020, Hong Kong, CN, Jan. 2005.

- J. Reichel, H. Schwarz, M. Wien, eds., “Joint Scalable Video Model JSVM 1,” JVT, Doc. JVT-N023, Hong Kong, CN, Jan. 2005.

- JSVM 1 software, JVT, Doc. JVT-024, Hong Kong, CN, Jan. 2005.

- H. Schwarz, T. Hinz, H. Kirchoffer, D. Marpe, T. Wiegand, “Technical Description of the HHI proposal for SVC CE1,” ISO/IEC JTC1/WG11, Doc. m11244, Palma de Mallorca, Oct. 2004.

- H. Schwarz, T. Hinz, D. Marpe, T. Wiegand, “Further Progress on Scalable Extension of H.264,” ITU-T VCEG, Doc. VCEG-X08, Palma de Mallorca, Oct. 2004.