| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC 1/SC 29/WG 11 and ITU-T SG 16 Q.6) 28th Meeting: Hannover, Germany, 20-25 April, 2008 | Document: JVT-AB200 (draft) File: input.doc |
| --- | --- |

| Title: | Draft Meeting Report of the 28th JVT Meeting (20-25 July 2008, Hannover, DE) |  |  |
| --- | --- | --- | --- |
| Status: | Draft Output document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Jens-Rainer Ohm Thomas Wiegand Ajay Luthra | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de +49 30 31002 617 wiegand@hhi.de +1 858 404 3470 aluthra@motorola.com |
| Source: | JVT management team |  |  |

_____________________________

# Abstract

The Joint Video Team (JVT) of ITU-T Q.6/16 and ISO/IEC JTC 1/SC 29/WG 11 held its 28th meeting during 20-25 July, 2008 at ITU-T Headquarters in Geneva, Switzerland. The JVT meeting was held under the chairmanship of Dr. Gary Sullivan (Microsoft/USA) and Dr. Jens-Rainer Ohm (RWTH Aachen/Germany), and under the associate chairmanship of Dr. Thomas Wiegand (Fraunhofer HHI/Germany) and Dr. Ajay Luthra (Motorola/USA). The JVT meetings opened at approximately 10:00 p.m. on Sunday 20 July 2008 and closed at approximately XX:XX p.m. on Friday 25 July 2008. Approximately 124 people attended the JVT meetings and approximately 35 input documents were discussed. The meetings took place in a co-located fashion with a meeting of ISO/IEC JTC 1/SC 29/WG 11 (MPEG) – one of the two parent bodies of the JVT. The subject matter of the JVT meeting activities consisted of work on video coding.

# Contents

Abstract	i

Contents	ii

1.	Documents of the JVT meeting	1

1.1.	Input documents	1

1.1.1	Administrative input contributions	1

JVT-AB000  List of documents of Hannover meeting	1

1.1.2	Input liaison statements and parent-body inputs	1

1.1.3	Non-administrative input contributions	1

JVT-AB036 (Errata 2.2.1/3.1) [Y.-K. Wang (Nokia)] SVC errata items	2

1.1.4	Late-registered input contributions, BoG reports, etc.	2

1.2.	Late document availability	2

1.3.	Withdrawn document registrations	3

1.4.	Major output documents	3

Meeting report	3

Overview of outputs planned for this meeting	3

JVT-AA200 Meeting report of the 27th JVT meeting (this document)	3

JVT-AA205-M (WG 11 Nxxxx) Draft conformance testing for SVC (V. Bottreau) [2008-02-29]	3

JVT-AA207 (WG 11 Nxxxx Joint multi-view video model (JMVM) 8 text [2008-02-15]	3

JVT-AA208 (WG 11 Nxxxx) JMVM 8 software [2008-02-22]	4

JVT-AA209 (WG 11 Nxxxx) Joint draft multi-view video coding (MVC) [2008-02-15]	4

JVT-AA210-M (WG 11 Nxxxx) ITU-T Rec. H.264 | ISO/IEC 14496-10 Advanced video coding defect report (G. Sullivan) [2008-04-11]	4

JVT-Y211-M (WG 11 Nxxxx) Draft reference software for SVC [2008-03-20]	4

1.5.	JVT internal output documents	4

JVT-AA202-M Joint scalable video model (JSVM) text	4

JVT-AA203-M JSVM software	4

2.	JVT administrative and liaison topics	4

2.1.	IPR policy reminder and update	4

2.2.	Meeting opening and remarks by the chairmen	5

The JVT contribution registration and upload deadline was Wednesday the 16th.	5

2.3.	JVT communication practices	6

2.4.	Scheduling and logistics notes	7

2.5.	Administrative documents	8

Administrative & AHG report topics	8

JVT-AB000  List of documents of Hannover meeting	8

JVT-AB001 (AhG) [G. J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata	8

JVT-AB002 (AhG) [T. Wiegand, K. Suehring, A. Tourapis, T. Suzuki, G. J. Sullivan] AHG Report: JM text, ref soft, bitstream, conf	8

JVT-AB003 (AhG) [H. Schwarz, J. Vieron, T. Wiegand, M. Wien, A. Eleftheriadis, V. Bottreau] AHG Report: JSVM text, S/W, conf	8

Table 1: CVS access parameters	9

2.6.	Closing session notes	13

2.7.	JVT liaison communications and parent-body communications	13

3.	AVC base specification, errata, and related topics	13

Comment 5(n).iii: May have already been fixed.	14

Comment 5(o): Maybe that's OK as-is.	14

Further details to be addressed in editing work.	14

4.	Scalable video coding (SVC)	14

4.1.	SVC bit depth, color gamut, and chroma format scalability	14

JVT-AB029 (Req 2.2/3.1) [Y.-J. Chiu, H. Jiang, Y.-T. Peng, L. Xu (Intel)] Requirements for scalable video coding: color gamut scalability and bit depth scalability	14

4.2.	SVC Conformance and verification	16

JVT-AB023-L (Draft) [V. Bottreau (Thomson), A. Eleftheriadis (Vidyo)] SVC conformance testing	16

4.2.1	SVC conformance specification	16

4.3.	SVC JSVM software coordination	16

5.	Multi-view coding (MVC)	16

5.1.	Comments on JD	16

JVT-AB020 (Prop 2.2/3.1) [Y.-J. Jeon, B.-M. Jeon (LG)] MVC comment on JD	16

JVT-AB022 (Prop 2.0?) [J. Huo, Y. Chang, M. Li, H. Yang (Xidian Univ.), S. Lin, S. Gao, L. Xiong (Huawei)] Comment on MVC JD 7.0	16

JVT-AB024 (Prop 2.2.1/3.1) [Y. -K. Wang, Y. Chen, M. M. Hannuksela (Nokia)] Comments to MVC JD 7.0	17

JVT-AB028 (Draft) [A. Vetro (MERL), P. Pandit (Thomson), H. Kimata (NTT), A. Smolic (HHI), Y. -K. Wang (Nokia)] Editor's Input on MVC	18

5.2.	MVC profiles	18

JVT-AB035 (Prop Prof 2.0/3.1?) [H. Kimata (NTT), T. Nomura (Sharp), H. Nakamura (JVC), T. Itoh (Fujitsu)] On Multiview High profile	18

JVT-AB037-L (Info) [A. Vetro (MERL)] MVC Profile/Level Definitions for Stereo	19

5.3.	MVC SEI	20

JVT-AB025 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for non-required view component SEI message	20

JVT-AB026 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for view dependency structure SEI message	20

JVT-AB027 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for operation point not present SEI message	20

5.4.	MVC JMVM	21

JVT-AB030 (Prop 2.2/3.1) [Y.-J. Jeon, B.-M. Jeon (LG), S. Gao, L. Xiong (Huawei)] Comments on JMVM 8.0	21

5.5.	MVC verification testing discussion	22

6.	AVC reference encoding methods	22

JVT-AB031 (Ref soft 2.0/3.1) [W.-S. Kim, P. Pahalawatta, Z. Li, A. M. Tourapis (Dolby)] New Video Quality Metrics in JM reference software	22

JVT-AB032 (Ref soft 2.0/3.1) [P. Pahalawatta, A. M. Tourapis (Dolby)] Reorg of Error Resilient RDO in the ref software	22

JVT-AB033 (Ref soft 2.0/3.1) [A. Leontaris, A. M. Tourapis (Dolby)] Weighted Prediction Alternative to Adaptive Interpolation mechanisms	22

JVT-AB034 (Ref soft 2.0/3.1) [E. Maani, L. Liu, A. M. Tourapis (Dolby)] JM Reference Software Enhancements	22

7.	JVT internal operating rules	22

8.	List of AHGs established	23

8.1.	JVT project management and errata reporting	23

8.2.	JM Text, reference software, bitstream exchange and conformance	24

8.3.	SVC JSVM text, software and conformance	24

8.4.	SVC bit depth, color gamut, and chroma format scalability	24

8.5.	MVC JD and JMVM text and software	24

8.6.	MVC JMVM coding tools	25

8.7.	Splicing operation	25

9.	Future meeting plans	25

10.	Attendance	25

# Documents of the JVT meeting

## Input documents

### Administrative input contributions

- JVT-AB000  List of documents of Hannover meeting

JVT-AB001-M (AhG) [G. J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

JVT-AB002-M (AhG) [T. Wiegand, K. Suehring, A. Tourapis, T. Suzuki, G. J. Sullivan] AHG Report: JM text, ref soft, bitstream, conf

JVT-AB003 (AhG) [H. Schwarz, J. Vieron, T. Wiegand, M. Wien, A. Eleftheriadis, V. Bottreau] AHG Report: JSVM text, S/W, conf

JVT-AB004 (AhG) [A. Segall, T. Wiegand, Y.-J. Chiu] AHG Report: SVC bit depth, color gamut, and chroma format

JVT-AB005 (Ahg) [H. Kimata, A. Smolic, P. Pandit, A. Vetro, Y. Chen] AHG Report: MVC JD & JMVM text & software

JVT-AB006 (AhG) [Y. Chen, S. Gao, Y.-J. Jeon] AHG Report: MVC JMVM coding tools

JVT-AB007-M (AhG) [G. J. Sullivan, A. Rodriguez, S. Narasimhan] AHG Report: Splicing operation

### Input liaison statements and parent-body inputs

The following ITU-T Q.6/16 and WG 11 parent-body input contributions were considered:

JVT-AB010 / VCEG-AI07 [HHI] ITU-T AAP Last Call comments on H.264(2007)/Cor.1

JVT-AB011 / VCEG-AI08 [Microsoft] ITU-T AAP Last Call comments on H.264(2007)/Cor.1

JVT-AB012 / VCEG-AI09 [Sony] ITU-T AAP Last Call comments on H.264(2007)/Cor.1

JVT-AB013 / M15558 [JNB of WG11] WG 11 NB comments on ISO/IEC 14496-10:200X/DCOR1

JVT-AB014 / M15538 [DVB TM-AVC] Incoming LS to WG 11 on Constrained Baseline

JVT-AB015 / M15554 [3GPP] Incoming LS to WG 11 on Constrained Baseline

JVT-AB016 / M15711 [ATSC] Incoming LS to WG 11 on AVC Profiles

### Non-administrative input contributions

JVT-AB020 (Prop 2.2/3.1) [Y.-J. Jeon, B.-M. Jeon (LG)] MVC comment on JD

JVT-AB021 (Prop 2.2/3.1) [K.Kazui, A.Nakagawa, S.L.Bhavani, S.Kobayashi (Fujitsu)] Modification of POC type 1 specification

JVT-AB022 (Prop 2.0?) [J. Huo, Y. Chang, M. Li, H. Yang (Xidian Univ.), S. Lin, S. Gao, L. Xiong (Huawei)] Comment on MVC JD 7.0

JVT-AB023-L (Draft) [V. Bottreau (Thomson), A. Eleftheriadis (Vidyo)] SVC conformance testing

JVT-AB024 (Prop 2.2.1/3.1) [Y. -K. Wang, Y. Chen, M. M. Hannuksela (Nokia)] Comments to MVC JD 7.0

JVT-AB025 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for non-required view component SEI message

JVT-AB026 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for view dependency structure SEI message

JVT-AB027 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for operation point not present SEI message

JVT-AB028 (Draft) [A. Vetro (MERL), P. Pandit (Thomson), H. Kimata (NTT), A. Smolic (HHI), Y. -K. Wang (Nokia)] Editor's Input on MVC

JVT-AB029 (Req 2.2/3.1) [Y.-J. Chiu, H. Jiang, Y.-T. Peng, L. Xu (Intel)] Requirements for scalable video coding: color gamut scalability and bit depth scalability

JVT-AB030 (Prop 2.2/3.1) [Y.-J. Jeon, B.-M. Jeon (LG)] Comments on JMVM 8.0

JVT-AB031 (Ref soft 2.0/3.1) [W.-S. Kim, P. Pahalawatta, Z. Li, A. M. Tourapis (Dolby)] New Video Quality Metrics in JM reference software

JVT-AB032 (Ref soft 2.0/3.1) [P. Pahalawatta, A. M. Tourapis (Dolby)] Reorg of Error Resilient RDO in the ref software

JVT-AB033 (Ref soft 2.0/3.1) [A. Leontaris, A. M. Tourapis (Dolby)] Weighted Prediction Alternative to Adaptive Interpolation mechanisms

JVT-AB034 (Ref soft 2.0/3.1) [E. Maani, L. Liu, A. M. Tourapis (Dolby)] JM Reference Software Enhancements

JVT-AB035 (Prop Prof 2.0/3.1?) [H. Kimata (NTT), T. Nomura (Sharp), H. Nakamura (JVC), T. Itoh (Fujitsu)] On Multiview High profile

- JVT-AB036 (Errata 2.2.1/3.1) [Y.-K. Wang (Nokia)] SVC errata items

### Late-registered input contributions, BoG reports, etc.

JVT-AB037-L (Info) [A. Vetro (MERL)] MVC Profile/Level Definitions for Stereo

## Late document availability

Non-administrative documents with document numbers suffixed in this report with "-L", "-Q", or "-M" were classified as late. Such documents will only be considered as information documents only (unless agreed otherwise by the group) if time permits, and consideration of them may be shifted to the end of the meeting as determined appropriate by the group.

For some time now, the JVT has agreed that no late-uploaded (non-AHG-report, non-liaison, non-verification, non-errata-report) contribution would be presented without having a minimum of 4 JVT participants (from different other than that of the primary contribution author) recorded by name as supporting the allowance of such a presentation, in addition to a consensus of the general JVT membership to allow the presentation. Such support to allow a presentation is to be understood to not necessarily imply support of the adoption of the content of the late contribution, but only as a positive expression that the document should be allowed to be presented. Additionally, the provider of a presented late contribution shall send an email apology to the JVT email reflector. This rule does not apply to material requested by the JVT at the meeting (e.g., reports of JVT-authorized side activities).

JVT decision: Agreed.

Suffixes for contribution numbers in this report are explained below:

- "-L" indicates a non-administrative contribution that was somewhat late but was available by the second meeting day (contributions JVT-AB023 and JVT-AB037 were in this category at this meeting).

- "-Q" were more late than that (there were no contribution in this category at this meeting).

- "-M" were still missing at the time of preparation of this draft of this report (there are no contributions in this category for this meeting).

- "-B" were break-out group discussion reports and other input requested during the meeting

Further suffixing by “V” indicates a contribution that contains a cross-verification of a proposal.

On consideration of JVT-AA041 (Mitsubishi Electric – Additional results on magnitude-dependent adaptive motion vector coding):

An apology was sent.

Supporting presentation were A. Segall, Y.-K. Wang, M. Wien, and H. Schwarz.

On consideration of JVT-AA044 (Tech. Univ. Tampere and Nokia – Scalable multi-view video coding):

An apology was sent.

Supporting presentation were A. Vetro, A. Segall, P. Pandit, and M. Wien.

There were no objections to presentations of late documents at this meeting.

It was noted that the situation surrounding the need for on-time availability of contributions has substantially improved since our lateness penalty rules were adopted.

## Withdrawn document registrations

None.

## Major output documents

Major output documents submitted to parent-body review included the following. (Dates listed are planned dates of availability.)

Review of missing outputs of last meeting:

- Meeting report

Prof prof reference software [ISO FDAM version, JVT version]

Study text for SVC conformance testing [ISO version]

WD 1 of MVC reference software [ISO JVT-AA212  w9761]

JMVM sofware [ISO JVT-AA208  w9763]

- Overview of outputs planned for this meeting

Errata [Study status for ISO/IEC]

Prof prof conformance testing [FDAM for ISO/IEC, JVT?]

SVC conformance testing [FDAM for ISO/IEC, JVT]

SVC reference software [Study status for ISO/IEC]

MVC text [FDAM for ISO/IEC]

Draft 2 of MVC reference software [PDAM for ISO/IEC]

Draft 1 of MVC conformance [WD for ISO/IEC]

JMVM?

NOTES NEEDING UPDATING BELOW:

##### JVT-AA200 Meeting report of the 27th JVT meeting (this document)

##### JVT-AA205-M (WG 11 Nxxxx) Draft conformance testing for SVC (V. Bottreau) [2008-02-29]

(Conveyed to WG 11 as "Text of ISO/IEC 14496-4:2004/FPDAM 31 Conformance Testing for Scalable Video Coding".)

##### JVT-AA207 (WG 11 Nxxxx Joint multi-view video model (JMVM) 8 text [2008-02-15]

##### JVT-AA208 (WG 11 Nxxxx) JMVM 8 software [2008-02-22]

##### JVT-AA209 (WG 11 Nxxxx) Joint draft multi-view video coding (MVC) [2008-02-15]

(Conveyed to WG 11 as "Text of ISO/IEC 14496-10:200X/FPDAM 1  Multiview Video Coding".)

##### JVT-AA210-M (WG 11 Nxxxx) ITU-T Rec. H.264 | ISO/IEC 14496-10 Advanced video coding defect report (G. Sullivan) [2008-04-11]

(Conveyed to WG 11 as "Text of ISO/IEC 14496-10:200X/DCOR 1".)

##### JVT-Y211-M (WG 11 Nxxxx) Draft reference software for SVC [2008-03-20]

(Conveyed to WG 11 as "Text of ISO/IEC 14496-5:2001/FPDAM 19 Reference Software for Scalable Video Coding".)

Software for Prof Prof.

## JVT internal output documents

JVT internal output documents included the following. (Dates listed are planned dates of availability.)

##### JVT-AA202-M Joint scalable video model (JSVM) text

##### JVT-AA203-M JSVM software

# JVT administrative and liaison topics

## IPR policy reminder and update

Participants were reminded of the IPR policy established by the parent organizations of the JVT and were referred to the parent body web sites for further information. The IPR policy was summarized for the participants.

Participants were particularly reminded of the need to supply a completed JVT IPR status reporting form in all technical proposals for normative standardization. Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVT as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

The JVT chair noted that the top-level parent bodies have agreed upon a common patent policy among ITU-T, ITU-R, ISO, and IEC.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, IEC and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvt-site (JVT contribution template for each meeting)

- http://www.itu.int/ITU-T/studygroups/com16/jvt/index.html (JVT founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (SC29 Procedures)

The JVT chair noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows (and as previously sent to the JVT email reflector), per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The JVT chair noted (as also previously remarked on the JVT email reflector) that since we are at the completion of the MVC amendment project, it was suggested that if anyone needs to report IPR on that topic and has not yet done so, now would be a good time to file formal notices to the parent bodies for any patent rights that are believed to be essential to the implementation of the MVC extensions (not to mention any notices not previously filed relating to the new SVC profiles, AVC professional profiles, or other previous projects).

It is suggested that, to enable proper interpretation of such formal notices, the MVC amendment should be clearly identified in such formal notices. For example, as “ITU-T Rec. H.264 and ISO/IEC 14496-10 Advanced video coding (2007 Ed.) Amendment 1 (2008): Multiview video coding”. Notices pertaining to other efforts should be made with a similar degree of clarity of identification of the specific standardization work item to which the declaration pertains.

The chair invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation and opened the floor for such reports: No such verbal reports were made.

## Meeting opening and remarks by the chairmen

The meeting was opened at approximately 10:00 a.m. on Sunday 20 July 2008.

The JVT meeting will end by lunchtime on Friday 25 July.

Our goal for this meeting is to finish all major JVT business by noon on Thursday 24 July, leaving only perhaps some minor "tidying up" work after that.

Document handling and other working practices were reviewed for the participants.

A document template was attached to various email announcements and could also be found at http://ftp3.itu.int/av-arch/jvt-site/JVT-ABxxx.dot.  It contained important instructions and policy information. Participants had been instructed to read it and use it as the basis of their contributions.

Documents were made available for download at http://ftp3.itu.int/av-arch/jvt-site/2008_07_Hannover.

The deadline was Wednesday 16 July 2008 for registrations and uploads.

The top areas of contributions were MVC and errata reports.

At the opening session of the meeting, the JVT chairs reminded participants of the relevant IPR policy as described above, and reviewed the status and plans for the major projects under way in the JVT. The largest area of activity consisted of multi-view video coding (MVC) extensions of the ITU-T Rec. H.264 | ISO/IEC 14496-10 Advanced video coding (AVC) standard. SVC work was categorized as "phase 1" or "phase 2", depending on whether the work related to the recently-designed initial SVC amendment or to a potential future further SVC extension.

Opening remarks:

- IPR policy reminder

- Professional profiles – follow up work on reference software and conformance

- SVC phase II – work areas included investigation of bit depth, color gamut, and chroma format scalability and fine-granularity scalability

- Multiview video coding (MVC) was a major project underway, with the goal of finishing the amendment text for ISO/IEC as an output of this meeting

- Corrigendum work is needed, and was a major priority for this meeting

Further work and additional needs on the development, standardization, and maintenance of the base specification and the recently-completed SVC and professional profiles, and of associated reference software and conformance specifications was noted.

The chair remarked that there were few late document uploads this time, and that the submitted documents seem to be generally adhering to the JVT guidelines in terms of formatting, filenames, etc., which is a good development, although further improvement (particularly in the formatting conventions) is still needed. The JVT operating rules on that subject have helped.

## JVT communication practices

JVT documents were available at http://ftp3.itu.int/av-arch/jvt-site.

These can also be accessed via ftp with the site name ftp3.itu.int, user ID avguest and password Avguest. Upon login, documents will then be found in the directory "jvt-site". Uploading of contributions is done by upload via ftp protocol to the "jvt-site/dropbox" directory using this account ID and password.

JVT email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jvt-xyz, and to send email to one of these reflectors, the email address is "jvt-xyz@lists.rwth-aachen.de", where "xyz" corresponds to

- "experts" for general experts group discussions

- "bitstream" for bitstream exchange activities

- "svc" for SVC work

- "mvc" for MVC work

## Scheduling and logistics notes

Information about the meeting, including participation registration information, had been made available at http://mpeg.tnt.uni-hannover.de/index.php.

Participants had been reminded of the need to be properly qualified to attend.  Interested parties had been instructed to contact the JVT management team if they sought clarification of what proper qualification entails or if they needed help learning how to obtain such qualification.

JVT document registration and contribution archiving followed ordinary JVT practices.

Requests to register documents were handled by email to Gary Sullivan (gary.sullivan@itu.int or garysull@microsoft.com).

The JVT work was suspended during plenary sessions of the ISO/IEC JTC 1/SC 29/WG 11 (MPEG) parent body.

Some “break-out group” (BoG) side activities and informal study efforts were conducted during the meeting. Documents produced by break-out group activities (if any) are listed in this report with the abbreviation “BoG” and are suffixed with "-B".

Note that the JVT has agreed that no late-uploaded (non-AHG-report, non-liaison, non-verification) contribution will be presented without having a minimum of 4 non-affiliated JVT participants from different organizations recorded by name as supporting the allowance of such a presentation, in addition to a consensus of the general JVT membership to allow the presentation. Additionally, the provider of a presented late contribution must send an email apology to the JVT email reflector.  Additional details on this topic are described elsewhere in this report.

## Administrative documents

- Administrative & AHG report topics

##### JVT-AB000  List of documents of Hannover meeting

##### JVT-AB001 (AhG) [G. J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

##### JVT-AB002 (AhG) [T. Wiegand, K. Suehring, A. Tourapis, T. Suzuki, G. J. Sullivan] AHG Report: JM text, ref soft, bitstream, conf

##### JVT-AB003 (AhG) [H. Schwarz, J. Vieron, T. Wiegand, M. Wien, A. Eleftheriadis, V. Bottreau] AHG Report: JSVM text, S/W, conf

This document presented the report of the AhG on JSVM text, JSVM software, and conformance.

The text of the Joint Scalable Video Model was not modified relative to JVT-X202 (JSVM-11).

The JSVM software has been improved relative to JVT-Y211/JVT-Z203. Known bugs have been fixed. Adoptions of the last JVT meeting and some new features (which were missing) have implemented into the software. The latest version of the JSVM software is JSVM 9.13.1 (CVS tag: JSVM_9_13_1).

Changes relative to JVT-Y211/JVT-Z203 (CVS tag: JSVM_9_12_2):

- Several bug fixes

  - derivation of intra prediction modes in AVC rewrite mode

  - direct mode in enhancement layers

  - field picture coding (base layer only)

- Several (minor) corrections related to decisions of the last JVT meeting in Geneva

  - prefix NAL unit syntax

  - removal of filler prefix NAL units

  - fix in residual upsampling

  - derivation of RestrictedSpatialResolutionChangeFlag

  - I_BL mode in loopfilter of MBAFF frames

  - border extension before intra upsampling for interlaced

- Correction of setting level_idc in encoder (included added support of level 1b)

- Full support of scaling matrices in encoder and decoder (including spatial & SNR scalability and including a new encoder option for specifying scaling matrices)

- Full IPCM support in encoder and decoder (including spatial & SNR scalability)

- Correction of DPB implementation in decoder in particular regarding MMCO commands for field pictures (more or less completely rewritten)

- Modification of coding order of pictures in encoder in particular for low-delay configurations (supports "real" low-delay coding)

- Encoder support for (high-delay) hierarchical P pictures

- Encoder and decoder support for long-term pictures (requires smaller DPB for hierarchical coding structures)

- Configuration file parameter for specifying the profile

- Check of profile constraints added in encoder

- Check of several level constraints added in encoder

  - Maximum number of motion vectors

  - Minimum block size for bi-prediction

  - Maximum slice size for Scalable Baseline & Scalable High profile

  - Support of interlaced coding tools (frame_mbs_only_flag == 0)

  - Support of CABAC & 8x8 transform in Scalable Baseline

- Some corrections in software manual

Required additions/corrections for future version:

- check/correction of SNR scalability with Mbaff frames (currently this is considered as spatial scalability in the software)

- support of field pictures in scalable configurations in encoder (currently only correctly supported in single-layer coding)

- addition of remaining profile/level constraints in encoder (add checks in decoder?)

- decoder support for AVC rewrite mode in connection with incomplete layer representations (as adopted at the Geneva meeting in April 2008)

- error detection & concealment (without the restriction to two layers and fixed prediction structures that was found in the previously implemented version)

Further desired improvements of the software:

- general improvement of encoder structure, including

  - using same functions for P and B pictures ("key" and "non-key" pictures)

  - using same functions for "slice mode"

  - combine "motion estimation", "motion compensation", and "residual encoding"

- support of more flexible encoder configurations, including

  - GOP structures with non-dyadic prediction structures

  - combination of MGS and CGS coding

  - higher flexibility of "MGS vector mode"

In order to keep track of the changes in software development and to always provide an up-to-date version of the JSVM software, a CVS server for the JSVM software has been set up at the Rheinisch-Westfälische Technische Hochschule (RWTH) Aachen. The CVS server can be accessed using TortoiseCVS, WinCVS, or any other CVS client. The server is configured to allow read access only using the parameters specified in Table 1. Write access to the JSVM software server is restricted to the JSVM software coordinators group.

- Table 1: CVS access parameters

| authentication: | pserver |
| --- | --- |
| host address: | garcon.ient.rwth-aachen.de |
| path: | /cvs/jvt |
| user name: | jvtuser |
| password: | jvt.Amd.2 |
| module name: | jsvm or jsvm_red |

Example 1 shows how the JSVM software can be accessed by using a command line CVS client.

Example 1: Accessing the JSVM software with a command line CVS client

cvs –d :pserver:jvtuser:jvt.Amd.2@garcon.ient.rwth-aachen.de:/cvs/jvt login cvs –d :pserver:jvtuser@garcon.ient.rwth-aachen.de:/cvs/jvt checkout jsvm

In Example 2, it is shown how a specific JSVM software version – specified by a tag (JSVM_9_8 in Example 2) – can be obtained using a command line CVS client. Note that co represents an abbreviation for the command checkout, which was used in Example 1.

Example 2: Accessing the JSVM software version with the tag JSVM_9_8 with a command line CVS client

cvs –d :pserver:jvtuser:jvt.Amd.2@garcon.ient.rwth-aachen.de:/cvs/jvt login cvs –d :pserver:jvtuser@garcon.ient.rwth-aachen.de:/cvs/jvt co –r JSVM_9_8 jsvm

It is possible to checkout only a reduced JSVM software package by using the module name jsvm_red instead of jsvm. In this case, the directories JSVM0-config-sample and MVC-Configs are omitted in the checkout, see Example 3.

Example 3: Accessing the JSVM software without the JSVM0 and MVC directories.

cvs –d :pserver:jvtuser:jvt.Amd.2@garcon.ient.rwth-aachen.de:/cvs/jvt login cvs –d :pserver:jvtuser@garcon.ient.rwth-aachen.de:/cvs/jvt co jsvm_red

The CVS repository includes a JSVM software manual, which provides further information on the JSVM software.

The text of the conformance document "Draft conformance testing for SVC" was submitted as JVT-AA205. A revised version of the conformance draft was provided as input to this meeting as document JVT-AB023.

##### JVT-AB004 (AhG) [A. Segall, T. Wiegand, Y.-J. Chiu] AHG Report: SVC bit depth, color gamut, and chroma format

This contribution reported on activity for SVC bit depth, color gamut, and chroma format.  The mandates of the AhG were:

–	Identify applications

–	Work out suggestions for detailed needs

–	Find/create test material

–	Study bit-depth reduction techniques, e.g., tone-mapping tools

–	Study color space and/or gamma conversion requirements

–	Study combined spatial and bit depth scalability

–	Define experiments and test conditions

–	Investigate software and text modification needs

–	Identify complexity issues

There was no activity reported on the reflector.

It was noted that there was one relevant input contribution on the topic: JVT-AB029.

The AhG recommended the following:

- To review related contributions during the meeting

- To continue the AhG

##### JVT-AB005 (Ahg) [H. Kimata, A. Smolic, P. Pandit, A. Vetro, Y. Chen] AHG Report: MVC JD & JMVM text & software

At the Geneva meeting, JVT established the AhG on MVC JD and JMVM text and software with the following mandates:

- Collect comments on draft, perform necessary editing and delivery.

- Maintain JMVM and JD document and collect comments on the text.

- Coordinate JD/JMVM software integration

- Coordinate bug-fixing process for the JD/JMVM software

- Maintain JD/JMVM software manual

The JMVM8 and JD7 were submitted to JVT as JVT-AA207 and JVT-AA209, respectively.

The JMVM8 text reflected changes described in the following contributions:

- JVT-AA028 [Huawei] Comment on Motion Skip

- JVT-AA031 [LG] High level syntax for motion skip mode

- JVT-AA047: Breakout Group Proposal for JMVM Motion Skip

It was reported that there had been several major changes made to the JD7 text. The changes were a result of input contributions and break-out group discussions during the Geneva meeting. The most significant changes are summarized below.

- Comments on JD [Ref: JVT-AA020 / JVT-AA021 / JVT-AA033 / JVT-AA046]

  - Level limits and decoding resource requirements not be specified in terms of the number of views

  - Bug fix to signal idr_pic_id in slice header (revised condition in 7.3.3 slice header syntax)

  - Added text for SPS/PPS activation, NAL unit order/associations, detection of first VCL NAL unit

  - Bug fix in NAL unit header SVC MVC extension syntax – no need to advance 3 bytes

  - Remove max_num_view_dep

  - Apply decoding process for POC independently for each view

  - RPL construction process with discarded view components (text added to H.8.2)

  - Remove RPL construction for anchor pictures (text removed from H.8.2.1)

  - Specify sub-bitstream extraction for more than one target view

  - For sub-bitstream extraction, added specification to ensure base view in output.

  - Assign SEI payload types

- New SEI Messages

  - JVT-AA034: Non-required view component SEI

  - JVT-AA035: View dependency change SEI and operation point not present SEI

- Other changes:

  - Aligned the structure of the syntax and semantics subclauses in Annex H to the base spec – therefore, the syntax and semantics of NAL unit header extension and subset SPS, and the semantics of slice header, have been moved to Annex H.

  - Updated Table 7-1 for description of different NAL unit types as well as VCL and VCL NAL units categarization, made corresponding changes in the base spec and Annex G.

  - Added some new definitions, namely anchor access unit, anchor view component, decoded view component, IDR picture, IDR view component, inter-view prediction reference, inter-view reference index, non-anchor access unit, non-anchor picture, non-anchor view component, and reference picture. Removed the definition of V-IDR picture as it turned out to be useless in the new context. Also updated some of the existing definitions. Renamed inter-view only reference picture to inter-view only reference component.

  - Updated NAL unit extension syntax (the order of syntax elements and name of the reserved bits syntax elements have been changed) and semantics, including some bug fixes (e.g. the first bit of the NAL unit header exttension shall be 1 for SVC and 0 for MVC, but was the other way round).

  - In the subset SPS syntax, one bit equal to 1 was added before the syntax strucutre seq_parameter_set_mvc_extension( ), to make the subset SPS syntax compatible to the SVC subset SPS syntax specified in Annex G.

  - Fixed the conditions related to slice_type in the ref_pic_list_reordering( ) syntax table.

  - Updated semantics for SPS MVC extension syntax elements and specified the persistence and accuracy of these syntax elements.

  - Updated the reference picture list reordering semantics with both editorial improvements and bug fixes – the bugs are related to the use of syntax element num_anchor_refs_lX[ i ].

  - Updated the text for the reference picture list construction process, with both editorial improvemens and bug fixes – the bugs are related to the use of syntax elements num_anchor_refs_lX[ i ], anchor_ref_lX[ i ][ j ], num_non_anchor_refs_lX[ i ], and non-anchor_ref_lX[ i ][ j ] in subclause H.8.2.1.

  - Updated the specification of the Multivew High profile regarding restriction to the base view bitstream and definition of the conforming decoders.

  - Added exentions to semantics of AVC and SVC SEI messages (mainly regarding being not included or included in an MVC scalable nesting SEI message).

  - Changed the name of scalable nesting SEI message to MVC scalable nesting SEI message to avoid using the same name as the one for SVC, and updated the semantics.

Some remaining open issues related to the JD text include:

- Revisit limit of 16 on max decoder frame buffering

- Review following text

  - SPS/PPS activation, NAL unit order/associations, detection of first VCL NAL unit

  - Sub-bitstream extraction process

  - All level limits

- Semantics of profile, level and HRD parameters etc. in SPS and subset SPS and whether to signal additional sets of profile and level in SPS and subset SPS

- Various editing notes in the text need to be resolved

- Consider relevant SVC errata [JVT-AA025]

The JMVM8 software had been delivered to the group on 6 June 2008 as JVT-AA208. This release contained:

- High-level syntax changes to accommodate adoptions related to motion skip tool

- Software improvements and bug fixes

The JD reference software had also been produced, which reportedly corresponded to the JD text, i.e., no block level coding tools. This software was delivered on 6 June 2008 as JVT-AA212.

Some software issues that still need to be addressed were reported to include:

- Output order of views is not sequential or parallel

- Prepare validation scripts (work in progress)

- Provide support GOPsize=1 (with motion skip & IC)

- Trace file support for arbitrary view_id assignments

The AhG recommended

- To consider the input to this meeting in preparing future versions of the JMVM and JD

- To improve the manual created for the JD/JMVM software

- To follow the same software integration guidelines present in JSVM (repeated below)

##### JVT-AB006-M (AhG) [Y. Chen, S. Gao, Y.-J. Jeon] AHG Report: MVC JMVM coding tools

The mandates of this AHG reported in the previous meeting are reused here.

- Investigate simplification and improvement of current JMVM coding tools (IC and motion skip)

- Investigate techniques for single loop decoding to reduce complexity starting with motion skip

- Investigate approaches for enhancing MVC coding efficiency using spatial downsampling

- Investigate low-complexity methods for mobile stereoscopic 3DTV applications

- Investigate other potential approaches to achieving enhanced MVC capability

- Coordinate software, test material, and experiment conditions for these techniques

- Evaluate performance of enhanced MVC proposals (including CAVLC operation in particular)

No email messages were reported to have been exchanged on this topic on the reflector.

The only contribution identified by the AHG as relevant to this AHG in this meeting was JVT-AB030.

The AHG recommended to review this contribution and to discuss the mandates and the need for having such an AHG in the future.

##### JVT-AB007-M (AhG) [G. J. Sullivan, A. Rodriguez, S. Narasimhan] AHG Report: Splicing operation

## Closing session notes

NOTES NEED UPDATING:

In the closing session there were no requests to reopen discussions of preceding agenda topics and side activities recorded elsewhere in this report.

The JVT thanked its ITU-T Q.6/16 parent body for hosting the 27th JVT meeting.

The meeting was closed at 12:45 p.m. on Tuesday 29 April 2008.

## JVT liaison communications and parent-body communications

The JVT did not receive formal liaison communications at this meeting.  However some JVT contributions were originated as WG 11 parent body National Body input liaison contributions as discussed below.

##### JVT-AB014 / M15538 [DVB TM-AVC] Incoming LS to WG 11 on Constrained Baseline

##### JVT-AB015 / M15554 [3GPP] Incoming LS to WG 11 on Constrained Baseline

##### JVT-AB016 / M15711 [ATSC] Incoming LS to WG 11 on AVC Profiles

# AVC base specification, errata, and related topics

The latest prior errata reporting status was provided in XX.

##### JVT-AB010 / VCEG-AI07 (Errata) [HHI] ITU-T AAP Last Call comments on H.264(2007)/Cor.1

This contribution contains various errata comments that were submitted as ITU-T AAP Last Call comments.  Notes on particular topics in the contribution are provided below.

Regarding MMCO = 3, just delete "short-term" and add "(equal to 3 or 6)".  In the following NOTE that contains "shall"s, change the "shall"s to "must"s and pluralize "field" at the end of the NOTE, and rephrase the extra NOTE.

##### JVT-AB011 / VCEG-AI08 (Errata) [Microsoft] ITU-T AAP Last Call comments on H.264(2007)/Cor.1

This contribution contains various errata comments that were submitted as ITU-T AAP Last Call comments.  Notes on particular topics in the contribution are provided below.

Comment 5(c): Seems to only be an issue when using Tools->Options->View-All, so it may not be necessary to fix.

Comment 5(k): Specify that the decoder shall ignore the content of the SEI message in those cases.

Comment 5(m): Re-examine aspects relating to "overscan", including in particular "do neither use".

Comment 5(n).i: We may have too many cases like that to try to fix them all, and in this case you can include the opening parenthesis in the search.

- Comment 5(n).iii: May have already been fixed.

- Comment 5(o): Maybe that's OK as-is.

- Further details to be addressed in editing work.

##### JVT-AB012 / VCEG-AI09 (Errata) [Sony] ITU-T AAP Last Call comments on H.264(2007)/Cor.1

##### JVT-AB013 / M15558 (Errata) [JNB of WG11] WG 11 NB comments on ISO/IEC 14496-10:200X/DCOR1

##### JVT-AB021 (Prop 2.2/3.1) [K. Kazui, A. Nakagawa, S. L. Bhavani, S. Kobayashi (Fujitsu)] Modification of POC type 1 specification

##### JVT-AB036 (Errata 2.2.1/3.1) [Y.-K. Wang (Nokia)] SVC errata items

# Scalable video coding (SVC)

## SVC bit depth, color gamut, and chroma format scalability

##### JVT-AB029 (Req 2.2/3.1) [Y.-J. Chiu, H. Jiang, Y.-T. Peng, L. Xu (Intel)] Requirements for scalable video coding: color gamut scalability and bit depth scalability

This document considers a new application scenario for video coding in which color gamut scalability of consumer electronics product is asserted to be desired. The architecture of such a system design was presented, which includes modules of color gamut compression and color gamut expansion to achieve the color gamut scalability in the framework of SVC. Suggested requirements for color gamut scalability were described, and preliminary results that aim to demonstrate the benefits of the wide color gamut video at the receiver sides were shown. The document also suggests that higher bit depth formats are required for the video conveyed in the enhancement layer to increase the description of the level of color fidelity contained in the wide color gamut video.

Remark: Part of the asserted motivation is a quality assurance motivation, as opposed to an interoperability enabling motivation.  Some systems will do a good job of gamut mapping if they receive out-of-gamut colors (and some will not) – that seems like a question of product quality assurance motivation rather than interoperability-enabling standardization motivation.

Counter-remark: But there is an issue here of trying to deploy new technology into a domain in which there is widespread use of existing technology that was not designed to accommodate the presence of the new signals.

The proponent indicated that there is a waste of bandwidth associated with sending wider color gamut representation to a limited-gamut receiver.

Question: But how much of a waste is that?  Most scenes don't contain those extreme colors, and even for those that do, how many bits would be saved by mapping them to a narrower color gamut before encoding?  Response from proponent: This is scene dependent – the bit rate savings potential may need study.

Envisioning a greater future emphasis on 4:4:4 coding and increased bit depths, the proponent indicated that the difference may become more important in such scenarios.

Examples were shown of pictures that look different, where the difference was asserted to be due to the limited gamut range of the BT.709 color primaries, coupled with a poor use of mapping (i.e., component-by-component clipping rather than proper color mapping) prior to limited-gamut display.

It was suggested that it would be preferable to design a transmitting system to perform the color mapping gamut limitation prior to encoding as a base layer, and then send the differences that extend the color gamut range as an enhancement layer.

Color gamut scalability had already been included in the scope of the relevant AhG work area.

Remark: Some of the demonstrated differences seem to have to do with 4:2:0 sampling rather than color gamut range.

Remark: We could offer a package of scalable features that would provide an enhancement of the sort that could not be provided by only increasing the bit rate/fidelity of the same representation.  Consider:

- extended gamut

- increased bit depth

- tone mapping

- 4:4:4 enhancement of 4:2:0 or 4:2:2

Remark: Such an enhancement layer could be a separate business model as well as a separate enhancement bitstream.

Remark: What is the application? Responding remark: Potentially, optical disc (e.g. Blu-ray).

Remark: Why would such an application need scalability?  Could someone just fit an extra single-layer bitstream on a Blu-ray disc.

JVT Disposition: Further study the requirements and determine the level of industry interest in such a scalable approach to delivery of these features.

## SVC Conformance

##### JVT-AB023-L (Draft) [V. Bottreau (Thomson), A. Eleftheriadis (Vidyo)] SVC conformance testing

## SVC JSVM software coordination

# Multi-view coding (MVC)

## Comments on JD

##### JVT-AB020 (Prop 2.2/3.1) [Y.-J. Jeon, B.-M. Jeon (LG)] MVC comment on JD

This contribution provided a comment on the MVC JD7.0 that was asserted to be editorial. It was asserted that the current specification does not clearly specify the decoding process of all the view components of the access unit. To clarify and specify the decoding process of all view components of the access unit, a comment describing the decoding process according to VOIdx (view order index) is presented.

JVT Disposition: The proponent was asked to work with the editors to clarify the text as necessary (without technical change).

##### JVT-AB022 (Prop 2.0?) [J. Huo, Y. Chang, M. Li, H. Yang (Xidian Univ.), S. Lin, S. Gao, L. Xiong (Huawei)] Comment on MVC JD 7.0

Two comments on the MVC JD7.0 were provided in this contribution. The first proposed change was related to the view dependency change SEI message, and a modification was proposed that was asserted to relate to compatibility with the sequence parameter set MVC extension.

The first proposed change was to replace the lower index of a "for" loop – changing it from 0 to 1 (skipping over view 0).  View 0 is the base view, which does not have dependencies, so its inclusion in the syntax loop was concluded to be an error.

JVT decision: Adopted (excess "{ }" curly brackets around single statements in syntax table will also be removed).

The second proposed change was related to the non-required view component SEI message, and a derivation method was proposed for the decoder to derive the non-required view component so that the explicit transmission of the SEI message for every related access unit could be avoided.

It was proposed to remove redundancy in non-required view SEI messages (depending on conditions establishing which other components are non-required components, so that the transmission of redundant information can be avoided).

Concern: Dependency is made from SEI messages of other access units. Deriving the information at the decoder may not be simple and require dependency information beyond the SEI message itself.

Remarks: Proposal is to avoid sending some SEI messages by depending on inferred relationships established from SEI messages of other access units.  It was commented that some of these relationships that would need to be inferred might not be so easily derived.  It was further commented that the current SEI message semantics would allow such information to be inferred by a decoder – the text just does not include any remark pointing out this fact.  Remark: The degree of redundancy expressed by use of the existing SEI message is already under the control of the encoder.  Suggestion: Have off-line BoG [Miska, Ye-Kui, proponent] to consider whether providing only some additional non-normative remarks to point out what the decoder may be able to infer when some information is not directly expressed in the syntax, and revisit to consider whether something more than such editorial improvement should be considered.

##### JVT-AB024 (Prop 2.2.1/3.1) [Y. -K. Wang, Y. Chen, M. M. Hannuksela (Nokia)] Comments to MVC JD 7.0

This document proposed to 1) remove the active view information SEI message from the MVC specification, and 2) address extraction of SEI messages in the sub-bitstream extraction process, which was not addressed in the latest MVC draft.

On active view information SEI message: It was proposed to remove the active view information SEI message. This SEI message was designed be included in the bitstream to indicate which views are the target output views, such that the target output views can be known from the bitstream. However, if the same MVC bitstream is used at a different operation point, then it was asserted that the content of the SEI message must be changed. It was asserted that this implies that for multicast with receivers using different operation points, some network elements between the sender and the receivers must change or insert active view information SEI messages into the outgoing sub-bitstreams. Due to this reason (among others), it was reportedly decided not to use this means to derive the set of target output views in the HRD specification in the MVC JD.

Suggestion: In the semantics of the SEI message, we can indicate that target view information may be conveyed by other means determined by the application, and that the determination of what to do in response to the content of the SEI message, if present, is determined by the application.

Suggestion: Remove the SEI message, specify that the target output views are determined by the application (by means not specified in this Specification) and that in the absence of any selection of target output views by the application, there shall be one target output view which is the base view.  JVT decision: Agreed.

Definition of "target output view" needs editing work to express that well.  Revisit.

Also add remarks as necessary to other new SEI messages, in the spirit of what is stated for, e.g., the film grain characteristics SEI message, that the actual decoder response to the SEI message is not normatively specified.  JVT decision: Agreed.

On the sub-bitstream extraction process: The extraction process proposed in the editors' input (JVT-AB028) does not address SEI messages. In our opinion, all the information needed for possibly further bitstream subsets, e.g. picture timing and buffering period SEI messages, should be retained after extraction and the extraction process should be appended accordingly. A note could be added to state that real applications can do extraction in other ways, e.g. to keep only those SEI messages required for certain operation points that are likely to be used. JVT decision: Agreed.

##### JVT-AB028 (Draft) [A. Vetro (MERL), P. Pandit (Thomson), H. Kimata (NTT), A. Smolic (HHI), Y. -K. Wang (Nokia)] Editor's Input on MVC

This document included editors’ input on the draft MVC specification. An important addition was the signaling of HRD parameters for bitstream subsets, along with related changes to the HRD subclauses. Other revisions include changes to the MVC scalable nesting SEI to enable signaling of HRD parameters. Some topics reported as bugs in the sub-bitstream extraction process had also reportedly been resolved, the semantics of SPS extension had reportedly been clarified, and some definitions had been added.

Regarding HRD, the MVC editors recommended that the approach documented in this contribution be adopted.

It was recommended that the revisions in the attached document be reviewed in detail in preparation for JD8 output.

The contribution was reviewed, various minor issues (typographical formatting, use of "if" versus "when", a space preceding a syntax structure parenthesis, etc.) were noted and recorded by the editors to be addressed.

JVT decision: Adopted (including suggestions described in included Powerpoint presentation deck – with minor modifications as recorded by the editors during the review) with number of views and max_dec_frame_buffering issues dealt with as discussed below in section on JVT-AB035.

## MVC profiles

##### JVT-AB035 (Prop Prof 2.0/3.1?) [H. Kimata (NTT), T. Nomura (Sharp), H. Nakamura (JVC), T. Itoh (Fujitsu)] On Multiview High profile

A revised version will be uploaded to clarify the IPR statement status.

Target applications and comments on the related requirements of level constraints of Multiview High Profile were presented, relative to the current MVC draft.

Target applications for Multiview High Profile were discussed.  Target applications were described as interactive video applications, such as panorama video and FTV / free viewpoint video, as well as multi-view 3D video.  In addition, it was discussed how to use MVC in those applications, and parameters such as the number of views were discussed.  Based on these parameters, comments were made on the level constraints of Multiview High Profile, relative to what is specified in the current MVC draft.

Particular points raised in the contribution:

- Number of views to be decoded.  Revisit.

- Limitation of 16 on max_dec_frame_buffering. Revisit.

- The value of num_views_minus1: In H.7.4.2.1.4 (Sequence parameter set MVC extension semantics), it was reported that there is a NOTE 2 for num_views_minus1, as follows: "NOTE 2 - This value should be changed by the sub-bitstream extraction process as specified in subclause H.8.3 to correspond with the exact number of views in a bitstream subset." Because NumViews reportedly affects the level constraints, it was suggested to change this to a normative specification. Revisit.

- The value of level_idc: Do we allow changing the value of level_idc by the sub-bitstream extraction process? Revisit.

- Calculations in the a) and b) level constraints: It was reported that there are calculations using mvcScaleFactor in a) and b) level constraints, as follows: Max( NumViews * PicSizeInMbs  mvcScaleFactor * MaxMBPS, fR ). Does mvcScaleFactor apply to MaxMBPS?  If so, it was suggested to change this to Max( NumViews * PicSizeInMbs  (mvcScaleFactor * MaxMBPS), fR ).  JVT Decision: Agreed.

- Calculations in the p) and q) level constraints: In the p) level constraint for VCL HRD parameters, NumViews is not multiplied for calculating BitRate[ SchedSelIdx ], however, in the q) level constraint for NAL HRD parameters, NumViews is multiplied for calculating BitRate[ SchedSelIdx ]. Clarification is needed if this is correct. JVT Decision: Agreed that this expression should not depend on NumViews, so condition "q" should be changed.

Suggestion: Any level definition for MVC should include capability of decoding at least two views of the corresponding type found as an ordinary AVC capability for the same level. This way the ordinary understanding of the capability associated with a level will still apply to the multiview case. (We did something like this in the SVC case.) Responsive remark from an editor: That's what the mvcScaleFactor does.

Suggestion: Add another dimension to level definitions to express view quantity capability, e.g., a level 3.2.x decoder can decode up to x views, where x is an integer greater than 1, with a DPB capacity that multiplies by x, max bit rate is R*(1+0.75*(x-1)) and CPB capacity C*(1+0.75*(x-1)), and limits on other things such as max_dec_frame_buffering may depend similarly on x.  When x is not provided for a decoder capability description, a value of 2 would be implied for x.  Revisit.

##### JVT-AB037-L (Info) [A. Vetro (MERL)] MVC Profile/Level Definitions for Stereo

This informative contribution highlights the benefits of MVC for stereo applications relative to other options such as simulcast and stereo interleaving. The suitability of the currently defined Multiview High profile and corresponding level limits is also considered.

There has been increased momentum recently in the production of 3D content for cinema applications; for the most part, this has been limited to stereo content. There are also a variety of display technologies on the market that support 3DTV, each offering a different viewing experience and having different input requirements. More specifically, stereoscopic displays support stereo content and require glasses, while auto-stereoscopic displays avoid the need for glasses by rendering view-dependent stereo pairs for a multitude of viewing angles.

There are also a wide range of standardization bodies and consortia considering 3D, including Blu-Ray Disc Association, ATSC, SMPTE and 3D@Home, to name a few. It was suggested that MPEG/JVT should consider opening up liaison communications with these bodies.

The current market needs for stereo content could be satisfied with MVC. This would also be an appropriate format for future extensions to support auto-stereoscopic displays, either with the existing Multiview High profile or a backward compatible extension that includes information such as depth as part of the format.

AVC "stereo video SEI" has stereo capability, but requires some system and decoder adaptation to support decoding. MVC bitstreams can be decoded by legacy decoders and can operate in legacy system designs (provided no buffering constraints are violated).

A hypothetical new Level 4.3 was suggested to be considered for MVC purposes (perhaps not taking into account the mvcScaleFactor).

When considering a simulcast approach, note that this doubles the full bit rate and CPB and DPB capacity requirements of the decoders.

Remark: Many devices already include full dual decoder capability for PIP support.

Liaison communication suggestion: Revisit.

## MVC SEI

##### JVT-AB025 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for non-required view component SEI message

At the Geneva meeting, a non-required view component SEI message (JVT-AA034) was adopted into Joint Draft 7.0 of MVC. It specifies the view components that are not required for decoding a view. In this contribution, a showcase for the non-required view component SEI message was presented.

Example decoder operation was demonstrated with decoding of substantially fewer view components when the SEI message is recognized.

JVT disposition: The showcase is considered adequate to demonstrate the usefulness of this SEI message.

##### JVT-AB026 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for view dependency structure SEI message

At the Geneva meeting, a view dependency change SEI message (JVT-AA035) was adopted into Joint Draft 7.0 of MVC. It specifies the changes of the view dependency, and the changed dependency is a subset of the view dependency specified in the active sequence parameter set. In this contribution, a showcase for the view dependency change SEI message was presented.

Example decoder operation was demonstrated with decoding of substantially fewer view components when the SEI message is recognized.

JVT disposition: The showcase is considered adequate to demonstrate the usefulness of this SEI message.

##### JVT-AB027 (Info SEI) [M. Ji, Y. Chen, Y. -K. Wang, M. M. Hannuksela, H. Li (Nokia)] Showcase for operation point not present SEI message

At the Geneva meeting, an operation point not present SEI message (JVT-AA035) was adopted into Joint Draft 7.0 of MVC. It indicates those operation points that are no longer present after a certain point in time. In this contribution, a showcase for the operation point not present SEI message was presented.

The SEI message indicates that some view is not present in the bitstream.

Question: Is this SEI message to indicate a situation that should not actually intentionally occur (a decoder operating in an application with a selection of target views such that some selected target views are missing from the bitstream)?  Responsive remark: Yes – or perhaps this message is intended to be the way to try to prevent such a situation from happening.  Remark: This seems philosophically broken/inconsistent, as it is an SEI message designed to enable/encourage a situation that should not actually be allowed to occur (i.e., a decoder operating with some selection of a target view that is not actually available in the bitstream).

Remark: A similar situation occurs for an SVC case, and there is a similar SEI message called a "layers not present" SEI message.  Responsive remark: When presented with a conforming bitstream, the SVC spec says to decode the highest layer that is present, not to guess that because the perfectly conforming bitstream has characteristics that differ from what the decoder has been designed to expect, the decoder can do something different from what the standard says that it shall do in response to the conforming bitstream.

The question is, somewhat, whether a decoder is allowed to act as if there is something wrong with the bitstream when the bitstream is a conforming bitstream, and whether we should supply information to help such decoders exhibit better behavior.

Revisit the question.

## MVC JMVM

##### JVT-AB030 (Prop 2.2/3.1) [Y.-J. Jeon, B.-M. Jeon (LG), S. Gao, L. Xiong (Huawei)] Comments on JMVM 8.0

At the last meeting, a one bit flag, non_anchor_ref_lX_sample_pred_flag, was adopted to allow different view dependencies for inter-view sample prediction (ISP) and motion skip (MS). From this, the view dependencies for interview sample prediction and motion skip can be set differently.

When inter-view sample prediction is used for decoding, the dependency for interview sample prediction is referred. [referred?  what?]

On the other hand, when motion skip is used, the dependency for motion skip is referred. [what?]

That is, these two different view dependencies are used for different purposes. In the current SPS MVC extension, there is only one dependency for non-anchor pictures [what?]. Therefore two dependencies for interview sample prediction and motion skip should be derived from the non-anchor view dependency of SPS. However, the derivation process of the dependencies for interview sample prediction and motion skip is missing in the current JMVM text. This contribution suggests adding this process to the current JMVM text. [not understood]

ISP = inter-view sample prediction

MS = inter-view motion prediction

In some old JMVM, the dependencies for ISP and MS were the same. At the last meeting, there were proposals to change that, and a flag was created for the SPS.  Flag = 0 indicates not used for ISP but used for MS; Flag = 1 indicates used for ISP and may or may not used for MS.

Flag is called non_anchor_ref_lX_sample_pred_flag.

Remark: There may be an issue with common conditions or software, but not with the draft spec.  ISP is allowed for P views for non-anchor pictures, although that capability is not used in the common conditions.

Proponent wants to be able to determine if a particular view is used for MS.  Remark: Why? What is important to know, for single-loop decoding purposes, is only whether a view is used for ISP, not whether it is used for MS.

Off-line study to determine if there is a spec problem or a common conditions problem or a software problem and revisit to determine appropriate action.

## MVC verification testing discussion

Revisit.

# AVC reference encoding methods

##### JVT-AB031 (Ref soft 2.0/3.1) [W.-S. Kim, P. Pahalawatta, Z. Li, A. M. Tourapis (Dolby)] New Video Quality Metrics in JM reference software

##### JVT-AB032 (Ref soft 2.0/3.1) [P. Pahalawatta, A. M. Tourapis (Dolby)] Reorg of Error Resilient RDO in the ref software

##### JVT-AB033 (Ref soft 2.0/3.1) [A. Leontaris, A. M. Tourapis (Dolby)] Weighted Prediction Alternative to Adaptive Interpolation mechanisms

##### JVT-AB034 (Ref soft 2.0/3.1) [E. Maani, L. Liu, A. M. Tourapis (Dolby)] JM Reference Software Enhancements

# JVT internal operating rules

JVT decision: The following clarifications/adjustments of JVT operating rules have been adopted.

The JVT decided that participants shall to refrain from long (=more than 4 Minutes) presentations of their proposal, if the results of their coding efficiency experiments have provided less than 2% bit-rate on average (or equivalently 0.1 dB gain on average).

Presentations should also not use "cherry picking" of results for summary reporting in abstracts and presentations. Summary reports must be true summaries – not highlights of best results while ignoring worst results.

Regarding late contributions: Due to our difficulties with a large quantity of late-submitted contributions at previous meetings, the JVT has agreed that for its next meeting, no late-uploaded (non-AHG-report, non-liaison, non-verification) contribution will be presented without having a minimum of 4 JVT participants (working for separate organizations other than that of the primary contribution author) recorded by name as supporting the allowance of such a presentation, in addition to a consensus of the general JVT membership to allow the presentation. Such support to allow a presentation is to be understood to not necessarily imply support of the adoption of the content of the late contribution, but only as a positive expression that the document should be allowed to be presented. Additionally, the provider of such a presented late contribution shall send an email apology to the JVT email reflector. This rule does not apply to material requested by the JVT at the meeting (e.g., reports of JVT-authorized "break out group" side activities).

For all contributions that have presentation material that is used to present them to the group (e.g., PowerPoint presentations), the presentation material should be provided along with the written contribution (within the same zip container file). PDF is preferred over PPT for presentations when the PPT filesize is large and there is no need for the slide deck to be editable by others.

All submissions must be made in JVT-Zxxx.zip format with the word docs, excel sheets and other information being in the zip container. The document must contain an abstract and be accompanied with an e-mail notification containing title, authors and abstract (identical to the one in the doc) which is no longer than 200 words and no shorter than 25 words and is written in 3rd person language in a manner that does not express endorsement of the content of the document.

On filenames inside of .zip containers – use a filename so that if someone takes the files out of the zip container, they would still know what contribution they came from. Every file (or directory) in the .zip container for document JVT-Zxxx should start with JVT-Zxxx. Example: JVT-Zxxx.doc (main document), JVT-Zxxx_presentation.pdf, JVT-Zxxx_results1.xls, etc.

When providing additional or revised files, do not include copies of files that were already included in the prior .zip archive for the same contribution and do not re-use the same filenames without adding revision numbers (r1, r2, etc.) – this saves us needing to worry about whether the files someone obtains with the same filenames are the same or different.

Independent verification (necessary for adoption of a proposal) is provided either through

- independent implementation by 1 or more organization different than that of the proponent based on the textual description (after adoption, both decoder source code versions must be made publicly available along with one encoder version), or

- providing source code to all CE participants prior to the meeting (CEs can only be joined at the meeting, when the CE is created. CEs are created at each meeting and last until the next meeting.)

Simply running binary executables provided by a proponent is not ordinarily considered independent verification. Source code should be provided and used, and the verifying party should invest a proper degree of effort to ensure that the “verification” they perform is a meaningful and professional study with significant depth rather than just a perfunctory procedural formality.

For every SEI message and every syntax element that are currently in the SVC/MVC draft, a showcase has to be provided in order to retain it in the JSVM/JMVM/JD. If such a showcase is not provided at the next meeting for an SEI message or parts of it, the SEI message or the respective parts will be removed from the JSVM/JMVM/JD. The source code and executables for the showcase must be made available.

When Core Experiments (CEs) are to be established, a first CE description should be available at the last day of the meeting (or at least within a few days). Changes of the CE description are only allowed until 3 weeks prior to the next meeting. These changes must be of evolutionary characteristic relative to the input documents on which the CE is based and must be agreed by those who contributed the respective input document(s) or be added as an option.

Contributions that are proposals of new technology that was not what was described as being tested in a CE (even if related to the tested technology) should not indicate that they are CE documents in their title and abstract.

# List of AHGs established

The following JVT “ad hoc groups” (AHGs) were established to progress work on identified topics until the next meeting of the JVT.

THIS SECTION NEEDS UPDATING

## JVT project management and errata reporting

Discussion: jvt-experts@lists.rwth-aachen.de

Chair: Gary Sullivan, Jens Rainer Ohm, Ajay Luthra, and Thomas Wiegand

Mandates:

- Collect errata reports on standards under management of JVT

- Coordinate overall interim JVT progress

- Prepare status information for JVT status reporting

## JM Text, reference software, bitstream exchange and conformance

Discussion: jvt-experts@lists.rwth-aachen.de

Chair: Thomas Wiegand, Karsten Sühring, Alexis Tourapis, Teruhiko Suzuki, Gary Sullivan

Mandates:

- Maintain and update JM algorithm description text

- Maintain and update JM reference software and its usage manual

- Facilitate exchange of test bitstreams to aid interoperability testing

- Collect bitstreams for inclusion in (non-SVC) Conformance specifications

- Identify and correct problems in Conformance specifications and associated bitstreams

## SVC JSVM text, software and conformance

Discussion: jvt-svc@lists.rwth-aachen.de

Chair: Heiko Schwarz, Jérome Vieron, Thomas Wiegand, Mathias Wien, Alex Eleftheriadis, Vincent Bottreau

Mandates:

- Edit and deliver improved JSVM text

- Coordinate JSVM software integration

- Coordinate bug-fixing process for the JSVM software

- Maintain JSVM software manual

- Plan, edit, and collect bitstreams for SVC conformance specification

## SVC bit depth, color gamut, and chroma format scalability

Discussion: jvt-svc@lists.rwth-aachen.de

Chair: Andrew Segall, Thomas Wiegand, Yi-Jen Chiu

Mandates:

- Identify applications

- Work out suggestions for detailed needs

- Find/create test material

- Study bit-depth reduction techniques, e.g., tone-mapping tools

- Study color space and/or gamma conversion requirements

- Define experiments and test conditions

- Investigate software and text modification needs

- Identify complexity issues

## MVC JD and JMVM text and software

Discussion: jvt-mvc@lists.rwth-aachen.de

Chair: Hideaki Kimata, Aljoscha Smolić, Purvin Pandit, Anthony Vetro, Ying Chen

Mandates:

- Collect comments on draft, perform necessary editing and delivery.

- Maintain JMVM and JD document and collect comments on the text.

- Coordinate JD/JMVM software integration

- Coordinate bug-fixing process for the JD/JMVM software

- Maintain JD/JMVM software manual

## MVC JMVM coding tools

Discussion: jvt-mvc@lists.rwth-aachen.de

Chair: Ying Chen, Shan Gao, Yong-Joon Jeon

- Investigate simplification and improvement of current JMVM coding tools (IC and motion skip)

- Investigate techniques for single loop decoding to reduce complexity starting with motion skip

- Investigate approaches for enhancing MVC coding efficiency using spatial downsampling

- Investigate low-complexity methods for mobile stereoscopic 3DTV applications

- Investigate other potential approaches to achieving enhanced MVC capability

- Coordinate software, test material (particularly including steroscopic), and experiment conditions for these techniques

- Evaluate performance of enhanced MVC proposals (including CAVLC operation in particular)

## Splicing operation

Discussion: jvt-experts@lists.rwth-aachen.de

Chair: Gary Sullivan, Arturo Rodriguez, Sam Narasimhan

Mandates:

- Study the use of bitstream splicing in applications

- Investigate potential needs for SEI data to aid in splicing operations, including consideration of JVT-Z040, JVT-Z041, and JVT-Z042 and the issues raised in their discussion

- Study the implications of ITU-T Rec. J.181 and the draft new ITU-T Rec. J.h-dpi

- Gather information about activities of other relevant organizations regarding the development of specifications relating to bitstream splicing

# Future meeting plans

The JVT chairmen proposed to hold the 30th JVT meeting during 12-17 October 2008 under WG 11 auspices in Busan, KR; and the 31st JVT meeting during 29 January – 3 February 2009 under ITU-T SG 16 auspices in Geneva, CH.

# Attendance

Persons registered to attend the meeting, as recorded by a sign-in sheet circulated during the meeting, were the following (124 listed participants):

- Bjøntegaard, Gisle (Tandberg)

- Bottreau, Vincent (Thomson R&D France)

- …

- Wiegand, Thomas (Fraunhofer HHI)

- …