ITU Telecommunication Standardization Sector		Document Q11-L-03

Study Group 16		Q15-J-37

Q.11-15/16 Rapporteur Meeting

Osaka, Japan, 15-19 May 2000		Date: May 3, 2000

SOURCE*: 	PictureTel, Polycom, Tandberg

TITLE:	Proposed changes to H.221 and H.242 to include the new H.263++ features

###### PURPOSE:	Proposal

______________________________________________________________________________

# Introduction

# This document contains the changes proposed to H.221 and H.242 to support the new “H.263++” (H.263v3) features. The new features are described in Annexes U, V, and W of H.263, as well as in a revised Appendix II of H.263. They are planned to be decided at the November 2000 meeting of SG16.

# The changes proposed are inspired from the changes that were proposed by the Rapporteur of Q15 to H.245v7 (TD28/G from Geneva) and from the draft revised Appendix II/H.263 (TD35/3 from Geneva).

We propose that the changes be incorporated in the H.221 and H.242 section of version 2 of the H.320 Implementor’s Guide.

# Changes to section 5.2.2/H.242 – H.262 and H.263 capabilities MBE message format

[Begin Correction]

For H.262, one byte is needed to signal each capability for a particular image format with its associated parameters (see 5.2.3). H.263 capabilities are specified using a variable number of bytes depending on the features and options selected (see 5.2.4). The ordering of the bytes within the single MBE message is as follows:

a. First byte B1 is	{highest standard format of H.263} then follow 0, 1 or 2 optional parameter bytes

b. then	{further standard formats of H.263 if needed (see 5.2.4) in descending order of resolution – with any optional parameter bytes}

c. then	{highest format of H.262}

d. then	{further formats of H.262 if needed (see 5.2.3) in descending order of resolution}

e. then	{extension codeword 01111111 if additional H.263 capabilities are to be specified}

f.  then	{enhancement bytes for additional H.263 capabilities of the highest format of H.263}

g. then	{enhancement bytes for additional H.263 capabilities of further formats of H.263 if needed in descending order of resolution}

h. then	{extension codeword 01111111 if second additional H.263 capabilities are to be specified}

i. then	{enhancement bytes for second additional H.263 capabilities of the highest format of H.263}

j. then	{enhancement bytes for second additional H.263 capabilities of further formats of H.263 if needed in descending order of resolution}

As shown above, the H.263 capability bytes for a format are not contiguous. The H.263 capabilities specified for a format prior to the extension codeword 01111111 will be referred to as initial H.263 capabilities. The H.263 capabilities specified after the first extension codeword will be referred to as additional H.263 capabilities and the bytes used to describe them as enhancement bytes. The H.263 capabilities specified after the second extension codeword 01111111 will be referred to as second additional H.263 capabilities and the bytes used to describe them as enhancement bytes.

For future expansion of the H.262/H.263 capabilities the extension codeword 01111111 will be used as follows:

A decoder shall interpret the extension codeword when encountered for the first time in the H.262/H.263 capabilities message to mean that the following bytes will contain additional H.263 capabilities.

A decoder shall interpret the extension codeword when encountered for the first time within the first byte of additional H.263 capabilities to mean that the following bytes will contain second additional H.263 capabilities.

However, if this extension codeword is encountered again within the first byte of second additional H.263 capabilities of a particular format, all of the data following the extension codeword until the end of this MBE message shall be ignored. The appearance of this code in an MBE message does not affect the meaning of any bytes prior to this code byte.

[End Correction]

# New section 5.2.4.6/H.242 – Second Additional H.263 capabilities

[Begin New Section]

#### 5.2.4.6	Second Additional H.263 capabilities

The second additional H.263 capabilities are specified using a variable number of enhancement bytes from the bytes described in this subclause. The bytes shall be transmitted in the order described in this subclause.

The second additional H.263 capabilities and second extension codeword 01111111 shall not be transmitted unless the “H.263(2000)” capability has been previously received from the far-end terminal, indicating support of the second additional H.263 capabilities described in this subclause.

The leading H.263 enhancement byte – secondAdditionalH.263CapByte – shall be transmitted for all formats larger than or equal to the smallest format for which distinct H.263 second additional capabilities are declared. The secondAdditionalH.263CapByte is structured as shown in table x.33/H.242:

Table x.42/H.242 – Second Additional H.263 Capabilities Byte

| Bit | Name | Value | Description |
| --- | --- | --- | --- |
| 1-2 | inherit/enhancedReferencePicSelect | 00 01 10 11 | Inherit second additional options from immediately larger format Not capable of using enhancedReferencePicSelect Capable of using enhancedReferencePicSelect without pruning Capable of using enhancedReferencePicSelect with pruning |
| 3 | Forbidden | 0 | Set to 0 to avoid emulation of extension codeword |
| 4 | dataPartionedSlices | 0 1 | Not capable of using dataPartionedSlices Capable of using dataPartionedSlices |
| 5 | fixedPointIDCT0 | 0 1 | Not capable of using FixedPointIDCT0 Capable of using FixedPointIDCT0 |
| 6 | interlacedFields | 0 1 | Not capable of using interlacedFields Capable of using interlacedFields |
| 7 | Reserved | 0 | Set to 0 |
| 8 | secondOptionExtByteFlag | 0 1 | secondOptionExtByte does not follow secondOptionExtByte follows |

inherit/enhancedReferencePicSelect, when 00 indicates inheritance of the H.263 second additional options signaled for the immediately larger video format. In this case, the remainder of the Second Additional H.263 Capabilities Byte shall be set to 0.

inherit/enhancedReferencePicSelect, when 01 indicates that the decoder can not use the Enhanced Reference Picture Selection mode of H.263 Annex U.

inherit/enhancedReferencePicSelect, when 10 indicates the capability of the decoder to use the Enhanced Reference Picture Selection mode of H.263 Annex U without pruning.

inherit/enhancedReferencePicSelect, when 11 indicates the capability of the decoder to use the Enhanced Reference Picture Selection mode of H.263 Annex U with reference picture pruning using H.263 Annex U. In this case, mpuHorizMBs and mpuVertMBs values shall follow as specified respectively in x.44/H.242 and x.45/H.242.

dataPartitionedSlices, when 1, indicates the capability of a decoder to support the Data Partitioned Slice mode defined in Annex V of Recommendation H.263.  dataPartitionedSlices shall be 0 if slicesInOrder-NonRect and slicesInOrder-Rect and slicesNoOrder-NonRect and slicesNoOrder-Rect are all 0 in the same H263Options message.

fixedPointIDCT0, when 1, indicates the capability of a decoder to support Reference IDCT 0 defined in Annex W of Recommendation H.263.

interlacedFields, when 1, indicates the capability of a decoder to support interlaced field coding as defined in Annex W of Recommendation H.263.

secondOptionExtByteFlag, when 1, indicates the presence of an extension byte immediately following the second additional H.263 capabilities byte, as specified in Table x.43/H.242.

Table x.43/H.242 – Second Additional H.263 Extensions Capabilities Byte

| Bit | Name | Value | Description |
| --- | --- | --- | --- |
| 1-2 | Reserved | 0 | Reserved for future use |
| 3 | Forbidden | 0 | Set to 0 to avoid emulation of extension codeword |
| 4-8 | Reserved | 0 | Reserved for future use |

Table x.44/H.242 - Horizontal macroblock size MPU

| Bit | Description |
| --- | --- |
| 00000000 | Forbidden |
| 00000001 - 10000000 | valid mpuHorizMBs |
| 10000001 - 11111111 | Forbidden |

mpuHorizMBs indicates the horizontal size in macroblocks of the minimum picture unit for reference picture pruning using H.263 Annex U.

Table x.45/H.242 - Vertical macroblock size MPU

| Bit | Description |
| --- | --- |
| 00000000 | Forbidden |
| 00000001 - 01001000 | valid mpuVertMBs |
| 01001001 - 11111111 | Forbidden |

mpuVertMBs indicates the vertical size in macroblocks of the minimum picture unit for reference picture pruning using H.263 Annex U.

<Text describing total partition memory (mpuTotalByte1, 2, and 3) will be added here>

[End New Section]

# Changes to Table A.1/H.221 – BAS numerical values

A new capability value is being added in Table A1/H.221 to allow the use of the second additional capabilities byte as described above in section 3:

(101) [26]	H.263 (2000)

# Changes to section A.6/H.221 – Video, MBE and encryption capabilities (101)

Explanatory text is being added to section A.6 to describe the new “H.263 (2000)” capability. Editorial corrections are made to the existing text to synchronize it with the values of  table A.1/H.221.

[Begin Correction]

…

4/29.97	Can decode video, having a minimum picture interval of 4/29.97 seconds, to Recommendation H.261.

H.263 (2000)	Can accept <H.262/H.263> MBE with second additional H.263 capabilities as described in section 5.2.4.6/H.242.

Video-MPEG-1	Can decode video to ISO/IEC 11172-2 (“MPEG-1”).

Esc-CF	Capability to accept escape code (111) [0].

Encryp.	Capable of handling signals on the ECS channel.

MBE-cap	Can handle multiple-byte extensions messages in the BAS position, those beginning with codes in the range (111) [25-31], in addition to other values.

…

[End Correction]

# Options Indicator table and related text

Appendix II/H.263 calls for a new profile 0 and a new profile 4 and makes textual changes to the existing “profile levels” to be called “profiles”. Therefore,

H.263 profiles – level 1 		becomes 	H.263 profile 1

H.263 profiles – level 1 and 2 	becomes 	H.263 profile 2

H.263 profiles – level 1, 2, and 3 	becomes 	H.263 profile 3.

The table (to be numbered x.16) and its explanatory text are being updated as follows:

[Begin Correction]

Table x.16/H.242 – Options Indicator

| Bit |  | OptionsIndicator |
| --- | --- | --- |
| 000 | H.263 options are signalled separately and/or H.263 scalability mode defined in Annex O/H.263 is supported. |  |
| 001 | Inherit H.263 options from immediately larger format. |  |
| 010 | H.263 profile 1 supported. |  |
| 011 | H.263 profile 2 supported. |  |
| 100 | H.263 profile 3 supported. |  |
| 101 | No additional H.263 options are supported. |  |
| 110 | H.263 profile 4 supported |  |
| 111 | Reserved for future profile extension. |  |

Inheritance shall imply inheritance of the H.263 options signalled in the additional H.263 capabilities only.

H.263 profiles 0, 1, 2, 3, and 4 consist of the H.263 options listed below. The Annexes mentioned are the Annexes of Recommendation H.263 and the profiles are the same as those specified in Appendix II/H.263.

H.263 profile 0 is composed of the following single mode: Advanced Prediction (Annex F). Profile 0 is not signalled in OptionsIndicator (table x.16/H.242), and instead, it shall be signalled using the AMP bit of the options byte as described in section 5.2.4.2.

H.263 profile 1 is composed of the following modes (it does not include the Advanced Prediction mode of Profile 0); Advanced Intra Coding (Annex I), Deblocking Filter (Annex J), Supplemental Enhancement Information (Full Frame Freeze Only) (Annex L, Subclauses L.4) and Modified Quantization (Annex T).

H.263 profile 2 is composed of the modes described above for Profile 1, plus the following: Unrestricted Motion Vectors (With UUI = "1" Sufficient) (Annex D), Slice Structured Mode (all submodes) (Annex K) and Reference Picture Resampling (Implicit Factor-of-4 Mode only) (Annex P).

H.263 profile 3 is composed of all of the modes described above for Profiles 0, 1, and 2, plus the following additional modes: Improved PB-frames (Annex M), Independent Segment Decoding (Annex R) and Alternate Inter VLC (Annex S).

H.263 profile 4 is composed  of all of the modes described above for Profile 0 and 1, plus the following additional modes: Reference Picture Selection (Annex N), Reference Picture Resampling (Implicit Factor-of-4 Mode only) (Annex P), and Enhanced Reference Picture Selection (Annex U).

[End Correction]

* Contact:	Patrick Luthi	Phone: +1 978-292-4354

PictureTel Corp., USA	E-mail: luthip@pictel.com

Michael Horowitz	Phone: +1 512 372 7091

Polycom Inc., USA	E-mail: mhorowitz@austin.polycom.com

Tom-Ivar Johansen	Phone: +47 67 838 413

Tandberg Telecom AS, Norway	E-mail: tij@tandberg.no