| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001 | Document VCEG-M08 Filename: VCEG-M08.doc Generated: 26 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Michael Horowitz Polycom Incorporated 5000 Plaza on the Lake Austin, TX 78746 USA | Tel: Fax: Email: | (512) 372-7091 (512) 372-7001 mhorowitz@austin.polycom.com |
| Title: | Objective Coding Performance of TML 5.9 and H.263+ |  |  |
| Purpose: | Information |  |  |

_____________________________

# 1. Introduction

In this document and associated document VCEG-M08.xls, the objective coding performance of the H.26L test model TML 5.9 without B-pictures is compared to that of H.263+ with annexes F, I, J and T.  In addition, the coding performance of TML 5.2 is compared.  In Section 2, details of the coding experiments are described.   Section 3 contains a summary of experimental data.  The data are presented in VCEG-M08.xls.

# 2. Experimental Details

The TML 5.2 encoder from mid 2000, TML 5.9 encoder from March 2001 and the University of British Columbia’s (UBC) H.263+ version 0.2 encoder from July of 1999 were compared.  The comparison was performed in the context of the common conditions described in document VCEG-L38 except that the sequence Silent Voice was not analyzed.  See Table 1 for a list of source sequences and their attributes.

| # Source Sequence | # Format | # Frame Rate | # No. Frames Coded |
| --- | --- | --- | --- |
| Container | QCIF | 10 fps | 100 |
| News | QCIF | 10 fps | 100 |
| Foreman | QCIF | 10 fps | 100 |
| Paris | CIF | 15 fps | 150 |
| Mobile | CIF | 30 fps | 300 |
| Tempete | CIF | 30 fps | 260 |

Table 1.  Source Sequences And Their Attributes.

Parameters are presented to both the TML and UBC encoders via specially formatted input files.  Some of the more relevant parameters are presented in Tables 2 and 3 for the TML and UBC encoders respectively.  For completeness, example input files for TML 5.9 and UBC 0.2 are reproduced in their entirety in the Appendix.

| Quantization parameter for first picture | {16,20,24,28} |
| --- | --- |
| Quantization parameters for remaining pictures | {16,20,24,28} |
| Hadamard transform | Used |
| Max search range | 15 |
| Number of frames used for motion search | 5 |
| Error robustness | Off |
| Inter block search 16x16 | Enabled |
| Inter block search 16x8 | Enabled |
| Inter block search 8x16 | Enabled |
| Inter block search 8x8 | Enabled |
| Inter block search 8x4 | Enabled |
| Inter block search 4x8 | Enabled |
| Inter block search 4x4 | Enabled |
| Slice mode | Off |

Table 2.  TML Encoder Input Parameters.

| Quantization parameter for first picture | {5,8,13,21} |
| --- | --- |
| Quantization parameters for remaining pictures | {5,8,13,21} |
| Max search range | 15 |
| R-D coding | On |
| R-D Quantization | Off |
| Intra macroblock refresh | Off |
| Annexes F, I, J, and T | On |

Table 3.  UBC Encoder Input Parameters.

# 3. Results

This section contains a summary of the experimental data presented in VCEG-M08.xls.   Specifically, PSNR for Y, U, and V data averaged over all pictures and data for the first (intra) coded picture for TML 5.2, 5.9 and UBC 0.2 encoders are presented.   Throughout this Section, inter-frame results include data from the first (intra) frame.  The summary includes a calculation of the difference SNR (DSNR) between TML 5.9 and H.263+ as well as a bit rate savings estimate of TML 5.9 over H.263+ for each source sequence.

## 3.1 Difference SNR

The DSNR for TML 5.9 and H.263+ (annexes F, I, J and T) was computed for inter- and intra-frame luminance PSNR rate-distortion curves using avsnr3.  Two DSNR values are provided for each case.  One is the DSNR over the entire range of the curves.  The second is the DSNR for the so called “sweet spot” which is the region of the TML 5.9 rate-distortion curve starting at the point corresponding to quantization parameter 20 and ending at the point corresponding to quantization parameter 24.  A detailed description of the algorithm used to compute the DSNR may be found in VCEG-M33.  The DSNR results are presented in Table 4.

|  | Inter-Frame Total DSNR (dB) | Inter-Frame Sweet-Spot DSNR (dB) | Intra-Frame Total DSNR (dB) |
| --- | --- | --- | --- |
| Paris | 1.34 | 1.44 | 0.21 |
| Mobile | 4.12 | 4.27 | 0.25 |
| Tempete | 2.52 | 2.65 | 0.13 |
| News | 1.52 | 1.65 | -0.03 |
| Foreman | 3.20 | 3.46 | 0.152 |
| Container | 1.84 | 2.22 | 0.03 |

Table 4.  TML 5.9 and H.263+ (Annexes F, I, J, and T) DSNR Values for Inter and Intra Frames.

## 3.2 Bit Rate Savings

The bit rate savings that TML 5.9 provides over H.263+ (annexes F, I, J, and T) are presented in Table 5 for inter- and intra-frame coding.  The values in the table were derived using a technique introduced to the author by Dr. Gary Sullivan.  To employ this technique, one chooses a luminance PSNR value of interest.  Next, the H.263+ bit rate values are scaled such that the H.263+ and the TML 5.9 rate-distortion curves intersect at the PSNR value of interest.  The bit rate savings that TML provides over H.263+ is  (1 – scaleFactor) * 100 percent where scaleFactor is the amount that the H.263+ bit rate values were scaled to achieve the intersection.  For example, if we wish the H.263+ and TML 5.9 rate-distortion curves for luminance to intersect at 32dB PSNR for Paris, then it is necessary multiply the H.263+ bit rate values by 0.82.  Therefore the bit rate savings that TML provides over H.263+ for Paris at a PSNR 32 dB is 18 percent.  In Table 5, bit rate savings for the inter-frame coding analysis are presented for three different PSNR values, 29 dB, 32 dB, and 34 dB, while only one value, 34 dB, was used for the intra-frame analysis.  The inter-frame bit rate savings data are shown graphically in Figure 1.

|  | Bit Rate Savings @ Average PSNR 29dB | Bit Rate Savings @ Average PSNR 32dB | Bit Rate Savings @ Average PSNR 34dB | Bit Savings @ PSNR 34dB (Intra-Frame) |
| --- | --- | --- | --- | --- |
| Paris | 18% | 23% | 26% | 11% |
| Mobile | 57% | 54% | 58% | 5% |
| Tempete | 44% | 45% | 42% | 1% |
| News | 11% | 21% | 27% | 10% |
| Foreman | 37% | 45% | 48% | 26% |
| Container | 23% | 34% | 38% | 13% |
| Average | 31.6% | 37.0% | 39.8% | 11% |

Table 5.  Bit Rate and Bit Savings of TML 5.9 over H.263+ (Annexes F, I, J, and T).

Figure 1.  Inter-Frame Bit Rate Savings of TML 5.9 over H.263+ (Annexes F, I, J, and T).

## 3.3 Comments

While an average bit rate savings of 37 percent (@32dB) for TML 5.9 over H.263+ is impressive, the VCEG goal is to achieve a savings of 50 percent.  Technically the 50 percent bit rate savings goal was made with respect to H.263+ annexes D, F, I, J, and T. Annex D was not included in this analysis because TML 5.9 has a maximum search range of 16 samples (in this analysis the search range was limited to 15 samples).  Consequently, the H.263+ encoder was also limited to a 15 samples thereby obviating any advantage added by using annex D (motion vectors over picture boundaries is part of annex F and is therefore included in this analysis).   In addition, it is known that coding with respect to multiple reference pictures provides significant coding gains for a wide variety of sequences.  As TML 5.9 uses multiple reference pictures (5 were used in this analysis), it is possible that a large portion of the TML gain stems from the multiple reference picture feature.  Another point of interest and possible area for further improvement is the intra coding.  It is surprising (at least to the author) that the more sophisticated TML intra coding techniques provide such modest gains over H.263+.  Finally, it is important to keep in mind that B-pictures were not used in this analysis so the differences between TML 5.9 and H.263+ (and TML 5.2) are understated in this analysis.

# Appendix

For completeness, example input parameter files for the TML and UBC encoders for the sequence container are reproduced in their entirety below.

Example input file for TML 5.9.

100 .................. 1:  Number of frames to be coded

28 .................... 2:  Quantization parameter for first frame (intra) (0-31)

28 .................... 3:  Quantization parameter for remaining frames (0-31)

2  ..................... 4:  Number of frames to be skipped in input (e.g 2 will code every third frame)

1  ......................5:  Hadamard transform (0=not used, 1=used)

15 .....................6:  Max search range

5  ......................7:  Number of previous frames used for inter motion search (1-5)

0  ......................8:  Postfilter, Not used in this version

0  ......................9:  Image format (0=QCIF 1=CIF)

0  .....................10:  Extra intra macroblock updates (0=off, N: One GOB every N frames are intra coded)

0  .....................11:  Write internal decoded image to file (0=OFF 1=ON)

1  .....................12:  Inter block search 16x16 (0=disable, 1=enable)

1  .....................13:  Inter block search 16x8  (0=disable, 1=enable)

1  .....................14:  Inter block search 8x16  (0=disable, 1=enable)

1  .....................15:  Inter block search 8x8   (0=disable, 1=enable)

1  .....................16:  Inter block search 8x4   (0=disable, 1=enable)

1  .....................17:  Inter block search 4x8   (0=disable, 1=enable)

1  .....................18:  Inter block search 4x4   (0=disable, 1=enable)

containerQCIF.yuv    ....19:  Input sequence, YUV 4:2:0

container.26l  …............20:  Output bitstream in Telenor H.26L format

0  .....................21:  NAL mode (0=TML bitstream, 1=Partition file)

0  .....................22:  Slice mode (0=off 1=fixed #mb in slice 2=fixed #bytes in slice 3=use callback)

100  .................23:  Slice argument (Arguments to modes 1 and 2 above)

0  .....................24:  Number of B frames inserted (0=not used)

10 ....................25:  Quantization parameter for B frames (0-31)

Example input file for UBC 0.2.  The text has been reformatted for brevity.

frame_start                         		0		image to start coding

frame_end                          			299		image to stop coding

use_wholefile                    			off		on: use all pictures  off: use seq_length pictures

seq_length                              		300		number of pictures in the sequence

seqfilename                             		./containerQCIF.yuv 	original sequence

outputfilename                          		off		filename for reconstructed frames

streamname                              		off		filename for bitstream

datafilename                            		off		filename for data file

diff_filename                           		off		filename for the difference images

writediff                               		off		write difference image to file diff_filename

source_format                           		2		n=1: SQCIF   n=2: QCIF   n=3: CIF

ref_frame_rate                          		30.00000	reference frame rate

orig_frameskip                          		0		frames skipped in original w.r.t. reference frame rate

chosen_frameskip                        		2		frames to skip between each encoded frame

rate_control_method                     		0		rate control off

targetrate                              		64000		target bitrate in bits/s, default is variable bitrate

seek_dist                               		15		integer pel search window

p_picture_quant                         		21		inter quantization parameter

i_picture_quant                         		21		QP for INTRA frames

b_picture_quant                         		13		to set QP for true B pictures (annex O)

bquant                                  		6		BQUANT parameter for PB-frame coding

unrestricted_mv_mode                 		off		Annex D

syntax_arith_coding                     		off		Annex E

advanced_prediction_mode                	on		Annex F

pb_frames                               		off		Annex G

advanced_intra_coding                   		on		Annex I

deblocking_filter                       		on		Annex J

slice_structure                         		off		Annex K

improved_pb_frames                      		off		Annex M

reference_picture_selection             		off		Annex N

successive_b_frames                     		0		Annex O

reference_picture_resampling            	off		Annex P

reduced_resolution_update                       	off		Annex Q

independently_segmented_decoding        	off		Annex R

alternative_inter_vlc              		off		Annex S

modified_quantization           		on		Annex T

write_repeated                       		off		write repeated reconstructed frames to disk

tracefilename                         		off		write trace to tracefile trace.intra/trace

start_rate_control                                            0		start rate control after  specified  % of sequence

headerlength                          		0		headerlength on concatenated 4:1:1 YUV input file

use_gobsync                          		0		insert sync after each n GOB (slice): 0 = no syncs

intra_mb_refresh                   		0		intra macroblock refresh rate

force_intra                             		0		intra frame refresh rate

roaming_quantization           		off		Roaming quantization

gob_lost                                		off		Simulation variable for encoder error concealment

prob_loss_packet                  		0.10000		Simulation variable for encoder error concealment

rd_coding                              		on      		Rate-Distortion mode decision and motion estimation

rd_quant                                		off      		Trellis based Rate-Distortion quantization

enhancement_config_file      		off

draw_mv_field                      		off