| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (VCEG) _________________ 13th meeting, Austin, Texas, USA, April 2-4, 2001 | Document VCEG-M22 Filename: VCEG-M22.doc Generated: 27 Mar 2001 |
| --- | --- |

| Question: | Q6/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Jani Lainema, Marta Karczewicz Nokia Research Center 6000 Connection Drive Irving, TX 75039, U.S.A. | Tel: Fax: Email: | +1 972 894 4045 +1 972 894 4589 jani.lainema@nokia.com |
| Title: | Further Improvements on TML Loop Filtering |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

This document proposes modifications to the TML loop filter in order to improve the visual quality of decoded sequences.

## Proposed Modifications

In low bitrates TML 5.9 produces occasional visual artefacts due to coarse quantization in transform coding, limited block size and rate-distortion optimizations. It turns out that with small adjustments to the current filter we can reduce these artefacts without sacrificing image sharpness. Proposed modifications to the TML filter are:

- Increasing the Alpha parameter (to allow more pixels to be filtered)

- Increasing the Beta parameter (to filter areas with higher frequencies)

- Allow the maximum of two pixels to be filtered on one side of the block boundary regardless of the strength parameter (exception: for chroma this is allowed only if strength parameter on that side of the boundary is equal to 3).

Proposed Alpha and Beta parameters are listed below and their relationship to the old ones is demonstrated in the Figures 1 and 2.

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 6 | 6 | 14 | 14 | 14 | 23 | 33 | 33 | 44 | 55 | 67 | 80 | 93 | 120 | 134 | 163 | 178 | 224 | 273 | 306 | 374 | 426 | 498 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 3 | 4 | 5 | 5 | 6 | 7 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |

Figure 1: The relationship between the proposed and TML 5.9 Alpha parameter.

Figure 2: The relationship between the proposed and TML 5.9 Beta parameter.

In the results below proposed modifications are integrated to the proposed output of the Low Complexity Loop Filtering Core Experiment (VCEG-M21) and compared against TML 5.9 results. More exact simulation results and PSNR curves can be found in VCEG-M22.xls.

Table 1: Loop filter complexity with proposed changes (operations per Macroblock)

| QP | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average | Worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 2193 | 6276 | 2536 | 2728 | 2592 | 5171 | 5318 | 3831 | 25984 |
| 20 | 1392 | 5843 | 2266 | 2510 | 2318 | 4603 | 4396 | 3333 | 25984 |
| 24 | 991 | 5390 | 1926 | 2224 | 1978 | 3513 | 3281 | 2758 | 25984 |
| 28 | 774 | 5061 | 1695 | 1886 | 1673 | 2672 | 2560 | 2332 | 25984 |
| Average | 1338 | 5643 | 2106 | 2337 | 2140 | 3990 | 3889 | 3063 | 25984 |

Table 2: Complexity relative to TML 5.9

| QP | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average | Worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 0.98 | 0.89 | 0.90 | 0.91 | 0.83 | 0.82 | 0.84 | 0.88 | 0.91 |
| 20 | 0.96 | 0.88 | 0.89 | 0.91 | 0.84 | 0.77 | 0.83 | 0.87 | 0.91 |
| 24 | 0.94 | 0.88 | 0.87 | 0.94 | 0.86 | 0.72 | 0.82 | 0.86 | 0.91 |
| 28 | 0.95 | 0.88 | 0.87 | 0.92 | 0.84 | 0.66 | 0.79 | 0.84 | 0.91 |
| Average | 0.96 | 0.88 | 0.88 | 0.92 | 0.84 | 0.74 | 0.82 | 0.86 | 0.91 |

Table 3: PSNR differences with respect to TML 5.9

|  | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Luma Sweet Spot | -0.02 | 0.12 | 0.09 | 0.13 | 0.10 | 0.04 | 0.02 | 0.07 |
| Luma Total | -0.02 | 0.11 | 0.07 | 0.13 | 0.09 | 0.05 | 0.02 | 0.06 |
| Chroma Sweet Spot | N/A | 0.12 | N/A | 0.14 | 0.21 | 0.16 | 0.14 | 0.16 |
| Chroma Total | N/A | 0.15 | N/A | 0.11 | 0.18 | 0.15 | 0.13 | 0.14 |

Table 4: Summary of results

| Average Complexity Reduction | 14% |
| --- | --- |
| Luma PSNR improvement (sweet spot) | 0.07 |
| Luma PSNR improvement | 0.06 |
| Chroma PSNR improvement (sweet spot) | 0.16 |
| Chroma PSNR improvement | 0.14 |

# Examples

Following figures demonstrate visual improvements in two extreme cases with proposed modifications to the TML 5.9 filter.

Figure 3: Part of the News sequence, QP = 24, 1 reference frame. TML 5.9 on the left, TML 5.9 with proposed changes on the right.

Figure 4: Part of the News sequence, QP = 24, 5 reference frames. TML 5.9 on the left, TML 5.9 with proposed changes on the right.

# Summary

Visual comparison of the TML 5.9 with and without proposed changes will be provided in the Austin meeting. Results obtained suggest that the compression efficiency and visual quality can be improved by introducing small modifications to the TML loop filter. Given the results, authors of this contribution would like to ask Video Coding Experts Group to adopt the proposed changes to the next version of TML.

Patent Statement and Licensing Declaration

(Typically one per ITU-T Recommendation)

This declaration does not represent an implied license grant

Please return to:	Director	Place des Nations

Telecommunication Standardization Bureau	CH-1211 Geneva 20, Switzerland

International Telecommunication Union	Fax: +41 22 730 5853

| Patent Holder/Organization: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Legal Name |  | Nokia Inc. |  |  |  |
| Contact for license application: |  |  |  |  |  |
| Name & Department |  |  |  |  |  |
| Address |  |  |  |  |  |
| Tel. |  |  |  |  |  |
| Fax |  |  |  |  |  |
| E-mail |  |  |  |  |  |
| ITU-T Recommendation: |  |  |  |  |  |
| Number |  | H.26L |  |  |  |
| Title |  |  |  |  |  |
| ### Licensing declaration |  |  |  |  |  |
| The Patent Holder believes to hold granted patents and/or pending applications, whose use would be required to implement the above ITU-T Recommendation and hereby declares, in accordance with the Statement on ITU-T Patent Policy (see ITU-T web site), that (check one box only). |  |  |  |  |  |
|  | 1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation. |  |  |  |  |
|  | 2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation. Such negotiations are left to the parties concerned and are performed outside the ITU-T. |  |  |  |  |
|  | 3 The Patent Holder is unwilling to grant licenses according to the provisions of either 1 or 2 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation are affected. - a description of the patent claims covering the Recommendation; |  |  |  |  |
| Signature |  |  |  |  |  |
| Organization |  |  |  |  |  |
| Name of authorized person |  |  |  |  |  |
| Title of authorized person |  |  |  |  |  |
| Signature |  |  |  |  |  |
| Place, Date |  |  |  |  |  |