| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001 | Document VCEG-M45 Filename: VCEG-M45.doc Generated: 26 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wedi Institut fuer Theoretische Nachrichtentechnik und Informationsverarbeitung, University of Hannover Applestr. 9A D-30167 Hannover, Germany | Tel: Fax: Email | +49 511 762 5304 +49 511 762 5333 wedi@tnt.uni-hannover.de |
| Title: | 1/8-pel Displacement Vector Resolution for TML-6 |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

In the document VCEG-L20 a complexity reduced motion compensated prediction with 1/8-pel displacement vector (DV) resolution was presented. In order to reduce the complexity a new interpolation filter was presented that directly interpolates a specific subpel position. In VCEG-L20 only objective rate distortion plots are given for the experimental results. Thus, the main reason for this contribution is to present subjective results at the meeting in Austin.

The 1/8-pel displacement vector resolution is implemented in TML-6. The experimental results are equivalent to the results of VCEG-L20. A gain up to 1.0 dB PSNR is obtained with the 1/8-pel DV resolution compared to 1/4-pel DV resolution in TML-6. Due to the direct interpolation filter this gain is obtained while the decoder complexity is the same than in TML-6. A further significant gain up to 0.5 dB PSNR is obtained by increasing the filter length to 8-tap.

# Motion Compensated Prediction using 1/8-pel displacement vector resolution

## Direct Interpolation Scheme

For the interpolation of subpel positions the direct interpolation scheme is used that is presented in VCEG-L20 and VCEG-M44. In contrast to the subsequent interpolation scheme, where all subpel positions of the whole image grid are interpolated, the direct interpolation scheme only interpolates the subpel positions that are used in the motion compensated prediction. For this purpose two different filters are applied that directly interpolate a specific subpel position (6-tap and 8-tap). With the 6-tap direct interpolation filter the decoder complexity of the 1/8-pel codec is the same than the decoder complexity of the TML-6 (1/4-pel).

## Estimation of Displacement Vectors

In a motion compensated prediction scheme the motion vector and the block-size pattern (e.g. 16x16, four 8x8, etc.) have to be estimated for each macroblock. This is done by the following 5 steps:

- Make a full search to find the best 1/1-pel vector.

- Check the 8 1/2-pel positions around the best 1/1-pel vector in order to find the best 1/2-pel vector.

- Check the 8 1/4-pel positions around the best 1/2-pel vector in order to find the best 1/4-pel vector.

- Check the 8 1/8-pel positions around the best 1/4-pel vector in order to find the best 1/8-pel vector.

- Select the motion vector and block-size pattern, which produces the lowest rate-distortion cost.

# Experimental Results

Table 1 shows the test-sequences that are used for the experimental results.

| Test-sequence | Res. | Frame rate | No.of frames |
| --- | --- | --- | --- |
| Mobile & Calendar | CIF | 30 | 300 |
| Flower Garden | CIF | 30 | 250 |
| Tempete | CIF | 30 | 260 |
| Paris | CIF | 15 | 300 |
| Silent Voice | QCIF | 15 | 300 |
| Container | QCIF | 10 | 300 |
| News | QCIF | 10 | 300 |
| Foreman | QCIF | 10 | 300 |

Table 1: Applied test-sequences.

For the objective results, rate-distortion plots are given in the Appendix. Each rate-distortion plot contains the following curves:

- TML-6: 1/4-pel with 6-tap and bilinear interpolation filter

- 1/8-pel with 6-tap direct interpolation filter (6-tap)

- 1/8-pel with 6-tap direct interpolation filter (8-tap)

The subjective results will be presented at the meeting in Austin.

Summary

In this contribution a motion compensated prediction scheme was presented that uses displacement vectors with 1/8-pel resolution and direct interpolation filters. Results for two different interpolation filters are given (6-tap and 8-tap filter). With the 6-tap filter the decoder complexity of the 1/8-pel codec is the same than the decoder complexity of the TML-6 (1/4-pel) codec. Nevertheless, a gain up to 1.0 dB PSNR is obtained compared to TML-6. A further significant gain up to 0.5 dB PSNR is obtained by increasing the filter length to 8-tap.

# References

VCEG-L20:	T. Wedi, “Complexity Reduced Motion Compensated Prediction with 1/8-pel Displacement Vector Resolution”, ITU-T SG16/Q6, Eibsee, Germany, Jan. 01.

VCEG-M44:	T. Wedi, “Direct Interpolation Filters in TML-6”, ITU-T SG16/Q6, Austin, Texas, USA, April 01.

Appendix