| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Twelfth Meeting: Austin, USA, 2-5 March, 2001 | Document VCEG-M50 Filename: VCEG-M50.doc Generated: 21 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Antti Hallapuro, Marta Karczewicz Nokia Research Center, Irving, Texas 75039, USA | Tel: Fax: Email: | +972 894 4188 +972 894 4589 marta.karczewicz@nokia.com |
| Title: | Complexity Analysis of H.26L |  |  |
| Purpose: | Proposal |  |  |

# Introduction

This document analyses H.26L complexity. Complexity of the encoder and decoder as well as most important individual routines are measured. The results are compared against the H.263+ results.

Also memory requirements are described briefly in the end.

# software version

Nokia's highly optimized C language implementation of H.26L Test Model (TML) is used in the simulations. Nokia's implementation includes almost all features found in TML version 5 [1] with an exception of the error resilience tools and B-frames.

Two features of the H.26L are used, namely reduced complexity loop filter [2] and direct interpolation method  [3], which have been accepted into TML, but have not been included into the official TML implementation at the time of writing this document. These features have large impact on complexity and memory requirements in decoder.

# encoder algorithm

Speed and performance of the H.26L encoder depends on implementation of few key algorithms that are used in encoding process. In this chapter the most important ones such as motion estimation, quantization and inter/intra selection are described in the context of Nokia encoder.

## Motion estimation

Two motion estimation algorithms are used. Here is a very compact description of both:

- Full search algorithm. Search proceeds in three steps. First, all integer-pel positions in a quadratic search window are checked and the best one is selected. Width and height of the search window are halved for blocks smaller than 16x16. In addition, search range is further halved for reference frames other than most recent one. Center of the search window is located at the predicted motion vector. For 16x16, 16x8, 8x16 and 8x8 blocks, Successive Elimination Algorithm (SEA) is used to quickly reject motion vectors [4]. Next, 9 ½-pel positions closest to the best integer-pel position are checked and the best is selected. Finally 9 ¼-pel positions closest to the best ½-pel position are checked and the best position is the final motion vector of the current motion block. During ½-pel and ¼-pel search, Hadamard transform is used in error computation in addition to simple SAD.

- Optimized algorithm. Algorithm proceeds in four steps of which the first step is only performed for 16x16 motion blocks and for the most recent reference frame. First, full search is performed for the 16x16 blocks in the subsampled domain, thus reducing number of checked position to ¼ compared to integer-pel full search. Next, integer-pel search in performed using 5x5 search window. Search window is centered at the best position from step 1 (16x16 blocks) or at the predicted vector. If long term memory is used, search window is further shrunk to 3x3 for blocks smaller than 16x16 in other than the most recent reference frame. In the third step, 9 ½-pel positions closest to the best integer-pel position are checked and the best is selected. Finally 5 ¼-pel positions closest to the best ½-pel position are checked and the best position is the final motion vector of the current motion block. Hadamard transform is used only in ¼-pel search.

The search range is fixed to 16 integer pixels for QCIF sequences and 8 for CIF sequence in the simulations. So, for QCIF in full integer-pel search, (2*16+1)^2 = 33*33 positions are checked (16x16 blocks). If long term memory is used, search range is further reduced for other than the most recent reference frame.

## Quantization

TML software uses RD-quantization for all transform coefficient quantization. This form of quantization is also used in Nokia implementation.

## Intra/Inter selection

Since it is possible to have intra and inter macroblocks in inter frames, we have to choose the best mode for each macroblock. Mode is selected by encoding macroblock as intra 4x4, intra 16x16 and inter, and then choosing the best based on the error measure.

In Nokia optimized implementation there is a possibility to use faster intra/inter decision algorithm. Macroblock is coded first as intra 16x16 and inter. Then, only if intra 16x16 error is smaller than inter error by some QP dependent margin, we check also intra 4x4 modes.

# simulations

Simulation environment is listed in the table below. The software was compiled with full optimizations enabled.

- CPU:	 733 MHz Pentium III

- Memory:	 256 MB RDDRAM

- OS:	 Microsoft NT 4 + service pack 5

- Compiler: Microsoft Visual C 6.0

Simulation results are obtained for the following implementations of H.26L and H.263+:

- Nokia's implementation of TML providing same compression efficiency as ITU-T's reference software. The results obtained using this software will be denoted as "Ref" (1 reference frame used) or "Ref5" (5 reference frames used).

- Nokia's implementation of TML with optimized motion estimation and intra/inter decision. The results obtained using this software will be denoted as "Opt" (1 reference frame used) or "Opt5" (5 reference frames used).

- Nokia's implementation of H.263+ (decoder). Results will be denoted as "H.263+"

- ITUT-T reference implementation of H.263+ provided by University of British Columbia. Results will be denoted as "H.263+ anchor"

In TML decoder simulations bitstreams generated using 1 reference frame are used since decoder complexity depends only to a small extend on the number of the reference frames.

Simulations parameters for H.26L encoder:

- QPs used:  13, 16, 19, 22, 25 and 28.

- Search range in motion estimation:  16 for QCIF 10 fps and 8 for CIF 30 fps.

- Motion block sizes used:  16x16, 16x8, 8x16, 8x8, 8x4 and 4x8. 4x4 not used.

Simulation parameters for Nokia H.263+ coder:

- QPs used: 5, 7, 10, 14, 18 and 26.

- Annexes used: I, J, D and T.

Three sequences are used in simulations: Container (QCIF format, 10 frames/sec.), Foreman (QCIF format, 10 frames/sec.) and Mobile (CIF format, 30 frames/sec.).

Overall complexity of the codec is measured using ANSI C clock() function. Complexity distribution among individual functions is measured using RDTSC instruction that can be found in Pentium and newer Intel x86 CPUs.

## Encoder complexity

Figure 1, Figure 2 and Figure 3 include performance graphs comparing complexity of the different encoder implementations described in Section 4. Number of seconds required to encode a single frame is given. Comparison of the compression efficiency (bitrate vs. PSNR plots) is given in Figure 4, Figure 5 and Figure 6.

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5

Figure 6

The optimized implementation of TML is 45%-60% less complex than the reference implementation when 1 reference frame is used. The difference is slightly larger when 5 reference frames are used – 55-63%. In both cases the drop in performance due to the algorithmic simplifications is less than 0.2 dB and the highest drop occurs for high bitrates. Usage of 5 reference frames instead of 1 increase encoder complexity 2.8-3.3 times for the reference implementation and 2.5-3 times for the optimized implementation. On a given platform QCIF sequences, 10 frames/sec are encoded real time when optimized implementation with 1 reference frame is used.

## Encoder complexity distribution

Distribution of the encoding time at different bitrates between different functional parts of the encoder is given in Figure 7. Results are obtained using optimized implementation of H.26L with one reference frame ("Opt"). The distribution is given only for the Forman sequence however for other sequences the distribution is similar.

Description of each segment of the graph:

Other: 		Overhead calculations including frame I/O operations, copying of pixel data, etc.

VLC: 		Variable length coding and bitstream output.

Subsampling: 	Subsampling of original and reference images by 2 in both directions (used subsequently in the motion search).

Loop filter: 		Deblocking loop filter.

Tranf. & quant.: 	Forward/inverse transform of prediction error and quantization/dequantization.

Interpolation: 	Routines used to obtain ½ and ¼-pixels interpolation.

Intra prediction: 	Luminance and chrominance intra prediction.

Motion estimation: 	Motion estimation including motion vector prediction.

Figure 7

As it can be seen in Figure 7 the most complex part of the encoder is motion estimation. On average it takes 65% of the encoding time.

## Decoder complexity

Comparison of TML and H.263+ decoder complexity is given in Figure 8, Figure 9 and Figure 10. TML decoder is from 15 to 40% more complex than H.263+ decoder.

Figure 8

Figure 9

Figure 10

## Decoder complexity distribution

Distribution of decoding time between different functional parts of the decoder for all tested sequences is given in Figure 11, Figure 12, and Figure 13.

Figure 11

Figure 12

Figure 13

Descriptions of each segment of the graph is given:

Other: 		this part contains frame I/O operations, copying of pixel data, motion vector prediction, macroblock handling overhead and other minor overhead.

VLD: 		fetching bits from the bitstream and variable length decoding.

Loop filter: 		deblocking loop filter.

Tranf. & quant.: 	contains inverse DCT functions and dequantization.

Interpolation: 	contains interpolation routine that produces ¼ precision pixels.

Intra prediction: 	contains luminance and chrominance intra prediction.

Two most complex part of the decoder are deblocking filter and sub-pixel interpolation. Time taken by the interpolation is fairly constant for a given sequence and does not have strong dependency on bitrate. However since decoder complexity tends to increase with increasing bitrate the share of the overall decoding time taken by interpolation varies from 28% for high bitrates to 46% for low bitrates. Time taken by the loop filter usually increases with the bitrate and it takes 33-46% of the decoding time.

# Analysis of individual tools

The description of the most important routines used in encoder and decoder is given in this chapter. For each routine, it is described what kind of operations it contains. Also, suitability for SIMD (Single Instruction, Multiple Data) processing is mentioned when it is relevant. MMX instructions found in newer x86-compatible processors are an example of SIMD instruction set.

## Motion estimation

Encoder:

This is by far the most important routine in encoder. Core of the ME is SAD calculation that contains memory loads, absolute value operations and additions. By looking at operations only, SAD calculation is good candidate for SIMD processing. However, because of the ¼-pel motion vector precision, only every fourth interpolated pixel is read when calculating SAD. SIMD instructions usually require data to be read consecutively. There are complicated solutions to this, such as rearranging data in CPU registers after loads or prearranging image buffer so that needed pixels can be read consecutively.

Another problem is the overhead in motion estimation caused by large number of different motion block sizes (16x16, 16x8, 8x16, 8x8, 8x4, 4x8 and 4x4). If all motion blocks are scanned, motion estimation loop has to be executed 41 times. However, 4x4 motion blocks don't seem to improve performance. Excluding 4x4 blocks from the motion estimation drops the total number of motion blocks to 25.

There are a couple of Hadamard transforms in final stages of estimation of one motion block. Although Hadamard transform is much simpler than DCT transform, it does still take time – up to 20% of the optimized motion estimation presented in this document. Hadamard transform can be easily implemented on SIMD using 16-bit arithmetic.

## Intra prediction

Encoder/decoder:

Building an intra prediction requires pixel copying, additions and shifts. 16-bit arithmetic is sufficient here. Building one prediction mode is similar in encoder and decoder.

Encoder:

All prediction modes are scanned and best is assigned for each of the blocks. The same Hadamard transform is used here in error calculation as in motion estimation and the same SIMD optimizations apply.

Decoder:

Intra prediction in decoder is much simpler than in encoder since only one, signaled prediction mode has to be build.

## Interpolation

Encoder/decoder:

Interpolation uses 6-tap filter that requires memory loads, multiplications, additions and shifts. Filter coefficients are small enough so that multiplications can be implemented using additions and shifts only, if the multiplication is too costly. Interpolation suits very well SIMD processing and requires only 16-bit arithmetic.

Encoder:

The whole frame is interpolated at once.

Decoder:

Only macroblocks and pixels in those macroblocks that are actually needed are interpolated. This is somewhat more complex and harder to optimize than encoder.

## Transform and quantization

Encoder/decoder:

H.26L uses simple integer transform/inverse transform algorithm having very small coefficients. Therefore transform can be implemented with additions and shifts without using multiplications. If multiplications are used, they have to have 32-bit precision (quantizations/dequantization constants require more than 16 bits). However, at the time of writing there is a plan to replace them with 16-bit versions.

Encoder:

RD-quantization that is used in Nokia implementation requires additions, shifts and processing of many conditional statements.

## Loop filter

Encoder/decoder:

Filter contains many conditional operations as well as pixel copying, absolute value operations, additions, shifts and multiplications with small constants. Multiplications can be replaced with few additions and shifts. 16-bit arithmetic is enough here.

# Memory requirements

Memory requirements of the codec depend heavily on the number of reference frames that are kept in memory. If more than one reference frame is used, other buffers are quite small when compared to reference frame buffers.

In the below tables including memory requirements, N is the number of reference frames. YUV (4:2:0) picture format is assumed.

## Encoder

Table 1 lists memory needed for each buffer in encoder. Memory requirements of the encoder depend on the motion estimation algorithm (e.g., usage of sub-sampling). Numbers listed in the table are for basic encoder implementation that doesn't need additional buffers.

Table 1

| Buffer name | Memory usage (in bytes) |  |
| --- | --- | --- |
|  | QCIF, 176x144 | CIF, 352x288 |
| Original frame | 38016 | 152064 |
| Reconstruction frame | 38016 | 152064 |
| One reference frame | 418176 | 1672704 |
| Motion vectors | 6336 | 25344 |
| Loop filter strength buffers | 1980 | 7920 |
| Loop filter activity buffer (all activities precalculated) | 8192 | 8192 |
| Intra prediction modes | 1584 | 6336 |
| Total | 94124 + N*418176 | 351920 + N*1672704 |

One reference frame contains a luminance frame interpolated in both dimensions and two non-interpolated chrominance frames.

## Decoder

Table 2 lists memory needed for each buffer in decoder. As it can be seen all buffers are same as in encoder with an exception of the reference frame buffer. In the decoder reference frames are stored entirely in non-interpolated form.

Table 2

| Buffer name | Memory usage (in bytes) |  |
| --- | --- | --- |
|  | QCIF, 176x144 | CIF, 352x288 |
| Original frame | 38016 | 152064 |
| Reconstruction frame | 38016 | 152064 |
| One reference frame | 38016 | 152064 |
| Motion vectors | 6336 | 25344 |
| Loop filter strength buffer | 1980 | 7920 |
| Loop filter activity buffer (all activities precalculated) | 8192 | 8192 |
| Intra prediction modes | 1584 | 6336 |
| Total | 94124 + N*38016 | 351920 + N*152064 |

# References

- ITU-T Q.15/SG16, H.26L Test Model Long Term Number 5 (TML-5) draft0, Doc. Q15-K-59, Oct. 2000.

- Bogdan P. Dobrin, Jani Lainema, Marta Karczewicz, "Reduced Complexity Loop Filter", Document  VCEG-L25, Video Coding Experts Group (VCEG) 12th meeting, Eibsee, Germany, January 9-12, 2001.

- Thomas Wedi, " Complexity Reduced Motion Compensated Prediction with 1/8-pel Displacement Vector Resolution ", Document  VCEG-L20, Video Coding Experts Group (VCEG) 12th meeting, Eibsee, Germany, January 9-12, 2001

- Muhammed Z. Coban and Russell M. Mersereau, "A Fast Exhaustive Search Algorithm for Rate-Constrained Motion Estimation", IEEE Trans. on Image Processing, vol. 7, no. 5, pp. 769-773,  May 1998.