| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001 | Document VCEG-M77 Filename: VCEG-M77.doc Generated: 1st May ’01 |
| --- | --- |

| Question | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Viktor Varsa, Marta Karczewicz Nokia Research Center 6000 Connection Drive Irving, TX, 75039 – USA Göran Roth, Rickard Sjöberg Ericsson Radio Systems AB KI/ERA/T/VV SE-164 80 Stockholm, Sweden Thomas Stockhammer, Günther Liebl Institute for Communications Eng. Munich University of Technology 80290 Munich, Germany | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: | +1 972 374 1861 +1 972 894 4589 viktor.varsa@nokia.com +46 8 757 2373 +46 8 757 5550 rickard.sjoberg@era.ericsson.se +49 89 28923473 +49 89 28923490 stockhammer@ei.tum.de |
| Title: | Common Test Conditions for RTP/IP over 3GPP/3GPP2 |  |  |
| Purpose: | Draft text for output document |  |  |

_____________________________

# Summary

These common test conditions are suited for transmission on 3GPP/3GPP2 networks for conversational and streaming video applications based on RTP/IP.

The test conditions serve to evaluate contributions and proposals specific to video applications in 3GPP/3GPP2 networks. Proposals are solicited for:

- Selecting the right coding tools, parameters, levels into these application

- Error resilience tools

- Network Adaptation Layer (Specific tasks of the NAL are: Encapsulation, Prioritization, Interleaving, Stuffing, Synchronization, Transfer of Temporal Information etc.)

To provide controlled environment for experiments, this document defines

- Offline Software Simulator of the 3GPP/3GPP2 radio bearer protocols and RLC layer error patterns (physical layer simulation)

- Set of sequences, bit rates, other parameters to be used in experiments

- A description how to present the results

Third generation mobile systems  defined by 3GPP and 3GPP2 are different in many concepts and aspects (for example Quality of Service guarantees are in UMTS, but not in CDMA-2000). However, these differences are either not relevant for these common conditions or it is assumed and foreseen, that the relevant concepts in the two will converge in the future. Most importantly, 3GPP and 3GPP2 has defined similar user plane protocols for UMTS and CDMA-2000 air interface. Thereby, these common testing conditions are designed to be general and due to the converging trends are hoped to be a valid model for both.

# Simulation Environment

## Overview

Both video source and receiver are assumed to reside in a private operator's network, which consists of a fixed IP-based core network and a radio access network. A video streaming server is directly connected to the operator's core network or both video call terminations are inside the mobile network. Other service models are also viable and supported (for example the streaming server is outside of the operator's network) as long as QoS guarantees are supported throughout the whole transmission path (not only best-effort). The network operator's core network is assumed to be over-provisioned so that the packet loss rate in the core network is negligible and the network resources bottleneck is at the radio interface. Thus any degradation to the video stream results from fading/shadowing errors at the radio interface.

## User plane protocols in 3G

### This section gives a basic overview of 3GPP/3GPP2 air interface user plane protocols. It is not intended to fully implement the stack in the offline software simulator, but rather a simplified model is to be devised from it. Therefore, this section serves as background information.

3GPP and 3GPP2 define similar user plane protocols for UMTS and CDMA-2000 specifications between the mobile station (MS) and radio base station (BS). In Figure 1 the packetization of application packets is shown in the protocol stack.

Figure 1: Packetization through the user plane protocol stack (CDMA-2000)

The figure shows the CDMA-2000 stack with native naming. For UMTS the only differences are:

- no PPP protocol is used, but the link layer (called RLC) includes signaling of the RLC payload boundaries,

- a physical layer frame is not split further into logical nits, but the size of a physical layer frame can be set up more flexibly.

In CDMA-2000 a physical layer frame can be divided into a number of Logical Transmission Units (LTU). The LTU is the smallest unit with a CRC to detect possible bit-errors. The size of an LTU (in bytes) is fixed for a session.

An RLC-PDU in UMTS is of fixed length and is determined when the bearer is setup, for example 80 octets.

For both it can be assumed, that one link layer frame is packed into one physical layer logical unit (LTU). This means, that each link layer frame can have a frame quality indicator (FQI) that indicates if there are bit-errors detected by the physical layer CRC.

An RTP/UDP/IP packet is packed into one PDCP/PPP packet which will become an RLC-SDU (RLC is used in this section to refer to PDUs or SDUs of both UMTS and CDMA-2000 link layers. RLC is the UMTS and RLP is the CDMA-2000 terminology).. Video packets are by nature of varying length, so RLC-SDUs will be of varying length as well. In the case that an RLC-SDU is larger than an RLC-PDU the SDU must be segmented into several PDUs. This is one main function of the RLC layer. Normally an SDU will not fill an integer number of PDUs such that some space will be left over. In case the flow of variable size RLC-SDUs is continuous attempting RLC-SDU to RLC-PDU alignment would need to introduce unused padding bits. This would increase the link layer overhead and thereby decrease the effective payload rate. Instead, the start of the next RLC- SDU shall be packed here.

RLC-SDUs where one or more RLC-PDUs that contain part of the RLC-SDU have not been received correctly (as checked by the CRC) shall be discarded. It becomes important to select appropriate-sized RLC-SDUs (IP-packets) that match the channel conditions.

The RLC/RLP layer can perform re-transmissions. The re-transmission scheme may be set up with different levels of persistency, such that the maximum number of re-transmission attempts (NAK rounds) is specified for a connection, which puts an upper limit to the variable RLC/RLP delay that re-transmission can cause. The RLC/RLP layer maintains in-order delivery, so usage of re-transmission can introduce high delay jitter.

It is likely that conversational bearers are realized without RLC re-transmissions. However, for streaming application the transfer delay requirements for streaming are typically in a range where at least in a part of this range RLC/RLP re-transmission should be used. It is assumed that the application's requirement on delay variation is expressed through the transfer delay attribute, which implies that there is no need for an explicit delay variation attribute.

In order to be able to re-assemble the SDUs at the receiving side, the SDU boundaries must be signaled. In UMTS this is done by means of a length indicator in the RLC-PDU header. An RLC-PDU always starts with a sequence number (minimum size is 7 bits) and is followed by a 1-bit indicator telling if the next octet is a length indicator or payload data. In CDMA-2000 the application packets are framed at PPP layer using the PPP protocol.

If applicable the RTP/UDP/IP header is compressed. For the tests we assume that the robust header compression scheme provides complete robustness, so that each compressed packet header can be decoded independent of which packets are lost. Depending on the packet loss rate, though, headers are compressed with different efficiency. The scheme also requires an initialization period, where all headers are sent uncompressed, this time is approximately the round-trip time of the connection.

Packetization overhead in UMTS protocol stack:

- Overhead per RLC PDU = RLC frame header (4 bytes) = 4 bytes

- Overhead per PDCP PDU = PDCP packet header (1 byte) + length info (1 byte) = 2 bytes

Packetization overhead in CDMA-2000 protocol stack:

- Overhead per RLC PDU =  RLC frame header (2 bytes) = 2 bytes

- Overhead per PPP PDU = PPP packet header (3 bytes) + bit stuffing (~2% of payload)+compressed RTP/UDP/IP header (3 bytes)

# Simulation Software

Simulation software will be available on the standard ftp-site.

Simulator requirements:

- Off-line simulation (file-to-file)

- In-order packet delivery

- No bit-errors or corrupted packets delivered to the application (RLC transparent mode is not simulated)

- No unequal error protection

- Simulate delay jitter in case of RLP/RLC re-transmission simulation, no delay jitter otherwise

Tasks performed by simulator:

Step 1: Framing at the PDCP/PPP level, add overhead of RTP/UDP/IP compressed header.

Step 2: Schedule and split application packets to RLC/RLP frames. Packets are not necessarily aligned with RLC/RLP frame boundaries. Add RLP/RLC frame header (2 or 4 bytes).

Step 3: Map bit-error pattern to RLC/RLP frames. Discard RLC/RLP frame if it contained bit-error

Step 4: Optional RLC/RLP re-transmission (in case of video streaming)

Step 5: If an RLC/RLP frame is lost the application packets that were covered by it (even if only partially) are all discarded

I/O File Format:

The simulator's input and output are RTP packets. The input/output file is a simple concatenation of packets, where the following "fields" are included for each packet:

Int32	size of the following packet in bytes

Int32	for input file: time when packet sent (output by the NAL)

for output file: time when packet arrived (at the simulator output)

in milliseconds with reference to the time for the first RTP packet in the file (which is set to 0).

[]byte	packet content, starting with the RTP header.

The simulator is reporting the number of bits that would have been sent in a real scenario, this number should be used for calculating the bitrate.

The payload of the packets is transparent to the simulator, only the RTP header is parsed and compressed. As a simplified model, the RTP header compression is simulated by simply reducing the number of bytes as packet header overhead to 3 bytes for each packet independent of its location in the stream, size or the packet loss rate. Thus, the compression and decompression is not actually performed, rather the RTP header content is simply presented in the output file as it was received in the input file, whenever the RTP packet is not lost.

Two modes will be supported. In the conversational modes no RLC retransmissions will be supported. For streaming services limited RLC retransmission is provided. The re-transmission is not full-persistent (like in RLC Ack-mode of UTRAN/GERAN in UMTS) but the  maximum allowed simulator delay  influences the persistency (number of trials) of the re-transmission scheme. This corresponds with the CDMA-2000 model, but it is assumed, that at the concept level also UMTS future releases will provide similar functionality.

# Input bitstream requirements

In order to avoid buffer overflow the video payload bitrate has to be smaller than or equal to the available payload bitrate of the radio bearer simulated. In the simulator continuous packet streaming is assumed, such that there is always application packet content available for transmission and there is no silence period between packets. It is up to the experimenter to ensure, that the packetized video stream with the added simulator packetization overhead and optional RLC retransmission overhead fits into the available radio bearer. For best resource utilization, the available radio bearer bitrate should be matched as closely as possible. In practice this means, that the total transmission time required to transmit the input packet stream has to be smaller than the session time (length of the sequence), and should be as close to the session time as possible. In order to compare the error robustness performance fairly, it is intended to eliminate the effect of using different rate control schemes by different proponents. Thus the simple rate control to be used in the different application simulations is defined below. When more sophisticated rate control scheme implementation becomes available in the TML software, usage of it can be considered.

### Video streaming applications:

- Only the average video rate has to match the available bitrate, and it is assumed, that the receiver buffer is large enough to smooth out any "playout time - time of arrival" variations.

- The required fixed frame rate is to be used, and video rate is allowed to be controlled only through changing the fixed QP that is used for all the frames of the sequence.

- It is allowed to use multi-pass off-line encoding, as streaming of pre-encoded stream can be assumed.

### Video conversational applications

- No receiver buffering is assumed, so the "playout time - time of arrival" difference has to be minimized. Rate can be controlled by frame skipping, but the same QP is to be used for all frames of the sequence. The average frame rate is to match the required frame rate. The number of frames skipped should be chosen, such that: abs(playout time - time of arrival) < 1/(input frame rate)

- It is assumed, that a sequence is encoded and sent real-time so the characteristics are not known a-priori. Thus, coding efficiency and error resilience tools and their parameters are not allowed to be tuned to a given test sequence characteristics.

The required video sequences and bitrates to be used in experiments are defined in Table 1.

| Sequence | Picture Size | Frame-rate (f/s) | To be used with physical layer rate (kbits/sec) |
| --- | --- | --- | --- |
| Hall Monitor | QCIF | 15 | 64 |
| Coastguard | QCIF | 10 | 64 |
| Foreman* | QCIF | 7.5 | 64 |
| Glasgow* | QCIF | 7.5 | 64 |
| Paris | CIF | 15 | 128 |

Table 1: Sequences to be used

*: These sequences originate from a 25 f/s source, but for simplicity it can be assumed, that the original rate was 30 f/s, so the target frame-rates can be achieved by using constant frame skip.

In 3G networks theoretically any (<2048 kbit/s) constant bitrate can be guaranteed by the network. In case the video stream bitrate is always equal or lower than the guaranteed bitrate, the application does not have to perform rate adaptation or congestion control.

For a given maximum application SDU size an SDU loss ratio can be guaranteed. This means, that the error robustness of a video packet stream can and should be tuned to a given packet loss ratio. It also means in practice, that the video packet size influences the packet loss rate.

The input sequence is to be encoded once (e.g. for Hall Monitor 300 original frames) and the encoded bitstream is run through the simulator 10 times with different starting position of the bit-error pattern, that are specified for each bit-error file.

# Error Patterns and RLP/RLC frame sizes

In order to simulate the guaranteed QoS concept, it is assumed, that channel conditions don't vary for the duration of the transmission, so that the provided network QoS (bitrate, packet loss rate) is nearly constant throughout the session. With the constant channel conditions the simulator can assume using a dedicated radio channel for the video stream with a given maximum channel bit-rate. Audio streams or other parallel flows of an application are assumed to have set up over separate radio bearers, where the possibly different QoS attributes required can be guaranteed. Thus the different flows are separated logically.

For simulating radio channel conditions bit-error patterns are used, that were captured in different real or emulated mobile radio channels. The bit-error patterns are captured above the physical layer and below the RLC/RLP layer, such that in practice they act as the physical layer simulation. It is assumed, that the statistical parameters (e.g. average residual BER and burstiness) of the bit-error pattern are similar for the whole transmission. The available bit-error patterns determine the bitrates and SDU error ratios that can be simulated.

Table 2 lists the bit-error patterns to be used in the experiments. These bit-error files are attached to the current document. Pattern 1. and 2. is mostly suited to be used in video streaming applications, where RLP/RLC layer re-transmissions can correct many of the frame losses. Patterns 3. to 6. are meant to simulate a more reliable, lower error-rate bearer, that is required in video conversational applications. The physical layer simulation used 16 bit Layer 1 CRC, but the bits corresponding to the CRC are not present in the error pattern files.

| No. | File name | Bitrate | Length | BER | RLC PDU size | Mobile Speed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 18681.3 | 64 kbps | 60 s | 9.3e-3 | 640 bits | 3 km/h |
| 2 | 18681.4 | 64 kbps | 60 s | 2.9e-3 | 640 bits | 3 km/h |
| 3 | Wcdma_64kb_3kph_5e-04.bin | 64 kbps | 180 s | 5.1e-4 | 640 bits | 3 km/h |
| 4 | Wcdma_64kb_50kph_2e-04.bin | 64 kbps | 180 s | 1.7e-4 | 640 bits | 50km/h |
| 5 | Wcdma_128kb_3kph_5e-04.bin | 128 kbps | 180 s | 5.0e-4 | 640 bits | 3 km/h |
| 6 | Wcdma_128kb_50kph_2e-04.bin | 128 kbps | 180 s | 2.0e-4 | 640 bits | 50km/h |

Table 2: Bit error patterns

### The starting positions for the 10 different runs of simulation are specified for each bit-error file in Table 3. The starting positions were determined randomly. The bit-error patterns are to be wrapped around (start reading from the beginning) when the end of file is reached. Fixing the starting positions for all experimenters gives directly comparable results for each run of the simulation.

| No. | Simulation starting positions (in bytes) |
| --- | --- |
| 1 | 227200, 259840, 128000, 348800, 81920, 79360, 392320, 56960, 440960, 458880 |
| 2 | 227200, 259840, 128000, 348800, 81920, 79360, 392320, 56960, 440960, 458880 |
| 3 | 551040, 251840, 613120, 609920, 616000, 57600, 1313120, 1248640, 273920, 1366400 |
| 4 | 551040, 251840, 613120, 609920, 616000, 57600, 1313120, 1248640, 273920, 1366400 |
| 5 | 1143200, 410240, 2480000, 561920, 2135840, 1954560, 283520, 730880, 698720, 154368 |
| 6 | 1143200, 410240, 2480000, 561920, 2135840, 1954560, 283520, 730880, 698720, 154368 |

Table 3: Simulation starting positions for the different bit-error patterns

The simulator overhead can be separated into payload independent and payload dependent parts:

- The RLP/RLC overhead includes RLC/RLP headers and optional L2 re-transmission overhead, and it is independent of the video stream used so it can be calculated for each bit-error pattern and starting position in advance. For calculating the L2 re-transmission overhead a round-trip delay has to be assumed by the algorithm. A table including these overhead values will be provided as soon as the simulator software is available.

- The compressed header + PDCP/PPP header overhead depends on the video packet sizes used. It has to be calculated for each video stream separately, so it is up to the experimenter.

# Anchors

The anchor results represent the comparison point for new proposals. The combination of application environment (streaming or conversational), sequences , bit-error patterns and simulator parameters for which anchor results are available are shown in Table 4.

| Anchor No. | Sequence | Application (type of input bitstream) | Bit-error pattern | RLP/RLC re-transmission* |
| --- | --- | --- | --- | --- |
| 1 | Hall Monitor | Conversational | 3. | No |
| 2 | Hall Monitor | Conversational | 4. | No |
| 3 | Coastguard | Streaming | 1. | Yes |
| 4 | Coastguard | Streaming | 2. | Yes |
| 5 | Foreman | Conversational | 3. | No |
| 6 | Foreman | Conversational | 4. | No |
| 7 | Foreman | Streaming | 1. | Yes |
| 8 | Foreman | Streaming | 2. | Yes |
| 9 | Glasgow | Streaming | 1. | Yes |
| 10 | Glasgow | Streaming | 2. | Yes |
| 11 | Paris | Conversational | 5. | ### No |
| 12 | Paris | Conversational | 6. | No |

Table 4: Test case combinations used in anchor results

*: The maximum allowed simulator delay is yet to be determined.

It is recommended and required for fair evaluation, that with new proposals results are presented for the same combinations! In streaming applications RLC re-transmissions are assumed to be always used, but for anchors of streaming type, results without RLC re-transmissions can be presented by proposals for comparison, in which case the RLC re-transmission overhead is not to be accounted for when setting the video stream bitrate.

Anchor bit-streams will be provided and their availability will be announced on the itu-adv-video reflector.

Though the inclusion of B pictures will be beneficial for streaming applications, in the anchor sequences only I and P pictures are to be used. With the development of more mature B-picture concept in the test model the anchors can be updated as necessary.

# Quality measurement

Objective quality measurement is done using PSNR. The PSNR is calculated between each and every frame of the source sequence (at frame rate as specified in Table 1), and the corresponding reconstructed frame. The PSNR of every frame is averaged over the sequence. This implies that more than one PSNR value might be calculated for a single reconstructed frame (against different source frames) in a scenario where one or more coded source frames are lost, or frames were skipped during encoding in video conversational applications, where the target frame rate is not matched exactly. In addition to average PSNR values sufficient information about skipped and lost frames if applicable are required.

The PSNR of the decoded video is calculated for each of the 10 runs and the average PSNR plus the best and worst cases of the 10 runs are shown. This method is used to show the variation of the PSNR depending on the position of the losses.

Additionally, subjective quality assertion by Q.6 experts, based on provided D1-tapes or other appropriate media, forms an important part of the quality assessment process for the evaluation of new techniques.

This document was generated as a merge of documents VCEG-M53 and VCEG-M55 presented at the Austin meeting on April 2nd, 2001 and some further comments and work during the meeting.