| ITU-T Video Coding Experts Group (ITU-T SG16 Q.6) 20th Meeting: 2-5 September 2003, San Diego | Document: VCEG-T03 Filename: VCEG-T03.doc |
| --- | --- |

| Title: | Some Potential Enhancements of H.264/AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gary Sullivan One Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garys@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract/Summary

This document contains a list of some technical features that could potentially enhance the capabilities of H.264/AVC.  This is not a proposal to launch a new project for extending the capabilities of the standard with these technical features.  It is only an effort to organize into one list some of the techniques that have previously been identified – in order to help capture some knowledge, organize future investigation of how much improvement could be obtained by various methods, and help determine what applications may benefit from such improvements.  Nothing presented here is new – this is only a non-exhaustive list of some techniques that previously appeared to have some promise in prior VCEG, JVT, or MPEG contributions.

Each technique discussed here is accompanied by a description of the type of enhancement it could provide, an estimate of the degree of enhancement it could provide, and an estimate of the complexity impact of the enhancement.  The estimates should be considered only to be half-educated guesses, not well-studied conclusions.

Since this document was written in a hurry, it does not do a good job of documenting the history of the various techniques discussed in it and referencing the appropriate contributions.

Coding efficiency enhancements are listed first (with prediction enhancement techniques listed first within that category and then residual coding enhancements later) and then functionality enhancements are grouped together at the end.

# 1.	Sub-macroblock intra/inter switching

In the H.264/AVC design adopted in February 2002, the partitioning scheme initially adopted from Wiegand et al included support of switching between intra and inter on a sub-macroblock (8x8 luma with 4x4 chroma) basis.  This capability was later removed at the Fairfax or Klagenfurt meeting in order to reduce decoding complexity.

Enhancement type: Coding efficiency improvement

Estimated degree of enhancement: 1-2% avg. (probably more perceptually)

Complexity impact: Moderate increase

# 2.	Overlapped block motion compensation

OBMC is a well-known technique for improving the fidelity of motion compensated video coding (see, for example, H.263 Annex F).  It has not yet been investigated in the H.264/AVC context.  Not only can it provide a significant benefit in terms of reduced quantity of prediction error in PSNR terms, but it can also essentially eliminate the blocking artifacts caused by motion compensated prediction.

It is important to note that incorporating OBMC within the motion compensation performed by a decoder can ordinarily provide a significant benefit, even if the motion estimation search process performed in the encoder does not use optimization for OBMC.

If OBMC is also incorporated within the optimization process of the motion estimation performed by an encoder, a further substantial improvement can be obtained.

Enhancement type: Coding efficiency improvement

Estimated degree of enhancement: 8% avg. (probably more perceptually)

Complexity impact: Significant increase

# 3.	Intra frame "motion compensation"

In contribution JVT-C151, Siu-Leong Yu and Christos Chrysafis of Divio proposed the use of block "motion compensation" prediction within an intra picture as a method of enhancing coding efficiency.  Although the benefit obtained by this technique was highly scene dependent (it seemed to work really well only on Foreman, and somewhat worked on Akiyo), some benefit seemed to be shown.

Enhancement type: Coding efficiency improvement

Estimated degree of enhancement: 1-2% avg. (highly scene dependent)

Complexity impact: Small/moderate increase

# 4.	Direct-P motion compensation

Motion compensation with "direct" inference of motion vectors for inter pictures showed some promise in contribution JVT-C128.

Enhancement type: Coding efficiency improvement

Approximate degree of enhancement: 1-2% avg.

Complexity impact: Small increase

# 5.	Increased-accuracy motion compensation

1/8-sample motion compensation was in drafts of H.264/AVC until it was dropped sometime in 2002 out of concerns over complexity.  For example, see prior work by Wedi.  It seemed to work particularly well on the Mobile & Calendar sequence.

Enhancement type: Coding efficiency improvement

Approximate degree of enhancement: 3% avg. (highly scene dependent)

Complexity impact: Significant increase

# 6.	Adaptive motion compensation interpolation tap values

The idea of having some form of adaptivity for the interpolation filter tap values for motion compensation was investigated in the JVT but was eventually no longer pursued due to concern over schedule, degree of benefit, complexity, etc.

Enhancement type: Coding efficiency improvement

Approximate degree of enhancement: 1-2% avg. (highly scene dependent)

Complexity impact: Small/moderate increase

# 7.	Adaptive motion vector precision

Prior work has shown that the accuracy needed for motion compensation is content-dependent.  Variable-precision motion vectors (per prior contributions by Ribas) could enable the use of precise motion vectors when advantageous while avoiding a waste of bits when more precision does not help.

Enhancement type: Coding efficiency improvement

Approximate degree of enhancement: 1-2% avg. (highly scene dependent)

Complexity impact: Small/moderate increase

# 8.	Multiple reference picture warping

Prior work by Wiegand described the possibility of combining reference picture warping with multiple reference picture motion compensation for coding efficiency enhancement.

Enhancement type: Coding efficiency improvement

Approximate degree of enhancement: 5% avg.

Complexity impact: Significant increase

# 9.	Variable block-size transform

The variable-block-size transform feature was a significant area of investigation in the development of H.264/AVC (see prior contributions by Wien), and remains of interest in the community as evidenced by contribution JVT-I022 from Steve Gordon of Sand Video.  It was dropped from the draft of H.264/AVC due to dual considerations of not being confident of an ability to specify it properly within the timeframe of the H.264/AVC specification development, and not being confident of the amount of benefit that could be gained by the technique.

Enhancement type: Coding efficiency improvement

Approximate degree of enhancement: 5-7% avg. on high-res video only

Complexity impact: Small/moderate increase

# 10.	Transform-bypass lossless coding

The idea of bypassing the transform and directly coding the residual signal using lossless coding has come up in several JVT contributions (May 2002 contribution JVT-C023 from Shijun Sun of Sharp Labs, later verified in July 2002 contribution JVT-D054 by Mathias Wien, Sept. 2003 contribution JVT-I012 from Woo-Shik Kim, Dae-Sung Cho, and Hyun Mun Kim of Samsung AIT, and Sept. 2003 contribution JVT-I024 from Shijun Sun of Sharp Labs).  This seems to be a well-performing method of lossless coding, and could be applied either to intra or inter coding.

Enhancement type: Efficient lossless coding for intra or inter

Approximate degree of enhancement: Added feature for additional application area

Complexity impact: Not complex, but difficult to quantify due to profiling issues

# 11.	Scalability features

Spatial and SNR scalability (either "fine-grained" or "coarse-grained") could be added to the codec design in a variety of ways (some straightforward, some not).

Enhancement type: Scalability

Approximate degree of enhancement: Added feature for additional application area

Complexity impact: Varies depending on scalability method chosen