| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 29th Meeting: Bangkok, 16-17 January, 2006 | Document VCEG-AB07 Filename:VCEG-AB07 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Tomoyuki Yamamoto, Maki Takahashi Sharp Corporation 1-9-2 Nakase, Mihama-ku Chiba-shi, Chiba 261-8520, Japan | Tel: Fax: Email: | +(81)43-299-8475 +(81)43-299-8609 yamamoto.tomoyuki@sharp.co.jp |
| Title: | Alternative colPic for temporal direct prediction |  |  |
| Purpose: | Information |  |  |

# Introduction

Direct Prediction, which has been incorporated into H.264, improves coding efficiency by omitting difference between a motion vector and its predictor. In the derivation process of a motion vector in  temporal direct mode, the motion vector shall be regarded as zero vector when the co-located macroblock is intra. However, such zero vector doesn’t represent the actual motion in many cases.

In this contribution, we introduce a method to derive a reasonable motion vector in temporal direct mode even when the default co-located macroblock is intra. We use another co-located macroblock which is on the reference picture referred by RefPicList0[0] instead of the one referred by RefPicList1[0]. The explanation of the algorithm is followed by some preliminary experimental results.

# Temporal Direct Prediction with Alternative ColPic

We will start by overviewing the motion vector derivation process in temporal direct mode, which is specified in the current H.264 recommendation[1]. Firstly, the reference picture referred by RefPicList1[0] is selected as the picture colPic. Then, List0 motion vector of the co-located macroblock on colPic is selected as mvCol. Finally, you can derive motion vector mvL0 and mvL1 by scaling mvCol based on the Picture Order Count of each picture. Fig.1 depicts the relation of mvCol and mvL0/mvL1. As a singular case, mvCol is forced to be zero vector when the co-located macroblock is intra. Therefore, both mvL0 and mvL1 become zero vectors in that case (see Fig.2).

colPic

currPic

tb

td

time

colPic

currPic

tb

td

time

mvL1

mvL0

mvCol

mvL1

mvL0

mvCol

Fig1. relation between mvCol and mvL0/mvL1 in temporal direct mode

Fig2. relation between mvCol and mvL0/mvL1 when co-located macroblock is intra

Intra

A problem occurs because mvL0 and mvL1 are derived without utilizing any temporal correlation when co-located macroblock is intra. Temporal direct mode is no longer efficient and that causes loss of coding efficiency. Additionally, in some applications, it is likely that one adopts a GOP structure with periodical I frames. In the specific B pictures, which refer to I picture as the most important List1 reference, it is more difficult to choose direct mode than in other B pictures. And those specific B pictures causes degradation of subjective quality as well as loss of coding efficiency.

There are some possible ways to solve the problem: to use spatial direct mode, to use neighboring blocks of co-located macroblock as substitution, and to use another reference picture as colPic. We have examined the last approach. We used the reference picture referred by RefPicList0[0] as an only candidate to alternate default colPic (RefPicList1[0]). When the co-located macroblock on the default colPic is intra, we use the co-located macroblock on the alternative colPic. And only when those two co-located macroblocks are intra, mvL0 and mvL1 are forced to be zero vectors.

Fig3 illustrates the relations between mvCol and mvL0/mvL1 when the alternative colPic is selected. We use the reference picture referred by RefPicList1[0] as the backward reference picture. The motion vectors mvL0 and mvL1 are derived according to the following equations.

mvL0 = tbtbb × mvCol

mvL1 = tb − tdtbb × mvCol

where tb is the difference of Picture Order Count between the alternative colPic and current Picture, and td is the difference between the alternative colPic and backward reference picture, and tbb is the difference  between the alternative colPic and the reference picture that is refered by mvCol. Note that extra information as to Picture Order Count is required to calculate tbb.

colPic

currPic

tb

td

time

mvL0

mvCol

tbb

mvL1

Intra

Fig3 : relation between mvCol and mvL0/mvL1 in

temporal direct mode with alternative colPic

# Experimental results

We have implemented the method introduced above into H.264 reference software (JM10.1), and performed an experiment. The experiment basically follows VCEG test conditions specified in the document VCEG-AA10 [2]. There are some special configurations for the experiment.

- GOP structure is IBBPBBP…  ( two B frames between I/P frames )

- Intra interval is 15 frames  ( four P frames between I frames )

Test sequences are mobile, tempete, and foreman (CIF).

Fig4 shows Rate-Distortion curve of each test sequence. QPY for I frame is 22,27,32,37, and 42. QPY for P/B frame is larger by one/two than that of I frames respectively. We refer to original temporal direct mode as TD (Temporal Direct mode), and newly implemented one as TDA (TD with Alternative colPic) for simplicity. As you can see in Fig4, TDA and TD show almost the same performance. In mobile sequence, there exists slight improvement at low bitrate.

Fig4 : R-D curve (CIF)

Fig5, which is to evaluate TDA performance on B frames, shows the Rate-Distortion curve as to B frames in the sequences. QPY for B frames is 29, 34, 39, and 44. You can see that TDA outperforms TD especially at very low bitrate.

Fig6 indicates the amount of bits needed to encode each B frame. In the right graph of Fig6, you can see that some frames consume more bits than other frames. Those are B frames whose display timing are between that of last P frame in GOP and that of I frame. And the amount of bits required to encode those frames are significantly reduced with TDA as is shown in the left graph of Fig6.

Fig5: R-D curve as to B frames (CIF)

Fig6: amount of bits used in each B frame (mobile) 
Left: TDA, Right: TD

From the point of view of the objective quality, TDA has an advantage. With TD, the accuracy of mvL0/mvL1 derived in temporal direct mode varies by the B frame’s location in GOP. Therefore, the accuracy of luma/chroma predictors of B_Skip or B_Direct blocks differes from frame to frame. And in addition to that, the ratio of each macroblock type in frame differes as well. Those two causes degradation of the objective quality. Such degradation is quite noticeable at lower bitrate because many blocks are coded as B_Skip or B_Direct block and transform coefficients are quantized with large step size.

# Conclusion

We introduced the new method to derive motion vector in temporal direct mode. We set the reference picture referred by RefPicList0[0] as the alternative colPic when the default co-located macroblock is intra. The impact on the coding efficiency is relatively small, but we could confirm that it can improve coding efficiency of B frames and the objective quality of decoded sequences especially at lower bitrate.

# Patent statement

Sharp may have IPR connected to the proposed method and based on reciprocity Sharp is prepared to grant a license on reasonable and non discriminatory terms.

# References

[1] “ITU-T Recommendation H.264”, March 2005

[2] T. K. Tan, G. Sullivan, T. Wedi, “Recommended Simulation Common Conditions for Coding Efficiency,” VCEG-AA10, October 2005