| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 42th Meeting: Guangzhou, China, 11-15 Oct, 2010 | Document VCEG-AO11 Filename: VCEG-AO11.doc |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Li Song Shanghai Jiao Tong University, 800 Dongchuan Road, Min Hang District, Shanghai, China, 200240 Zhengyi Luo Shanghai Jiao Tong University, 800 Dongchuan Road, Min Hang District, Shanghai, China, 200240 Xiaokang Yang Shanghai Jiao Tong University 800 Dongchuan Road, Min Hang District, Shanghai, China, 200240 | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: | +86 (21) 34205492 +86 (21) 34204155 song_li@sjtu.edu.cn +86 (21) 34205492 +86 (21) 34204155 llzzyynjupt@sjtu.edu.cn +86 (21) 34205492 +86 (10) 34204155 xkyang@sjtu.edu.cn |
| Title: | On reference sample filtering and transform-bypass lossless intra coding |  |  |
| Purpose: | Proposal |  |  |

_____________________________

ABSTRACT

We propose to remove the reference sample pre-filtering of the Intra_8x8 prediction for H.264/AVC lossless coding. Experimental results show that reduced bitrate can be achieved with lower computational costs. Besides, the corresponding statement changes of this scheme are also proposed for the H.264/AVC standard.

# Problem Statement

Recently, as demands for high quality images increase rapidly, lossless coding has been introduced to the H.264/AVC standard [1]. H.264/AVC lossless coding inherits many techniques from the lossy coding of the standard and good compression efficiency has been reported.

For the luma component H.264/AVC lossless coding still adopts 3 intra prediction modes – Intra_4x4, Intra_8x8 and Intra_16x16 like the lossy coding. As far as the Intra_8x8 mode is concerned, an 8x8 block may be predicted based on the neighbouring 25 samples as shown in Figure 1. There are 9 prediction mode directions in the Intra_8x8 mode. Except the DC prediction (mode direction 2), the other 8 mode directions are illustrated in Figure 2, where mode direction 0 and 1 denote respectively the vertical and horizontal prediction with SbS DPCM [2] applied in the case of lossless coding.

Figure 1.  Illustration of the Intra_8x8 prediction.

Figure 2.  Intra_8x8 prediction mode directions.

According to the H.264/AVC standard, in the Intra_8x8 mode neighbouring samples shall be filtered before used for prediction [1]. But for H.264/AVC lossless coding the filtering process may reduce the correlation between the neighbouring samples and the samples to be predicted and lead to low prediction efficiency. So we propose to remove the reference sample pre-filtering of the Intra_8x8 mode in the case of H.264/AVC lossless coding. And the experiments validating the effectiveness of our scheme are shown in the next section.

# Experiments

After being implemented in the JM 17.2 software [3], the proposal is evaluated with representative 4:2:0 sequences. In the experiment, the first 100 frames of every sequence are lossless intra encoded with CABAC applied and only the Intra_8x8 mode is used for the luma component.

Here we compare the performance of 3 methods. Method 0 follows the latest H.264/AVC standard [1] and denotes all the 9 mode directions of the Intra_8x8 mode use filtered reference samples; method 1 denotes mode direction 0 and 1 use unfiltered reference samples, while the other 7 mode directions use filtered ones; method 2 follows the proposed scheme and denotes all the 9 mode directions use unfiltered reference samples. Then the experimental results for the first 100 frames of every sequence are shown in Table 1, where size reduction of method 1 and method 2 is calculated respectively as follows:

Table 1.  Experimental results for the first 100 frames of every sequence.

| Format | Sequence | Resultant File Size (bytes) |  |  | Size Reduction (%) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Method 0 | Method 1 | Method 2 | Method 1 | Method 2 |
| 176x144 | Bus | 2,977,515 | 2,934,532 | 2,934,700 | 1.4436 | 1.4379 |
|  | Container | 2,043,627 | 2,008,416 | 2,007,600 | 1.7230 | 1.7629 |
|  | Football | 2,452,557 | 2,433,708 | 2,433,241 | 0.7685 | 0.7876 |
|  | Foreman | 2,223,736 | 2,213,921 | 2,191,000 | 0.4414 | 1.4721 |
|  | Mobile | 3,891,290 | 3,870,065 | 3,869,358 | 0.5454 | 0.5636 |
|  | Mother and Daughter | 1,961,717 | 1,949,646 | 1,947,629 | 0.6153 | 0.7181 |
|  | News | 1,981,960 | 1,951,147 | 1,949,344 | 1.5547 | 1.6456 |
|  | Paris | 3,038,618 | 2,980,419 | 2,979,703 | 1.9153 | 1.9389 |
|  | Tempete | 3,389,645 | 3,359,350 | 3,359,667 | 0.8938 | 0.8844 |
| 352x288 | Bus | 10,699,260 | 10,552,584 | 10,551,851 | 1.3709 | 1.3777 |
|  | Football | 7,805,827 | 7,752,872 | 7,747,473 | 0.6784 | 0.7476 |
|  | Foreman | 7,034,812 | 7,002,453 | 6,956,883 | 0.4600 | 1.1078 |
|  | Mobile | 13,974,705 | 13,885,636 | 13,874,032 | 0.6374 | 0.7204 |
|  | Mother and Daughter | 6,741,063 | 6,693,873 | 6,685,012 | 0.7000 | 0.8315 |
|  | News | 6,677,217 | 6,593,277 | 6,589,296 | 1.2571 | 1.3167 |
|  | Paris | 9,529,289 | 9,284,917 | 9,281,930 | 2.5644 | 2.5958 |
|  | Tempete | 11,328,251 | 11,256,642 | 11,256,685 | 0.6321 | 0.6317 |
| 704x576 | City | 33,058,766 | 32,578,718 | 32,571,705 | 1.4521 | 1.4733 |
|  | Crew | 24,193,048 | 24,076,022 | 24,076,115 | 0.4837 | 0.4833 |
|  | Harbour | 32,138,856 | 31,567,414 | 31,564,929 | 1.7780 | 1.7858 |
|  | Soccer | 26,313,545 | 26,061,112 | 26,055,188 | 0.9593 | 0.9818 |
| 1280x720 | BigShips | 60,862,456 | 60,277,475 | 60,255,863 | 0.9612 | 0.9967 |
|  | City | 66,672,652 | 66,018,583 | 66,001,885 | 0.9810 | 1.0061 |
|  | Crew | 47,214,838 | 47,038,100 | 47,038,523 | 0.3743 | 0.3734 |
|  | Harbour | 67,047,973 | 66,578,524 | 66,572,729 | 0.7002 | 0.7088 |
|  | Raven | 48,054,323 | 48,073,188 | 48,063,309 | -0.0393 | -0.0187 |
|  | Sheriff | 47,576,036 | 47,271,219 | 47,263,692 | 0.6407 | 0.6565 |
|  | ShuttleStart | 34,153,519 | 33,984,716 | 33,983,869 | 0.4942 | 0.4967 |
|  | SpinCalendar | 79,808,428 | 79,295,782 | 78,490,797 | 0.6423 | 1.6510 |

From the experimental results we can see that both method 1 and method 2 achieve bit saving for most sequences (except for 1280x720 Raven), and in most cases method 2 saves more bits than method 1 (except for 176x144 Bus, 176x144 Tempete, 352x288 Tempete, 704x576 Crew and 1280x720 Crew). Furthermore, since method 2 removes the filtering operations for all the 9 mode directions and hence no reference sample pre-filtering is needed, method 2 requires the lowest computation costs as well. In a word, our scheme, i.e. method 2, can achieve higher coding efficiency for H.264/AVC lossless coding.

# Proposed Statement Changes

Option 1

In subclause 8.3.2.2 “Intra_8x8 sample prediction”, there is the statement:

“

The reference sample filtering process for Intra_8x8 sample prediction in subclause 8.3.2.2.1 is invoked with the samples p[ x, y ] with x = −1, y = −1..7 and x = 0..15, y = −1 (if available) as input and p′[ x, y ] with x = −1, y = −1..7 and x = 0..15, y = −1 as output.

”

Based on the effectiveness of our scheme, we propose to remove the reference sample pre-filtering of the Intra_8x8 prediction for H.264/AVC lossless coding and modify the statement as follows:

“

If TransformBypassModeFlag is equal to 0, the reference sample filtering process for Intra_8x8 sample prediction in subclause 8.3.2.2.1 is invoked with the samples p[ x, y ] with x = −1, y = −1..7 and x = 0..15, y = −1 (if available) as input and p′[ x, y ] with x = −1, y = −1..7 and x = 0..15, y = −1 as output.

else p′[ x, y ]= p[ x, y ], with x = −1, y = −1..7 and x = 0..15, y = −1 (if available).

”

Option 2

Or in a more complex manner, whether the reference samples shall be filtered may be decided at the macroblock level.

Specifically, for any p[x, y] with x=-1, y=-1..7 and x=0..15, y=-1 (if available)

if TransformBypassModeFlag of the macroblock that p[x, y] belongs to is 0

p[x, y] is used for prediction after filtering,

else

p[x, y] is used for prediction directly.

# Conclusion

In this proposal, we propose to remove the reference sample pre-filtering of the Intra_8x8 prediction for H.264/AVC lossless coding. And experimental results show that this scheme can achieve reduced bitrate with lower computational costs. So we recommend adopting the scheme and the corresponding statement changes.

# References

- ISO/IEC and ITU-T, “Advanced video coding for generic audiovisual services”, March 2010.

- Y.-L. Lee, K.-H. Han and G. J. Sullivan, “Improved lossless intra coding for H.264/MPEG-4 AVC”, IEEE Transactions on Image Processing, Sept. 2006.

- JM 17.2 Reference Software, Available at: http://iphome.hhi.de/suehring/tml/download/, July 2010.

# IPR Statement

Shanghai Jiao Tong University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form