ITU-TS SG15	LBC-96-155

Working Party 15/1	April 1996

Expert’s Group For Very Low Bitrate Visual Telephony

Dallas (23-26)   April 1996

Title:	MPEG-4 Video Verification Model flag settings for H.263L

Authors:	John Muller

The ITU-LBC will use the MPEG-4 Video Verification Model as its starting point for core experiments. Because the MPEG-4 VM is extremely flexible, the LBC has decided on a common set of settings for the flags in the Session and VOP layers. Core experiment proposers in ITU-LBC are expected to use these settings. Note that the flags listed below are only a subset of the flags in the Session and VOP layers. All flags not listed here should be used and set according to the specifications in the MPEG-4 Video VM description.

| Session Layer Flag | Setting |
| --- | --- |
| more_than_one_VOP | “0” |
| VOP_quant_type_sel | “0” |
| intra_dcpred_disable | “0” |

| VOP Layer Flag | Setting |
| --- | --- |
| VOP_ID | “0” |
| VOP_visibility | “1” |
| VOP_of_arbitrary_shape | “0” |
| VOP_predictoin_type | “00” for I; “01” for P |
| VOP_fcode_for | “10” |
| seperate_motion_texture | “0” |