| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, USA 24-27 June 1997 | Document: Q15-A-63 Document: Q15-B-01 Filename: q15a63.doc Filename: q15b01.doc |
| --- | --- |

| Question: | Q.15/16 |  |
| --- | --- | --- |
| Subject: | Report of the First Meeting (Meeting A) of the ITU-T Q.15/16 Video Coding Experts Group Portland, Oregon USA June 24-27, 1997 (Study Period 1997-2000) |  |
| Source: | Gary Sullivan, Q.15 Rapporteur PictureTel Corporation 100 Minuteman Rd. M/S 635 Andover, MA 01810 USA | Tel: +1 508 623 4324 Fax: +1 508 749 2804 Mail: garys@pictel.com |

_____________________________

CONTENTS

0.0 OVERVIEW

1.0 OPENING SESSION (HELD JOINTLY WITH Q.11/16)

1.1 Previous Meeting Reports [Q15a01]

1.2 Ad Hoc Committee Reports [Q15a02, Q15a03, Q15a04]

1.3 Organizational Items [Q15aTD-1, Q15a25, Q15aTD-0]

1.4 Liaison with Other Standards Organizations

1.4.1 ITU-T Study Group 12 [Q15a16]

1.4.2 ITU-T Study Group 16 Question 13 [Q15a42]

1.5 Review of Contributions [Q15a00]

1.6 Meeting Plan [Q15aTD2]

1.7 Proposed Future Meetings Plan

2.0 H.263+ and H.263++ Activity [Q15a03, Q15a05, Q15a56, Q15a60]

2.1 Introduction

2.2 Actions Taken

2.2.1 Proposals to Change the Drafted H.263+ Content

2.2.1.1 Ad-Hoc Report Review [Q15a03, Q15a05]

2.2.1.2 Custom Source Format [Q15a17]

2.2.1.3 Annex J Deblocking Filter [Q15a12, Q15a22]

2.2.1.4 Annex M Improved PB Frames [Q15a14, Q15a36]

2.2.1.5 Annex N Reference Picture Selection [Q15a28]

2.2.1.6 Annex O Scalability [Q15a21, Q15a24, Q15a36, Q15a47]

2.2.1.7 Annex P Reference Picture Resampling [Q15a34]

2.2.1.8 Annex Q Reduced-Resolution Update [Q15a31]

2.2.1.9 Annex T Modified Quantization [Q15a40]

2.2.1.10 4MV with DQUANT [Q15a23]

2.2.1.11 Motion Compensation Error Accumulation [Q15a35]

2.2.1.12 Continuous-Presence Multipoint [Q15a46]

2.2.1.13 Start-Code Emulation [Q15a54]

2.2.2 Review and Verification of Drafted Content [Q15a05]

2.2.2.1 Bitstream Exchanges [Q15a39]

2.2.2.2 Annex I Advanced Intra Coding [Q15a48]

2.2.2.3 Annex P Reference Picture Resampling [Q15a34, Q15a43]

2.2.2.4 Annex Q Reduced Resolution Update [Q15a32]

2.2.3 TMN7 Enhancement Efforts [Q15a04]

2.2.3.1 Post-processing [Q15a13, Q15a50]

2.2.3.2 Rate Control [Q15a20]

2.2.3.3 Motion Estimation [Q15a45]

2.2.3.4 Software availability [Q15a37, Q15a43, Q15a45]

2.2.4 Coordination with Q.11/16 through Q.14/16 [Q15a01, Q15a29, Q15a30, Q15a38, Q15a42, Q15a53]

2.2.5 Preferred mode combinations for H.263/H.263+ [Q15a26, Q15a41, Q15a42, Q15a44, Q15a53, Q15a57]

2.2.6 Relationship with ISO/IEC JTC1 SC29/ WG11 (MPEG) [Q15a19, Q15a27]

2.2.7 Proposals for H.263++ extensions

2.2.7.1 Custom Source Formats [Q15a17]

2.2.7.2 Semi-Fixed length motion vectors [Q15a18]

2.2.7.3 4MV with DQUANT [Q15a23]

2.2.7.4 Multi-mode warping prediction [Q15a33]

2.2.7.5 Motion Compensation Error Accumulation [Q15a35]

2.2.7.6 Change to Continuous Presence Multipoint Operation [Q15a46]

2.2.7.7 Adaptive Quantization [Q15a49]

2.3 Conclusions [Q15a57, Q15a59, Q15a60]

3.0 H.263L ACTIVITY [Q15a02, Q15a55]

3.1 Conditions and Requirements for H.263L Proposals [Q15a02, Q15a06, Q15a07, Q15a15, Q15a16, Q15a52, Q15a62]

3.2 Proposals for H.263L Content [Q15a33, Q15a51]

3.3 Relationship with ISC/IEC JTC1 SC29/WG11 (MPEG) [Q15a19, Q15a27]

3.4 H.263L Workplan

3.4.1 General

3.4.2 Time schedule

3.5 Discussion on Requirements for H.263L

3.6 Handling of the methods already proposed

3.7 Relation between H.263++ and H.263L

3.8 Conclusions

4.0 VIDEO CODING FOR SIGN LANGUAGE AND LIP-READING USE [Q15a61]

4.1 Study of User Needs [Q15a08, Q15a09]

4.2 Conclusions and Workplan [Q15a10, Q15a11]

5.0 CLOSING PLENARY (HELD JOINTLY WITH Q11/16)

5.1 Presentation of reports and Review of Results of Breakout Sessions [Q15a55, Q15a56, Q15a61]]

5.2 Liaisons [Q11a68/Q15a58]

5.3 Ad Hoc Committees

5.4 Future Meetings

6.0 SUMMARY OF MEETING RESULTS [Q15a57, Q15a58, Q15a59, Q15a60, Q15a62]

ANNEX A - Ad Hoc Committees

A.1 H.263+ Draft Text Refinement

A.2 H.263+ Recommended Modes and Integration with Terminal Standards (Joint AHC with Q11)

A.3 H.263+ Bitstream Verification

A.4 H.263+ Packetization

A.5 Test Model Enhancement and Software Development

A.6 H.263++ Development

A.7 H.263L Development

A.8 Video Coding for Sign Language and Lip-Reading Use

ANNEX B - Attendee List [Q15aTD-1]

ANNEX C - Document List [Q15a00]

ANNEX E - Liaison to SG 12 [Q11a68/Q15a58]

ANNEX F - Proposed Draft Text for Reduced-Complexity Reference Picture Resampling (Annex P of H.263+)

# 0.0	OVERVIEW

The first meeting (Meeting “A”) of the ITU-T Video Coding Experts Group (Q.15 / SG 16) was held at the Benson Hotel in Portland Oregon USA, June 24-27, 1997 and was chaired by the Q.15 Rapporteur, Mr. Gary Sullivan.  Excellent arrangements were provided by Intel Corporation, the host organization.  A list of the 64 collaborating experts attending the meeting is included in Annex B to this report, and a list of the 63 technical contributions and 3 TD’s which were presented at the meeting is provided in Annex C to this report.  The meeting agenda is provided in Annex D to this report.

The goals of this meeting were to:

- Further the work regarding the editorial status of the “H.263+” revision of Recommendation H.263 as an incremental enhancement of the existing Recommendation, toward decision in January of 1998,

- Further the work toward generation of a new “H.263++” revision of Recommendation H.263 as an incremental enhancement of the existing Recommendation,

- Further the work toward generation of a new “H.263L” video coding Recommendation as a longer-term video coding method,

- Consider action to address the needs of video coding use for lip-reading and sign-language applications, and

- Coordinate our work with that of other relevant organizations

One joint meeting was held with SG16 Q.11 (Circuit-Switched Network systems) to further the work on definition of H.263+ mode selection and support signaling in H.245.  This work will be continued in the interim by an ad hoc set up within Q.15 and again in the September 1997 joint meetings in Bend Oregon USA.

Documents for this meeting, for other meetings, and other data pertinent to the activities of the Video Coding Experts Group can be found on the Q15 ftp site managed by the Rapporteur:

ftp://standard.pictel.com/video-site

Email conversations pertaining to the activities of this group is routinely conducted using the email reflector managed by Mike Zeug of Iterated Systems.  Those wishing to subscribe or unsubscribe to this email reflector are asked to submit their requests to:

itu-adv-video-request@listserv.iterated.com

and the address for email to be sent to all members of the email reflector list is:

itu-adv-video@listserv.iterated.com

With the excellent progress made at this meeting, I am pleased to report that the group believes we are on track to have the new “H.263+” version of Recommendation H.263 ready for decision in January of 1998.

No indication was found of any need for action at this meeting regarding maintenance issues for prior video coding standards (H.120, H.261, H.262) or video-specific needs for non-conversational services.

The meeting was organized into the five areas listed below, the activities for which were conducted without parallel sessions during our meeting, and for which reports were provided which detail the work accomplished in each area.

| SUBJECT | CHAIRPERSON(S) |
| --- | --- |
| 1. Opening Session | Tom Geary Q.11 Gary Sullivan Q.15 |
| 2. H.263+ and H.263++ | Gary Sullivan |
| 3. H.263L | Keiichi Hibi & Mike Zeug |
| 4. Sign Language and Lip-Reading | Gunnar Hellström |
| 5. Closing Session | Tom Geary Q.11 Gary Sullivan Q.15 |

# 1.0	OPENING SESSION (HELD JOINTLY WITH Q.11/16)

The group gave special thanks to the two representative hosts of the Intel Corporation, who in turn outlined the meeting logistics.

The Rapporteur discussed the importance of the disclosure of patents and of the filing of statements pertaining to such intellectual property claims with the ITU.

## 1.1	Previous Meeting Reports [Q15a01]

A meeting report from SG16 Meeting March 1997 Geneva CH was reviewed.

## 1.2	Ad Hoc Committee Reports [Q15a02, Q15a03, Q15a04]

Reports were presented by the Chairpersons of the three Ad Hoc Committees which were appointed at the previous meeting.  The reports which were presented are listed below.

| AD HOC COMMITTEE | CHAIRPERSON | DOCUMENT |
| --- | --- | --- |
| H.263L | Mike Zeug | Q15a02 |
| H.263+ | Gary Sullivan | Q15a03 |
| TMN7 Enhancement | Keiichi Hibi | Q15a04 |

## 1.3	Organizational Items [Q15aTD-1, Q15a25, Q15aTD-0]

The attendee list was circulated and the experts list and meeting invitation were made available.

## 1.4	Liaison with Other Standards Organizations

### 1.4.1	ITU-T Study Group 12 [Q15a16]

The incoming liaison statement from Study Group 12 pertaining to video quality assessment was presented and the group agreed that this should be reviewed in the H.263L sub-session.

### 1.4.2	ITU-T Study Group 16 Question 13 [Q15a42]

The incoming liaison from Study Group 16 Question 13 was reviewed and the group agreed that this should a be reviewed in the H.263+ sub-session.

## 1.5	Review of Contributions [Q15a00]

The list of meeting contributions was reviewed, updated, and approved.  The majority of participants expressed an interest in reducing the use (and even the availability) of paper documents at the meetings of the experts group, in favor of electronic documents instead. However, paper as well as electronic distribution of documents will continue to be requested for the time being.  The vast majority of documents for this meeting had been uploaded to the ftp site for the group several business days prior to the meeting.  This represents a very positive step forward for our work, and advance electronic distribution will continue to be our strongly-preferred practice.  Late, unannounced contributions hand-carried to the meetings were noted to be accepted only with the consensus of the meeting participants.  A few late contributions were made at this meeting as noted in the document list (documents not uploaded prior to a 3-business-day advance distribution deadline are noted by italics in the list), and these were all accepted.

## 1.6	Meeting Plan [Q15aTD2]

The meeting plan outlined in Q15aTD2 was reviewed, modified, and approved.  Room assignments and joint meeting sessions were planned in coordination with Q.11.

## 1.7	Proposed Future Meetings Plan

Experts meeting, September 8-11, 1997, Bend OR USA, hosted by Intel Corp.
	Experts meeting, December 2-5, 1997, TBD, need host for this meeting.

It was agreed to revisit final meeting schedules at the end of this meeting during the closing session.

# 2.0	H.263+ and H.263++ Activity [Q15a03, Q15a05, Q15a56, Q15a60]

## 2.1	Introduction

This section reports the results of progress on H.263+ and H.263++ topics made during the Portland meeting.  Section 2.2 lists the actions of the group and section 2.3 provides conclusions.

## 2.2	Actions Taken

### 2.2.1	Proposals to Change the Drafted H.263+ Content

#### 2.2.1.1	Ad-Hoc Report Review [Q15a03, Q15a05]

The Ad-Hoc activity report [Q15a03] was reviewed, and

- The changes made to produce the 12th draft text [Q15a05] were approved.

- The changes described in items 2, 5, 6, 10, 11, 12, 13, 17, 18, and 19 of “some comments on what is not in the draft” [Q15a05] were adopted.

- Actions were also taken on some of the other items listed in the same section.  Those actions are described in other sections below

- The group agreed with the status information and conclusions of the report [Q15a03]

#### 2.2.1.2	Custom Source Format [Q15a17]

A document [Q15a17] was presented which proposed allowing signaling of an interlaced picture format (without otherwise altering the syntax).  The group decided to wait for more evidence that the adoption of such content was useful and necessary.

#### 2.2.1.3	Annex J Deblocking Filter [Q15a12, Q15a22]

A proposal [Q15a12] was presented for a performance improvement to the drafted loop filter of Annex J.  The performance of the filter enhancement was verified by another party [Q15a22].  Video demonstrations were shown to provide an evaluation of subjective performance.  The group decided to adopt the modified filter.

#### 2.2.1.4	Annex M Improved PB Frames [Q15a14, Q15a36]

A proposal was made to add a backward-only block prediction mode to the annex, for cases such as a scene change or uncovered background just prior to an IPB frame.  The group asked for further justification on this issue.  A need was expressed for clarification regarding the type of inverse quantization to be applied to the B part of the IPB frame.  The group agreed to clarify this to specify that the same type of inverse quantization is always used for the B part of an IPB frame, even if the P part uses the Advanced Intra Coding mode (Annex I).  It was pointed out that there is no real need for “oddification” to be applied to the B part of the IPB frame.  However, the group did not feel a need to change the draft regarding the oddification description.

A proposal was made to add a form of prediction for the value of motion vectors in the B part of Improved PB frames.  The use of a simple predictor was advocated.  Evidence was provided that this could save a large proportion of the number of bits used for encoding motion vector values for some sequences, and the proposal was adopted.

#### 2.2.1.5	Annex N Reference Picture Selection [Q15a28]

Several problems regarding the syntax for Annex N were discussed.  The first four of the proposed changes were adopted.  The resolution of the fifth issue is described elsewhere herein.

#### 2.2.1.6	Annex O Scalability [Q15a21, Q15a24, Q15a36, Q15a47]

A proposal was made to allow a non-zero motion vector for the “skipped” mode of MBTYPE in Annex O.  A significant performance gain was shown, and the proposal was adopted.

The need to send a reference layer number was questioned in regard to B pictures within the same layer as the corresponding reference pictures, and the suggested change was adopted to remove this need.

There was an expression of a need to clarify whether a PB frame or Improved PB frame could appear in the base layer of a bitstream using SNR or spatial scalability.  It was agreed that this should be allowed and that the text should be clarified if necessary to indicate that this is the case.

The need to change the GFID field for B pictures within the same layer as the corresponding reference pictures was questioned.  However, the group was not certain that changing this requirement would be an improvement and thus did not decide to change the text in that regard.

A proposal was made to add a form of prediction for the value of motion vectors in enhancement pictures.  No change was adopted in response to this proposal, and the group was leaning against the notion of adopting such a change in the future.

A proposal was made to allow spatial scalability with upsampling only in a single dimension, rather than performing both horizontal and vertical upsampling at the same time.  The group was favorably disposed to the concept of the proposal, but wished to defer the issue until a demonstration of the usefulness of this technique was provided.

#### 2.2.1.7	Annex P Reference Picture Resampling [Q15a34]

A proposal was made which suggested two modifications of the Reference Picture Resampling mode:

- Adding a half-pixel accuracy version of the picture warping method, and

- Altering the picture warping method to a new “virtual frame” warp.

The two alterations were described as having significantly reduced complexity relative to the current drafted content, and a demonstration was provided to show that performance on the demonstrated scenes was not harmed.  The group decided to add a half-pixel warping version of the resampling, as a separately-negotiable type of mode support.  The group thought that the new virtual frame warping method appeared to also be a good idea, but did not feel that the group had yet had a sufficient opportunity to assess its value.  In the interest of maintaining stability in the drafted technical content, the virtual frame warp was not adopted.  However, the group indicated that it would be likely to adopt the new method in the future, provided that no significant drawbacks were discovered in its design.  A description of the new proposed method was recommended to be incorporated in some manner into the meeting report. This is found in Annex F to this report.  The group also expressed a desire for Annex P of H.263+ to include a better explanation of the simpler special cases of limited support for the mode.  (Also see Section 2.2.2.3.)

#### 2.2.1.8	Annex Q Reduced-Resolution Update [Q15a31]

A document was presented which compared the existing Reduced Resolution Update mode to an alternative known as the MRR mode which had been proposed at the Nice LBC meeting.  Evidence was provided to show superior performance on some scenes for the existing RRU mode.  Therefore the group did not further consider the MRR alternative.  (Also see Section 2.2.2.4.)

#### 2.2.1.9	Annex T Modified Quantization [Q15a40]

Experimental results were shown which indicated that the step size specified for chrominance quantization when using small step sizes and the current drafted Annex T mode could significantly increase bit rate on some sequences without improving perceptual fidelity.  A new proposal was made to mitigate this, which relied on altering the relationship between the luminance and chrominance step sizes so that no large difference between the step sizes existed when the step sizes were small.  The new proposal was adopted.

#### 2.2.1.10	4MV with DQUANT [Q15a23]

A proposal was made to allow a change of quantizer step size to be signaled for macroblocks which use four motion vectors.  It was found to be possible to accomplish this goal without changing the VLC codes for any of the other macroblock prediction types.  The proposal was adopted.

It was also proposed to allow four motion vectors to be allowed whenever PLUSPTYPE is used.  This was not adopted.  Four motion vectors will remain allowed only when Annex F or Annex J is in use.

#### 2.2.1.11	Motion Compensation Error Accumulation [Q15a35]

A phenomenon was demonstrated which showed that reddish chroma artifacts could be produced when the following conditions were true:

- few Intra blocks were sent, and

- the quantization step size was large, and

- a continuously moving object was present in the scene

This was reported to be due to bias in the round-off process during half-pixel motion compensation.  A method was suggested to fix this problem by changing the direction of the bias from picture to picture, using a bit in the picture header to signal the direction of the rounding bias.  It was proposed that this new type of rounding be allowed whenever PLUSPTYPE was present, rather than being a negotiable option.  This proposal was adopted.

#### 2.2.1.12	Continuous-Presence Multipoint [Q15a46]

A proposal was received to allow continuous-presence multipoint operation with arbitrary and possibly differing picture resolutions in the different sub-bitstreams (rather than just QCIF).  This issue was addressed in a joint session with the systems group.  It was pointed out that the continuous presence multipoint feature of H.263 has never been used to the best of our knowledge, and the participants were unsure whether it could work properly.  A need was expressed to examine the H.263+ content to see whether that type of operation was feasible.  In particular, the mode inference rules of the PLUSPTYPE picture header should be checked and changed if necessary to clarify the operation of this feature.  There was some expression of support for the goal of this contribution, but a significant amount of needed work was believed to be necessary.  It was questioned whether the same goal could be achieved using the ISD mode of H.263+.  Although this appeared to be an expression of a desire to multiplex multiple video bitstreams, which would seem to indicate that the mux layer should be modified to handle this functionality, incorporating this into the existing H.221 was judged to be unpalatable and thus the “video-mux” solution appeared to be the best available solution if such a scheme were to be adopted.

#### 2.2.1.13	Start-Code Emulation [Q15a54]

During the meeting one of the members wrote a contribution describing several start-code emulation problems.  The recommendations of this contribution were adopted, with a bit added to one entry (index 22) in a table to ensure future expandibility.  A need for examination of the text to clarify that the four motion vectors for a macroblock would only be allowed when Annex F or J was active was expressed.

### 2.2.2	Review and Verification of Drafted Content [Q15a05]

#### 2.2.2.1	Bitstream Exchanges [Q15a39]

A summary was presented of the activity that has taken place regarding bitstream exchanges for verification of the drafted content.  Several bitstreams have been decoded with no significant errors covering a number of features of the current drafted text (although generally one feature at a time rather than several in combination).  This has become an active and successful exchange of material, although more work is certainly needed in this area.

#### 2.2.2.2	Annex I Advanced Intra Coding [Q15a48]

Results were presented of experiments using the current design of Annex I.  This work confirmed to the group that the drafted content was a good design.  It was noted that when the quantization step size is low, Annex I is best when used in conjunction with Annex T (to prevent clipping of the range of DCT coefficient values).

#### 2.2.2.3	Annex P Reference Picture Resampling [Q15a34, Q15a43]

Two demonstrations were given to show that Annex P is effective for providing a warping capability for global motion compensation.  This work confirmed to the group that the drafted content had adequate performance.  (Also see Section 2.2.1.7.)

#### 2.2.2.4	Annex Q Reduced Resolution Update [Q15a32]

The results of two sets of experiments by different companies were presented to try to show that Annex Q was effective.  However, the consensus of the group was that no compelling demonstration of any benefit for this annex was shown.  The quality of the video when not using the annex was judged to be superior to that of Annex Q by many of the participants.  The group has asked for compelling evidence to be shown by the next meeting to show that Annex Q is useful.  It was requested that Annex Q also be compared against QCIF-only coding as well as against CIF-only coding.  The proponent indicated that he thought that some other video sequences or other bit rates might show better performance than the demonstrations that have been shown at the Nice and Portland meetings.  If the absence continues of any evidence that this mode is useful, necessary, and worth its complexity, the group has expressed that it would have to remove this mode from the draft text at the next meeting. (Also see Section 2.2.1.8.)

### 2.2.3	TMN7 Enhancement Efforts [Q15a04]

A report of interim test model enhancement activities was presented, and the group expressed appreciation for the work in this area.

#### 2.2.3.1	Post-processing [Q15a13, Q15a50]

Demonstrations were shown for two post-processing filers.  One of these [Q15a13] was intended to operate as a post-processing stage when the in-loop Annex J deblocking filter was in use.  This filter was judged to be effective and to work well in conjunctions with Annex J, and was adopted into the test model.

A second filter [Q15a50] was described which was intended to operate as a post-processing stage when the in-loop Annex J deblocking filter was not in use.  Although this filter was judged to have very good perceptual performance by the group, it was not adopted into the test model, in order to avoid having two very different post-processing filters in the test model at the same time.

#### 2.2.3.2	Rate Control [Q15a20]

A method of controlling the quantization step size was presented which was demonstrated to the group’s satisfaction to have superior performance to the current TMN7 rate control.  Since the performance superiority of this method was demonstrated and since the complexity of the proposed scheme did not seem overly burdensome (although it does require motion estimation to be performed on the whole picture prior to starting difference coding of any macroblocks), the new rate control method was adopted into the test model.

#### 2.2.3.3	Motion Estimation [Q15a45]

A method of reduced-computation motion estimation was proposed for adoption into the test model reference encoder description.  This was reported to have performance very close to that of the current full-search minimum-SAD method currently found in our test model.

Several members of the group reported that they were impressed with the reported results.  There was some skepticism that the results could be verified, that the results would remain the same at high bit rates, or that certain sequences would make the method fail.  A consensus was reached to put a description of this reduced-computation method into test model as an alternative to the full-search method which will also remain included in the test model description.  Further comparative testing of these methods by the group is expected.

#### 2.2.3.4	Software availability [Q15a37, Q15a43, Q15a45]

Software for the test model will be made available by Deutsche Telecom on an ftp site listed in Q15a37, which supports Annexes D, E, F, G, I, J, M, L, O, T, and possibly P.  The encoding and decoding routines in this software (everything but the display routines) were implemented independently of the Telenor software.

Global motion estimation software as used in [Q15a43] will be made available from PictureTel, and decoding warping software for the Reference Picture Resampling mode decoding operation has been made available by the same party.

Software for the a variation on the test model is also available from the University of British Columbia [Q15a45].  This software is based on the Telenor implementation of TMN6, and has a reduced complexity motion estimation

### 2.2.4	Coordination with Q.11/16 through Q.14/16 [Q15a01, Q15a29, Q15a30, Q15a38, Q15a42, Q15a53]

The group agreed with the coordination plan which was adopted in Geneva [Q15a01].

A joint session was held with Q.11/16 at this meeting.  System-level requirements for H.263+ Annex N were discussed in that joint session [Q15a29, Q15a30], and the group agreed that the drafted solution appeared to be a good approach.  A proposal was made to adopt H.263+ into H.320 [Q15a38].  It was mentioned that the definition of preferred mode combinations was necessary for this activity to succeed.  This issue was commented on by the Rapporteur of Q.12/16, who mentioned that he was in favor of the proposal in spirit, but was particularly concerned that the preferred modes should be supported across different network and terminal types, in order to achieve maximal cross-network interoperability.  A letter was received from the Q.13 Rapporteur [Q15a42] regarding preferred modes and the requirements of H.323 environments.  The preferred mode definition process was later taken up by Q.15/16 in a separate session.

### 2.2.5	Preferred mode combinations for H.263/H.263+ [Q15a26, Q15a41, Q15a42, Q15a44, Q15a53, Q15a57]

A document was presented which outlined a method of defining profiles for different basic types of systems and levels for enhanced performance capability within the profiles.  The group agreed that this contribution provided a good framework for starting our discussion about this issue.  Some misgivings were expressed regarding 1) the ability of this solution to interoperate across different types of networks, 2) the total number of different recommended modes constructed by this “product code” plan, and 3) whether all subsets of all of the recommended modes were required.

A contribution [Q15a41] and an informal letter from the Rapporteur of Q.13/16 [Q15a42] were presented which described the needs of H.323 systems for improvement in video performance.  A large range of operating environments and error/loss probabilities was targeted for work in these documents.  Document Q15a41 particularly indicated that the modes which he found most useful in these environments include 1) Independent Segment Decoding, 2) Reference Picture Selection, 3) Slice-Structured Coding, and 4) Scalability used in “separate logical channel” operation.  The first two of these four were particularly emphasized.

A contribution was provided which listed a set of six modes as proposed for having a good tradeoff of complexity and compression efficiency [Q15a44].  The group greeted this list enthusiastically and adopted it as the basis for further definition of preferred mode combination definitions.  The list was split into a few layers of increasing capability and then used as the first draft of a recommended mode combination list.

A letter was received from the Q.12/16 Rapporteur which expressed great interest in the work on the definition of preferred mode combinations.  An issue mentioned to be particularly of concern in that letter was that the preferred modes should be supported across different network and terminal types, in order to achieve maximal cross-network interoperability.

### 2.2.6	Relationship with ISO/IEC JTC1 SC29/ WG11 (MPEG) [Q15a19, Q15a27]

The relationship between the ITU and MPEG video coding standards was discussed at some length, including the presentation of two contributions on that topic.  The members of the group were encouraged to attend MPEG meetings and make their interests known there.  No specific liaison action was deemed  appropriate at this time.

### 2.2.7	Proposals for H.263++ extensions

#### 2.2.7.1	Custom Source Formats [Q15a17]

See section 2.2.1.2.

#### 2.2.7.2	Semi-Fixed length motion vectors [Q15a18]

A proposal was made to consider a modified entropy coding method for motion vectors which used a more constant number of bits per motion vector, in order to prevent loss of synchronization in the bitstream in the case of errors.  The group decided that further work was needed on this scheme to demonstrate its benefits in realistic environments.

#### 2.2.7.3	4MV with DQUANT [Q15a23]

See section 2.2.1.10.

#### 2.2.7.4	Multi-mode warping prediction [Q15a33]

A proposal was made for a new type of motion prediction using additional prediction block types.  A demonstration of this was provided, and a significant performance benefit seemed to be shown for some scenes (at a significant expense in encoder complexity).  However, the group did not take any specific action regarding this proposal at this time.

#### 2.2.7.5	Motion Compensation Error Accumulation [Q15a35]

See section 2.2.1.11.

#### 2.2.7.6	Change to Continuous Presence Multipoint Operation [Q15a46]

See section 2.2.1.12.

#### 2.2.7.7 Adaptive Quantization [Q15a49]

A method was presented which modified the reconstruction levels for inverse quantization of DCT coefficients.  The modified reconstruction values were signaled on a macroblock basis.  A significant performance benefit was shown (approximately 10% on some scenes), but at a large expense in encoder complexity.  The group agreed that this work appeared promising, and encouraged further investigation of the scheme.

## 2.3	Conclusions [Q15a57, Q15a59, Q15a60]

The work toward H.263+ decision in January of 1998 is progressing well, and appears to be on schedule to meet its goal.  The H.263++ extension work has not yet produced concrete drafted content by the group.  Work will be expected to better define the workplan of H.263++ extensions.  More work is necessary to reach verification and completion of H.263+, and to get it adopted into the appropriate systems.  However, we expect that work to progress with due speed.  The output draft of H.263+ [Q15a60], the output test model number eight (TMN8) description [Q15a59], and the output draft of preferred mode combinations [Q15a57] representing the consensus actions made at this meeting were later generated by the designated editors in the days following the meeting, and we thank the editors for their efforts.  No indication has since been received that these documents conflict with the consensus reached at the meeting on any detail whatsoever, with perhaps one minor exception (one person, Peter List, has reported that in the H.263+ draft [Q15a60], the prediction of the B motion vector values as described in section 2.2.1.4 above should have been initialized to zero rather than half of the P motion vector values when no motion vector would otherwise be available for use as a predictor).  This minor issue will be considered in our further work.

# 3.0	H.263L ACTIVITY [Q15a02, Q15a55]

H.263L addressed three major areas at this meeting:

- Conditions and Requirements for H.263L proposals

- Proposals for H.263L content, and

- Relationship with ISO/IEC JTC1 SC29/WG11 (MPEG).

## 3.1	Conditions and Requirements for H.263L Proposals [Q15a02, Q15a06, Q15a07, Q15a15, Q15a16, Q15a52, Q15a62]

Six contributions were addressed in this area, as listed in the table below.

| Document | Title |
| --- | --- |
| Q15a02 | Review of H.263L Ad Hoc Group Activites |
| Q15a06 | Review of H.263L Common Conditions for Testing |
| Q15a07 | Workplan and Objectives for H.263L |
| Q15a15 | Codec delay model for H.263L |
| Q15a16 | Liaison re: Audiovisual Quality Measurement |
| Q15a52 | User Requirements for H.263L |
| Q15a62 | Revised Common Conditions |

Document Q15a02 presented a review of the activities of the H.263L ad hoc group since the Nice meeting.  The only particular action considered by the group in that regard was the consideration of the frame rate specified for the 112 kbps sequence in the set of common conditions, for which the consensus reached by the ad hoc group was to not change the frame rate specified

Document Q15a06 presented the common conditions agreed upon for H.263L testing.  Changes were made to the set of common conditions based upon the liaison statement received from SG12 (Q15a16) and discussions during the meeting.  Based on the liaison from SG12, two additional sequences (Container Ship and News) were added to the set of 10 kbps sequences and one sequence (Silent) was added to the set of 112 kbps sequences.  This brings the number of sequences specified at each bit rate to four as was specified in the liaison from SG12.  The liaison from SG12 also indicted that end-to-end delay was an important issue.  To that end, it was agreed to require that H.263L proposals include information on delay performance.  Based upon input from the H.263L ad hoc group, it was agreed to add text to the common conditions document to indicate that in addition to the mandatory demonstration mode, those submitting demonstration material are encouraged also to present that material on alternative display technologies representative of those likely to be used in commercial products.

Document Q15a15 presented a proposal for modeling delay in video systems.  There was general agreement that this modeling effort would be useful in describing the delay in H.263L proposals.  However, it was noted that the description provided in the contribution was incomplete.  It was agreed that an ad hoc group be formed to refine the delay model (chaired by R. Fryer).  This ad hoc group will hold discussions using the advanced video coding email reflector and will present an updated delay model at the next meeting.

Document Q15a52 presented a set of user requirements for H.263L.  This document had been submitted to the Nice meeting, but was not fully discussed at that meeting.  No agreement could be reached on the specific performance requirements or the usage profiles contained within this contribution.  There was considerable discussion revolving around whether H.263+ could adequately address these usage profiles.

Document Q15a16 was a liaison from SG12.  SG12 recommend that changes be made to the set of common conditions for H.263L tests.  The group agreed to incorporate these changes (four sequences per bit-rate and mandatory delay information) into the set of common conditions.

Document Q15a07 presented a proposal for a workplan and objectives for H.263L.  The group discussed the objectives for H.263L and possible timeframes for completing the work.  It was agreed that a small group would meet with the objective of developing a time line for the completion of the work.  The results of this small group discussion were then presented at the meeting and are provided below.

Document Q15a62 was produced by the editor of the common conditions description, to represent the outcome of the common conditions actions at this meeting.

## 3.2	Proposals for H.263L Content [Q15a33, Q15a51]

Two H.263L proposals were presented, including video tape demonstrations.  Contribution Q15a33 presented a proposal using multimode warping prediction.  This work is based upon MPEG-4 core experiment P6.  It provides for increased coding efficiency.  It was presented as a candidate coding tool for either H.263++ or H.2.63L.

Contribution Q15a51 presented the results of experiments conducted by Nokia using their proposed algorithm.  An analysis of the decoder complexity was presented.  Also presented was a demonstration of real-time decoding on a 166 MHz Pentium PC.  A D-1 tape demonstration was presented which compared H.263 PB frame mode with the Nokia proposal utilizing B frame.  At low bit-rates the group felt that the Nokia proposal produced superior results.  At higher bit-rates the subjective differences were more difficult to observe.  It was noted that the complexity of the Nokia proposal was greater than the complexity associated with H.263, but the Nokia proposal produced improved video quality.

Both of these proposals are expected to be candidates for H.263L.

## 3.3 Relationship with ISC/IEC JTC1 SC29/WG11 (MPEG) [Q15a19, Q15a27]

Document Q15a19 presented the results of the coding efficiency work at the April MPEG meeting.  This was presented to the group for information.

Document Q15a27 presented important discussion points on the relationship between MPEG-4 and the ITU-T SG16 Q15.  It was noted that the target applications of MPEG-4 and SG16 Q15 are not completely aligned and therefore complete bit stream level compatibility probably cannot be achieved.  Requiring each standard to provide a compatibility mode is not a complete solution since any compatibility mode is not likely to provide full functionality.  As implementations move towards software based solutions, it may be possible to realize dual mode codecs, however implementations become easier if both standards are closely aligned.

The group discussed the alignment issue and determined that there were several technical differences between the MPEG-4 video coding standard and the H.263+ video coding standard.    One solution discussed was to attempt to influence MPEG-4 to make changes to their specification to minimize the differences.  Another option was to make changes to H.263+ to bring it into alignment with the MPEG-4 specification.  After much discussion it became clear that there was no strong agreement to either create a liaison to MPEG-4 or to modify H.263+.  Therefore no action was taken by the group.

## 3.4	H.263L Workplan

The work plan of H.263L was discussed in the small group meeting held from 11:00 to 13:00 on Thursday, and the following results were obtained:

### 3.4.1	General

A concern was expressed that we may need more research to find out the novel video coding algorithm which can fit into the H.263L framework. The consensus of the group was that the definition of the requirements for H.263L is currently the most important issue to be considered.

### 3.4.2	Time schedule

It was pointed out that Jan. 98, Nov. 98, Aug. 99, and June 2000 are the possible dates for determination or decision of H.263L, if we assume that the SG16 meeting should be held at regular 10 month intervals. In order to encourage the members to develop their technologies within the time-span of the schedule, it is essential to make a clear schedule. The group agreed, therefore, to the following tentative time schedule (knowing that there still are opportunities to revise it at specific points of time).

Complete requirements: December 97

Call for proposal issued: early 98

Evaluation of the proposed technologies: November 98

Decision will be made whether the proposed technologies have the potential to meet the requirements after the collaborative effort for improvement by the group. If not, the schedule will be delayed.

Test Model of H.263L (TML1?) defined

The project steps into the collaboration phase towards determination.

Note: continuation of the work will be dependent upon an analysis of the suitability of submissions received.

Determination: August 99

The result of collaborative improvement will be evaluated again at this point. If it is decided that the requirements are not met, the determination will be delayed to a later date.

Decision: June 2000

It was also agreed that ITU-T should always be open for new proposals which are submitted after the deadline specified in the call for proposals. Such proposals will be considered as candidate technologies for work items in the next study period.

## 3.5	Discussion on Requirements for H.263L

The group reached a consensus that H.263L should support functionalities that cannot be supported by  H.263++. However, since the target performance of H.263++ is currently not clear, it was agreed to start the H.263L work with challenging requirements. Although H.263L should cover various applications including both real-time and non-real-time applications, it was agreed that a single codec solution should be sought.

The requirements document will be drafted by the H.263L Ad Hoc committee through e-mail discussion using the Q.15 e-mail reflector. Mr. Keiichi Hibi, the Associate Rapporteur of Q.15, was assigned as the editor of this document. The group will start their work with the documents whose structure and contents are open at this moment. However, the following four items pointed out in document Q15a52 were agreed as the guideline for the work:

- the work must focus on the very low bitrate real-time profiles, as indeed this is the most challenging

- emphasis needs to be placed on low complexity software solutions, as the easiest markets to open will be those with software only implementation

- rate control mechanisms must be adaptable, preferably within one codec, between both constant frame rate modes and variable frame rate modes, depending upon application

- test material needs to be chosen to reflect the fast motion aspects of true audiovisual source materials, with many scene changes, pans and tilts etc.

It was agreed that a sufficient amount of time should be allocated for the discussion on H.263L requirements at the next September meeting. The chairman also expressed his view of having a joint session with the Q.11-Q.14 systems related members at the next meeting, for the discussion about the H.263L requirements and the future work plan of Q.15.

## 3.6	Handling of the methods already proposed

The chairman expressed his concern about how to deal with the methods which have already been proposed for H.263L. The response from the members was that these proposals should be re-submitted with possible improvement after the call for proposal is issued.

The group agreed that technical proposals/information before the call for proposals are also welcome, for catching up with the state of the art.

## 3.7	Relation between H.263++ and H.263L

The members requested that the work plan of H.263++ be strictly defined and the difference between H.263L and other areas of work be clarified.

The group agreed that a technology proposed for H.263L may be categorized as a canditate technology for H.263++, if the group considers it to be a more preferable place, according to the result of the evaluation for the proposal. It is expected that proposals for H.263++ will maintain the H.263 syntax. Proposals for H.263L are not required to maintain the H.263 syntax.

## 3.8	Conclusions

The group reports that it has formulated a workplan for H.263L and is anticipating fruitful work toward the creation of this new standard in the future.

# 4.0	VIDEO CODING FOR SIGN LANGUAGE AND LIP-READING USE [Q15a61]

Our group is considering ways to ease implementation of video telephony with good performance for sign language at low bitrates. Possible output of this activity is an appendix to H.263+ approved by SG 16 in January 1998.

The decision to initiate this activity was taken in Q15/16 after discussion of the needs from the sign language application and an acknowledgement of the importance to meet the requirements from the sign language users.

## 4.1	Study of User Needs [Q15a08, Q15a09]

The meeting reviewed the documents Q15a08, Q15a09 reporting user needs investigations and performance of H.263 on sign language.

## 4.2	Conclusions and Workplan [Q15a10, Q15a11]

The conclusions and suggestions for controls in Q15a11 were presented and the suggestions for contents and work plan for an appendix to H.263+ in Q15A10 was reviewed and agreed.

# 5.0	CLOSING PLENARY (HELD JOINTLY WITH Q11/16)

The following sections report on the Q15 activities at the joint closing plenary.

## 5.1	Presentation of reports and Review of Results of Breakout Sessions [Q15a55, Q15a56, Q15a61]]

- The H.263L activity report [Q15a55] was presented by the activity chairperson Mr. Keiichi Hibi

- The H.263+ and H.263++ activity report [Q15a56] was presented by the activity chairperson Mr. Gary Sullivan

- The Sign Language and Lip-Reading activity report [Q15a61] was presented by the activity chairperson Gunnar Hellstrom

## 5.2	Liaisons [Q11a68/Q15a58]

A liaison to SG12 (created in joint with Q.11), written thanking them for their response regarding performance aspects of audio and video in multimedia terminals [Q11a68/Q15a58] was approved.  It appears in Annex E to this report.

Having not received a formal reply to the last liaison to MPEG4, the group agreed that there was no need to send a liaison at this time.  However, it was agreed that Mr. Gary Sullivan of PictureTel Corp., USA, will be a coordinator to MPEG4 regarding Q15 work (also see Ad Hoc Committee section below regarding a planned coordination meeting).

## 5.3	Ad Hoc Committees

The following Ad Hoc Committes were formed.  The membership of these Ad Hoc Committees is listed in Annex A to this report.  These AHCs will conduct their work primarily by email using the Q15 advanced video email reflector.  The H.263+ Draft Text Refinement AHC will also hold a meeting during the Stockholm MPEG meeting of July 21-25.

| AD HOC COMMITTEE | CHAIRPERSON |  |
| --- | --- | --- |
| H.263+ Draft Text Refinement |  | Gary Sullivan |
| H.263+ Recommended Modes and Integration with Terminal Standards (Joint AHC with Q11) |  | Stephan Wenger |
| H.263+ Bitstream Verification |  | Tom Gardos |
| H.263+ Packetization |  | Tom Gardos |
| Test Model Enhancement and Software Development |  | Keiichi Hibi |
| H.263++ Development |  | Gary Sullivan |
| H.263L Development |  | Keiichi Hibi |
| Video Coding for Sign Language and Lip-Reading Use |  | Gunnar Hellström |

## 5.4	Future Meetings

| Experts meeting | September 8-11 1997 | Bend OR USA | Hosted by Intel Corp. |
| --- | --- | --- | --- |
| Experts meeting | December 2-5 1997 | TDB | HOST URGENTLY SOLICITED |

There was an active discussion on participation of Q.15 at the 8-11 September 1997 Experts meeting.  The host noted that there were limited meeting facilities at the site and that another conference was previously booked there before it was known that Q.15 was also planning to meet with Q.11-Q.14.  The host felt that they had direction to prioritize Q.11 and Q.23 work before other questions or WP3.  They were willing to try to arrange additional space in the near vicinity (within about 10 miles) but were not able to commit at this time.  It was agreed that both Q.11 and Q.15 would try to work out the most time optimized schedules in an effort to accommodate both groups.  This will be worked with appropriate Working Party Chairmen and the host for the planned meeting.

The group gave a much deserved Thank You to the hosts, Intel Corporation, to the provider of the D-1 Video equipment, Sharp Corporation and to the meeting management team, Ms. Julie Higgins of Preferred Meetings,  Mr. Hansen and Mr. Gardos of Intel Corporation.

Having no further business, the meeting was adjourned.

# 6.0 SUMMARY OF MEETING RESULTS [Q15a57, Q15a58, Q15a59, Q15a60, Q15a62]

The results of the meeting are summarized as follows:

- Various revisions to the drafted text of the H.263+ revision of ITU-T Recommendation H.263 were approved.  These revisions appear in document Q15a60.  H.263+ appears to be on-target for decision in January of 1998.

- A draft of recommended mode combinations for H.263+ was created.  This appears in document Q15a57.  The final version of this text is intended for possible adoption as an informative Appendix to Recommendation H.263.

- A revised test model number 8 description of a reference encoder for was agreed upon for use in future activities of our Question.  This appears in document Q15a59.

- A workplan was adopted for the H.263L future standardization effort.  The workplan appears in section 3.4 of this report.

- A set of revised common conditions were adopted for proposals for the H.263L future standardization effort.  This set of conditions appears in document Q15a62.

- The group agreed to conduct further work toward consideration of the use of video coding for sign language and lip-reading applications.

- A liaison statement to SG12 was produced.  This appears in document Q11a68/Q15a58 and in Annex E to this report

- Various Ad-Hoc Committees were formed which are listed in Annex A of this report.

# ANNEX A - Ad Hoc Committees

## A.1	H.263+ Draft Text Refinement

- Refine Draft of H.263+ optional enhancements to Recommendation H.263 with the goal of Decision status in January 1998.

- This AHC will meet during the July 21-25 MPEG meeting in Stockholm

Membership:

G. Sullivan, Chairperson

| H. Ahn | O. Andersson | B. Andrews | C. Aeyeung |
| --- | --- | --- | --- |
| M. Bace | G. Bang | S. Battista | A. Bist |
| G. Bjontegaard | P. Boissonade | L. Brown | R. Burns |
| G. Campbell | T. Chen | W. Chen | S. Cheung |
| J. Cho | D. Choi | C. Choo | C. Christopoulos |
| K. Chu | R. Chuang | P. Crouch | S. Eckart |
| T. Einarsson | N. Faerber | C. Fan | R. Fischer |
| J. Fournier | M. Frater | R. Fryer | S. Fukunaga |
| T. Gardos | R. Grinnell | S. Gupta | I. Haignere |
| G. Hellström | B. Haskell | K. Hibi | H. Hsieh |
| P. Hsieh | A. Hsueh | C. Huang | F. Huffman |
| Y. Hung | K. Illgner | R. Ivy | J. Jeon |
| T. Joe | G. Jung | J. Jung | T. Kawahara |
| M. Karczewicz | A. Kaup | M. Kerdranvat | C. Ketcham |
| G. Kim | J. Kim | G. Klungsøyr | K. Kneib |
| N. Lam | D. Lappe | G. Liang | K. Lillevold |
| S. Lin | D. Lindbergh | P. List | N. Lu |
| M. Luomi | Y. Machida | Y. Mao | J. Mason |
| Y. Matsumura | T. Miki | J. Moon | J. Muller |
| P. Mulroy | A. Nakagawa | T. Nakai | Y. Nakaya |
| M. Nilsson | G. Nitsche | K. O’Connell | K. Oehler |
| T. Ohya | K. Okada | J. Osterman | N. Patel |
| D. Pian | D. Qian | C. Quist | X. Ran |
| R. Reynolds | A. Riccomi | P. Riglet | A. Rodriguez |
| G. Russo | A. Salai | R. Schaphorst | J. C. Schmitt |
| I. Sebestyen | R. Sjoberg | H. Tanaka | E. Tanskanen |
| Y. Tomita | P. Voois | D. Wang | Y. Wang |
| J. Webb | W. Welsh | C.-J. Westelius | M. Willebeek-Lemair |
| M. Whybray | I. Wong | W. Wu | M. Zeug |
| C. Zhu | Q. Zhu |  |  |

## A.2	H.263+ Recommended Modes and Integration with Terminal Standards (Joint AHC with Q11)

For every transport stack including H.323, H.324, and H.320

- work on a set of recommended mode combinations

- work on signaling of these mode combinations in H.245 and H.242

- discuss suggestions for reducing the number of optional mode combinations

In addition to the general issues above

- work on the Tandberg proposal regarding H.263 Annex C [Q11a57/Q15a46]

- work on H.245 code points for H.263 Draft Annex N [Q11a55/Q15a29, Q11a56/Q15a30]

Work on preferred mode combinations of the modes affecting the complexity (MIPS) / quality tradeoff will be based on the accepted proposal Q15a44 as refined in Q15a57.  Base on this document, a small number of levels (probably 3 or 4) should be identified.

In addition, the AHC will discuss the impact of the more system-related optional modes, especially H.263 Draft Annexes K, N, O, and R for various applications, which will be listed by their recommendation family numbers (e.g., H.320, H.323, and H.324).  Based on Mr. Okubo’s input [Q11a59/Q15a53], care will be taken to ensure easy interoperability between the different network worlds to allow gateways without transcoding.  However, an optional adaption to the network characteristics of the widely accepted networks (ISDN H.320, PSTN H.324, Internet and LAN H.323) will be given priority.

The primary means of communication will be the Q15 email reflector.  Information regarding system issues (especially regarding H.242 and H.245) will also be posted to the appropriate system reflectors for Q11 and Q13.

Membership:

- Wenger, Chairperson

| P. Boissonade | S. Cheung | I. Haignere | Y. Machida |
| --- | --- | --- | --- |
| J. Schmitt | G. Sullivan |  |  |

## A.3	H.263+ Bitstream Verification

This group will perform exchanges of coded H.263+ video bitstreams and verify the H.263+ syntax description validity.

Membership:

Tom Gardos, Chairperson

| B. Andrews | J. Muller | T. Nakai | K. Lillevold |
| --- | --- | --- | --- |
| P. List | G. Sullivan |  |  |

## A.4	H.263+ Packetization

This group will work toward definition of an RTP payload packetization format for H.263+ video bitstreams.

Membership:

- Gardos, Chairperson

| B. Andrews | G. Bjontegaard | K. Hibi | A. Kaup |
| --- | --- | --- | --- |
| H. Li | Y. Machida | T. Nakai | R. Schaphorst |
| G. Sullivan | H. Tanaka | Y. Tomita | O. Yamagishi |
| M. Zeug |  |  |  |

## A.5	Test Model Enhancement and Software Development

This group will act with a mandate to:

- Improve the description of TMN8 for H.263+ Annexes

- evaluate technology for non-normative enhancements to TMN8

- develop reference software and useful software tools for video coding activities

The objectives of the group are to

- improve the performance of the coding method described in the test model, incorporating proposed technology which may not be necessary in the final standard text

- demonstrate the achievement by the H.263+ extensions using the test model specification for those outside the ITU-T Q15/SG16 group if possible

- draft an implementers guide, informative appendix, or other such information to give good examples to users of the standard if necessary

The technical areas relevant to the ad-hoc activity are

- pre- and post- processing

- rate control issues

- other encoder-specific content such as motion estimation methods, motion vector search ranges, mode decision mechanisms, etc.

- evaluation of technology to realize a low-complexity codec (especially for an encoder)

- define experimental conditions and/or new video sequences to provide good demonstrations of the performance of various methods

Membership:

- Hibi, Chairperson

| H. Ahn | O. Andersson | B. Andrews | C. Aeyeung |
| --- | --- | --- | --- |
| M. Bace | G. Bang | S. Battista | A. Bist |
| G. Bjontegaard | P. Boissonade | L. Brown | R. Burns |
| G. Campbell | T. Chen | W. Chen | S. Cheung |
| J. Cho | D. Choi | C. Choo | C. Christopoulos |
| K. Chu | R. Chuang | P. Crouch | S. Eckart |
| T. Einarsson | N. Faerber | C. Fan | R. Fischer |
| J. Fournier | M. Frater | R. Fryer | S. Fukunaga |
| T. Gardos | R. Grinnell | S. Gupta | I. Haignere |
| G. Hellström | B. Haskell | K. Hibi | H. Hsieh |
| P. Hsieh | A. Hsueh | C. Huang | F. Huffman |
| Y. Hung | K. Illgner | R. Ivy | J. Jeon |
| T. Joe | G. Jung | J. Jung | T. Kawahara |
| M. Karczewicz | A. Kaup | M. Kerdranvat | C. Ketcham |
| G. Kim | J. Kim | G. Klungsøyr | K. Kneib |
| N. Lam | D. Lappe | G. Liang | K. Lillevold |
| S. Lin | D. Lindbergh | P. List | N. Lu |
| M. Luomi | Y. Machida | Y. Mao | J. Mason |
| Y. Matsumura | T. Miki | J. Moon | J. Muller |
| P. Mulroy | A. Nakagawa | T. Nakai | Y. Nakaya |
| M. Nilsson | G. Nitsche | K. O’Connell | K. Oehler |
| T. Ohya | K. Okada | J. Osterman | N. Patel |
| D. Pian | D. Qian | C. Quist | X. Ran |
| R. Reynolds | A. Riccomi | P. Riglet | A. Rodriguez |
| G. Russo | A. Salai | R. Schaphorst | J. C. Schmitt |
| I. Sebestyen | R. Sjoberg | H. Tanaka | E. Tanskanen |
| Y. Tomita | P. Voois | D. Wang | Y. Wang |
| J. Webb | W. Welsh | C.-J. Westelius | M. Willebeek-Lemair |
| M. Whybray | I. Wong | W. Wu | M. Zeug |
| C. Zhu | Q. Zhu |  |  |

## A.6	H.263++ Development

To consider the need for adopting additional incremental enhancements to Recommendation H.263 beyond those the current H.263+ draft

Membership:

G. Sullivan, Chairperson

| H. Ahn | O. Andersson | B. Andrews | C. Aeyeung |
| --- | --- | --- | --- |
| M. Bace | G. Bang | S. Battista | A. Bist |
| G. Bjontegaard | P. Boissonade | L. Brown | R. Burns |
| G. Campbell | T. Chen | W. Chen | S. Cheung |
| J. Cho | D. Choi | C. Choo | C. Christopoulos |
| K. Chu | R. Chuang | P. Crouch | S. Eckart |
| T. Einarsson | N. Faerber | C. Fan | R. Fischer |
| J. Fournier | M. Frater | R. Fryer | S. Fukunaga |
| T. Gardos | R. Grinnell | S. Gupta | I. Haignere |
| G. Hellström | B. Haskell | K. Hibi | H. Hsieh |
| P. Hsieh | A. Hsueh | C. Huang | F. Huffman |
| Y. Hung | K. Illgner | R. Ivy | J. Jeon |
| T. Joe | G. Jung | J. Jung | T. Kawahara |
| M. Karczewicz | A. Kaup | M. Kerdranvat | C. Ketcham |
| G. Kim | J. Kim | G. Klungsøyr | K. Kneib |
| N. Lam | D. Lappe | G. Liang | K. Lillevold |
| S. Lin | D. Lindbergh | P. List | N. Lu |
| M. Luomi | Y. Machida | Y. Mao | J. Mason |
| Y. Matsumura | T. Miki | J. Moon | J. Muller |
| P. Mulroy | A. Nakagawa | T. Nakai | Y. Nakaya |
| M. Nilsson | G. Nitsche | K. O’Connell | K. Oehler |
| T. Ohya | K. Okada | J. Osterman | N. Patel |
| D. Pian | D. Qian | C. Quist | X. Ran |
| R. Reynolds | A. Riccomi | P. Riglet | A. Rodriguez |
| G. Russo | A. Salai | J. Salonen | R. Schaphorst |
| J. C. Schmitt | I. Sebestyen | R. Sjoberg | H. Tanaka |
| E. Tanskanen | Y. Tomita | P. Voois | D. Wang |
| Y. Wang | J. Webb | W. Welsh | C.-J. Westelius |
| M. Willebeek-Lemair | M. Whybray | I. Wong | W. Wu |
| M. Zeug | C. Zhu | Q. Zhu |  |

## A.7	H.263L Development

The H.263L Ad Hoc group will conduct activities toward the following

- Enhance the “Common Conditions” for Testing (M.Zeug).

- Coordinate/merge proposals of interest to H.263L (R. Fryer).

- Completion of a Delay Model based on Q15a15 (R. Fryer).

- Development of an H.263L Requirements Document (K. Hibi).

Membership:

K. Hibi, Chairperson

| H. Ahn | O. Andersson | B. Andrews | C. Aeyeung |
| --- | --- | --- | --- |
| M. Bace | G. Bang | S. Battista | A. Bist |
| G. Bjontegaard | P. Boissonade | L. Brown | R. Burns |
| G. Campbell | T. Chen | W. Chen | S. Cheung |
| J. Cho | D. Choi | C. Choo | C. Christopoulos |
| K. Chu | R. Chuang | P. Crouch | S. Eckart |
| T. Einarsson | N. Faerber | C. Fan | R. Fischer |
| J. Fournier | M. Frater | R. Fryer | S. Fukunaga |
| T. Gardos | R. Grinnell | S. Gupta | I. Haignere |
| G. Hellström | B. Haskell | K. Hibi | H. Hsieh |
| P. Hsieh | A. Hsueh | C. Huang | F. Huffman |
| Y. Hung | K. Illgner | R. Ivy | J. Jeon |
| T. Joe | G. Jung | J. Jung | T. Kawahara |
| M. Karczewicz | A. Kaup | M. Kerdranvat | C. Ketcham |
| G. Kim | J. Kim | G. Klungsøyr | K. Kneib |
| N. Lam | D. Lappe | G. Liang | K. Lillevold |
| S. Lin | D. Lindbergh | P. List | N. Lu |
| M. Luomi | Y. Machida | Y. Mao | J. Mason |
| Y. Matsumura | T. Miki | J. Moon | J. Muller |
| P. Mulroy | A. Nakagawa | T. Nakai | Y. Nakaya |
| M. Nilsson | G. Nitsche | K. O’Connell | K. Oehler |
| T. Ohya | K. Okada | J. Osterman | N. Patel |
| D. Pian | D. Qian | C. Quist | X. Ran |
| R. Reynolds | A. Riccomi | P. Riglet | A. Rodriguez |
| G. Russo | A. Salai | R. Schaphorst | J. C. Schmitt |
| I. Sebestyen | R. Sjoberg | H. Tanaka | E. Tanskanen |
| Y. Tomita | P. Voois | D. Wang | Y. Wang |
| J. Webb | W. Welsh | C.-J. Westelius | M. Willebeek-Lemair |
| M. Whybray | I. Wong | W. Wu | M. Zeug |
| C. Zhu | Q. Zhu | P. Haavisto | M. Jändel |
| H. Li |  |  |  |

## A.8	Video Coding for Sign Language and Lip-Reading Use

Goals:

- Create an informative condensed description of characteristics of sign language and lip reading of interest for the performance of video coding.

- Determine requirements in terms of traditional video transmission terms.

- Measure acceptable blur of details in motion in sign language and lipreading, and find a way to express the blur measurements.

- Seek a subjective evaluation on the effects on sign language and lip reading from each option in H.263+.

- Determine the need for control over video coding from sign language and lip reading users.

- Conduct information transfer between the video quality related work in SG12 and this ad hoc group.

- Influence the test model, test sequences and goals of H.263+ and H.263L.

Time scale:

A first draft document should be finished by September 8.  A document should be ready for approval in Jan-98.

Membership:

G. Hellström, Chairperson

| B. Andrews | S.-C. Cheung | R. Fryer | M. Jändel |
| --- | --- | --- | --- |
| H. Li | R. Schaphorst | G. Sullivan | M. Whybray |
| M. Zeug |  |  |  |

# ANNEX B - Attendee List [Q15aTD-1]

| R/A | Q.11 or Q.15 |  |  |  | NAME |  | ORGANIZATION |  | COUNTRY |  | TELEPHONE |  | FACSIMILE |  | E-MAIL |  | AREA OF INTEREST 1 2 3 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  | Q11Q15 | Y | Reuven Ackner |  | Acer Advanced Labs |  | USA |  | +1 408 467 7415 |  | +1 408 467 7475 |  | reuven_ackner@acer.com |  | X |  | X | X | X |
| A |  |  | Y | Vladimire Alekseyev |  | Xirlink, Inc. |  | USA |  | +1 408 324 2100 |  | +1 408 324 2101 |  |  |  |  |  |  |  |  |
| R |  | Q15 | Y | Barry Andrews |  | 8x8, Inc |  | USA |  | +1 408 654 0905 |  | +1 408 980 0432 |  | andrews@8x8.com |  | X |  | X |  | X |
| R |  | Q15Q11 | Y | Ola Andersson |  | Telia Research AB |  | Sweden |  | +46 87 075559 |  | +46 87 075310 |  | ola.b.andersson@telia.se |  | X |  |  | X | X |
| R |  | Q15 | Y | Matthew Bace |  | PictureTel |  | USA |  | +1 508 623 4236 |  | +1 503 749 2804 |  | bace@pictel.com |  | X |  |  |  |  |
| R |  | Q15Q11 | Y | Goran Bang |  | Ericsson Radio Systems |  | Sweden |  | +46 8 757 0168 |  | +46 8 757 5550 |  | goran.bang@era-t.ericsson.se |  | X |  |  | X | X |
| R |  | Q11 | Y | Suneel Bhagat |  | PictureTel |  | USA |  | +1 508 623 4058 |  | +1 508 749 2804 |  | sbhagat@pictel.com |  |  |  |  |  | X |
| R |  | Q15 | Y | Anurag Bist |  | Rockwell Semiconductor Systems |  | USA |  | +1 714 221 6873 |  | +1 714 221 6361 |  | anurag.bist@nb.rockwell.com |  | X |  |  |  |  |
| R |  | Q15 | Y | Gisle Bjontegaard |  | Telenor Satellite Services AS |  | Norway |  | +47 23 13 8381 |  | +47 22 77 7980 |  | gisle.bjontegaard@oslo.satellite.telenor.no |  | X |  |  |  |  |
| A |  |  | Y | Patrick Boissonade |  | France Telecom |  | France |  | +33 1 45 29 41 40 |  | +33 1 45 29 52 94 |  | Patrick.boissonade@issy.cnet.fr |  | X |  |  |  |  |
| R |  | Q11Q15 | Y | James Brailean |  | Motorola |  | USA |  | +1 347 576 7883 |  | +1 347 576 8378 |  | brailean@ccrl.mot.com |  | X |  |  |  | X |
| R |  | Q11Q15 | Y | Simao Campos Neto |  | COMSAT |  | USA |  | +1 301 428 4516 |  | +1 301 428 9287 |  | simao@ctd.comsat.com |  | X |  | X | X | X |
| R |  | Q11Q15 | Y | Sen-ching Cheung |  | VTEL/CLI |  | USA |  | +1 408 922 5489 |  | +1 408 232 6165 |  | sccheung@clix.com |  | X |  | X | X | X |
|  |  |  | Y | C.C. Chou |  | Xirlink, Inc. |  | USA |  | +1 408 324 2100 |  | +1 408 324 2101 |  | chi-chin.chou@xirlink.com |  |  |  |  |  |  |
| A |  | Q15 | Y | Guy Coté |  | University of British Columbia |  | Canada |  | +1 604 822 4988 |  | +1 604 822 5949 |  | guyc@ee.ubc.ca |  | X |  |  |  |  |
| R |  |  | Y | Pedro Crespo |  | Telefonica |  | Spain |  | +34 13 37 41 33 |  | +34 13 37 44 91 |  | pcrespo@roger.tid.es |  | X |  |  | X | X |
| R |  |  | Y | Goran Djuknic |  | Lucent Technologies |  | USA |  | +1 973 386 4162 |  | +1 973 386 2651 |  | goran@lucent. com |  |  |  |  | X |  |
| R |  | Q15Q11 | Y | Christer Englund |  | MIT |  | Finland |  | +1 617 258 7939 (+358 0 456 6503) |  | +1 617 253 7140 (+358 0 456 7028) |  | cenglund@rpcp.mit.edu |  | X |  | X | X | X |
| R |  | Q11Q15 | Y | Roberto Flaiani |  | Aethra Telecomunicazioni |  | Italy |  | +39 71 218981 |  | +39 71 887077 |  | aethra@fastnet.itor roberto.flaiani@itu.ch |  | X |  |  |  |  |
|  |  |  |  | Chad Fogg |  | Chromatic |  | USA |  | +1 408 752 9352 |  | +1 408 752 9300 |  | cfogg@chromatic.com |  | X |  |  |  | X |
| R |  | Q15 | Y | Jérôme Fournier |  | Phillips |  | France |  | +33 1 45 10 69 20 |  | +33 1 45 10 69 60 |  | fournier@lep.reasearch.philips.com |  | X |  |  |  |  |
| R |  | Q15 | Y | Richard Fryer |  | University of Strathclyde |  | UK |  | +44 141 552 4400 |  | +44 141 552 5330 |  | rjf@cs.strath.ac.uk |  | X |  |  |  |  |
| R |  | Q15Q11 | Y | Frank Fu |  | Nokia Mobile Phones |  | USA |  | +1 972 257 9832 |  | +1 972 257 9849 |  | frank.fu@nmp.nokia.com |  | X |  | X | X | X |
| R |  | Q15 | Y | Tom Gardos |  | Intel Corporation |  | USA |  | +1 503 264 6459 |  | +1 503 264 9372 |  | trgardos@ibeam.jf.intel.com |  | X |  |  |  |  |
| R |  | Q11Q15 | Y | Tom Geary |  | Rockwell Semiconductor Systems |  | USA |  | +1 714 221 4092 |  | +1 714 221 9642 |  | tom.geary@nb.rockwell.com |  | X |  | X | X | X |
| A |  | Q15 | Y | Jay Gilbert |  | Intel |  | USA |  | +1 503 264 8798 |  | +1 503 264 6181 |  | Jay_Gilbert@ccm.jf.intel.com |  | X |  | X | X | X |
| R |  | Q11 | Y | Chet Graham |  | 8x8, Inc. |  | USA |  | +1 617 740 2547 |  | +1 408 980 0432 |  | chet@8x8.com |  | X |  | X |  | X |
| R |  | Q11 | Y | Dr. Gary Greenbaum |  | Progressive Networks |  | USA |  | +1 206 679 2339 |  |  |  | GaryG@prognet.com |  | X |  | X |  |  |
| R |  | Q11Q15 | Y | Petri Haavisto |  | Nokia Research Center |  | Finland |  | +358 3 272 5836 |  | +358 3 272 5897 |  | petri.haavisto@research.nokia.fi |  | X |  | X | X | X |
| R |  | Q11 | Y | Chris Hansen |  | Intel Corporation |  | USA |  | +1 503 264 8877 |  | +1 503 264 3375 |  | chris_hansen@ccm.jf.intel.com |  |  |  |  |  | X |
| R |  | Q11 | Y | Matti Harmovaara |  | Nokia Research Center |  | Finland |  | +358 50 553 4755 |  | +358 10 505 6794 |  | matti.harmovaara@nmp.nokia.com |  | X |  |  | X | X |
| R |  | Q11Q15 | Y | Gunnar Hellström |  | Omnitor |  | Sweden |  | +46 701 100 501 |  | +46 855 600206 |  | gunnar.hellstrom@omnitor.se |  | X |  |  |  | X |
| R |  | Q15Q11 | Y | Keiichi Hibi |  | Sharp Corporation |  | Japan |  | +81 43 299 8613 |  | +81 43 299 8608 |  | hibi@trl.mkhar.sharp.co.jp |  | X |  | X | X | X |
| A |  | Q11 | Y | Ryan Holmqvist |  | Lucent Technologies |  | USA |  | +1 610 709 1161 |  | +1 610 709 1076 |  | ryanh@lucent.com |  |  |  |  | X | X |
| A |  | Q15Q11 | Y | Chien-Min Hueng |  | Sorenson Vision |  | USA |  | +1 801 792 1116 |  | +1 801 792 1101 |  | chien-min@s-vision.com |  | X |  | X | X | X |
| R |  | Q15Q11 | Y | Magus Jändel |  | Ericsson Telecom AB |  | Sweden |  | +46 8 719 4983 |  | +46 871 90460 |  | m.jandal@cla.ericsson.se |  | X |  |  |  | X |
| A |  | Q11 | Y | Timor Kadir |  | Motorola |  | UK |  | +44 1256 4843 |  | +44 1256 4844 |  | BTK002@email.mot.com |  |  |  |  | X | X |
| A |  | Q11 | Y | Toshiro Kawahara |  | NTT DoCoMo |  | Japan |  | +81 468 59 3931 |  | +81 468 57 7909 |  | kawahara@mlab.nttdocomo.co.jp |  | X |  | X | X | X |
| A |  | Q11 | Y | Masaki Kameya |  | OKI Electric Industry Co., Ltd. |  | Japan |  | +81 273 25 1119 |  | +81 273 24 2186 |  | kameya@hardx.takasaki.oki.co.jp |  |  |  |  | X | X |
| R |  | Q11 | Y | Greg Kamosa |  | UCLA Electrical Engineering |  | USA |  | +1 310 794 1631 |  | +1 310 825 7928 |  | kamosa@icsl.ucla.edu |  |  |  |  | X | X |
| A |  | Q15 | Y | Marta Karczewicz |  | Nokia Research Center |  | Finland |  | +358 3 272 5379 |  | +358 3 272 5322 |  | marta.karczewicz@research.nokia.com |  | X |  |  |  | X |
| R |  | Q15 | Y | Dr. Andre Kaup |  | Siemens AG |  | Germany |  | +49 89 636 49015 |  | +49 89 636 48000 |  | Andre.Kaup@mchp.siemens.de |  | X |  |  |  | X |
| R |  | Q11Q15 | Y | Michel Kerdranvat |  | Thomson Multimedia |  | France |  | +33 2 99 27 30 00 |  | +33 2 99 27 30 01 |  | kerdranvat@thmulti.com |  | X |  |  |  | X |
| R |  | Q11Q15 | Y | Dan Klenke |  | VTEL |  | USA |  | +1 408 922 5521 |  | +1 408 922 4608 |  | dank@clix.com |  | X |  | X | X | X |
| R |  | Q11Q15 | Y | Gunn Kristin Klungsoyr |  | Telenor R&D |  | Norway |  | +47 63 84 00 |  | +47 63 81 00 76 |  | gkk@nta.no |  | X |  |  |  |  |
| R |  | Q11 | Y | Li-Feng Lai |  | CCL/Industrial Technology Research Institute |  | Taiwan |  | +886 35 91 79 49 |  | +886 35 82 00 27 |  | ltlai@m2sun3.ccl.itri.org.tw |  | X |  |  |  |  |
|  |  | Q11 | Y | Kyminh Liang |  | Sorenson Vision |  | USA |  | +1 801 792 1117 |  | +1 801 792 1101 |  | kyminhl@s-vision.com |  |  |  |  |  | X |
| R |  | Q11Q15 | Y | Shawmin Lei |  | Sharp Laboratories |  | USA |  | +1 360 817 8446 |  | +1 360 817 8436 |  | shawmin@sharplabs.com |  | X |  |  | X |  |
| R |  | Q15 | Y | Haibo Li |  | Linkoping University Dept. Electrical Engineering |  | Sweden |  | +46 13 28 22 07 |  | +46 13 13 92 82 |  | haibo@isy.liu.se |  | X |  |  | X |  |
| R |  | Q11Q15 | Y | David Lindbergh |  | PictureTel |  | USA |  | +1 508 623 4351 |  | +1 508 749 2804 |  | lindbergh@pictel.com |  | X |  | X | X | X |
| R |  | Q15 | Y | Karl Lillevold |  | Intel Corp |  | USA |  | +1 503 264 7923 |  | +1 503 264 9372 |  | klillevo@ibeam.jf.intel.com |  | X |  |  |  |  |
| R |  | Q15 | Y | AlanLippman |  | Progressive Networks |  | USA |  | +1 206 674 2291 |  | +1 206 674 2699 |  | alan@prognet.com |  | X |  | X | X | X |
| R |  | Q15 | Y | Peter List |  | Deutsche Telekom |  | Germany |  | +49 6151 83 3878 |  | +49 6151 83 4842 |  | P.List@berkom.de |  | X |  |  |  |  |
| R |  | Q15 | Y | Paul Long |  | Smith Micro Software |  | USA |  | +1 503 641 1221 ext. 12 |  | +1 503 641 3344 |  | plong@smithmicro.com |  |  |  |  |  | X |
|  |  | Q15 | Y | Bruno Loret |  | France Telecom/CNET |  | France |  | +33 1 45 29 60 74 |  | +33 1 45 29 52 94 |  | loret@issy.cnet.fr |  | X |  |  |  |  |
| R |  | Q15 | Y | Naiqian Lu |  | Hitachi America Ltd. |  | USA |  | +1 408 922 4118 |  | +1 408 954 8907 |  | nlu@hsmi.com |  | X |  |  |  | X |
|  |  |  | Y | Marko Luomi |  | Nokia Research Center |  | Finland |  | +358 3 272 5690 |  | +358 3 272 5241 |  | marko.luomi@research.nokia.com |  | X |  |  |  | X |
| R |  | Q11Q15 | Y | Yutaka Machida |  | Matsushita |  | Japan |  | +81 45 939 1287 |  | +81 45 934 8765 |  | ymachida@adl.mci.mei.co.jp |  | X |  |  |  | X |
| R |  | Q11Q15 | Y | Yuhai Mao |  | ESS Technology |  | USA |  | +1 510 492 1409 |  | +1 510 492 1233 |  | yuhai.mao@esstech.com |  | X |  | X | X | X |
| R |  | Q15 | Y | Fernando Martins |  | Intel Corporation |  | USA |  | +1 503 264 1184 |  | +1 503 264 9372 |  | fernando.martins@ccm.jf.intel.com |  | X |  |  |  |  |
| R |  | Q15 | Y | Dr. John Mason |  | ImageCom |  | UK |  | +44 1 628 ____ |  | +44 1 628 3913 |  | j_mason@imagcom.demon.co.uk |  | X |  |  |  |  |
| R |  | Q15 | Y | Kristine Matthews |  | Sharp Laboratories |  | USA |  | +1 360 817 7529 |  | +1 360 817 8436 |  | matthews@sharplabs.com |  | X |  |  |  | X |
| R |  | Q15 | Y | Robert McKenzie |  | Smith Micro Software, Inc. |  | USA |  | +1 503 641 1221 |  | +1 503 641 334??? |  | bmckenzie@smithmicro.com |  | X |  | X |  | X |
| R |  | Q15 | Y | John Muller |  | Iterated Systems |  | USA |  | +1404 264 8048 |  | +1 404 264 8313 |  | jmuller@iterated.com |  | X |  |  | X | X |
| R |  | Q15 | Y | Akira Nakagawa |  | Fujitsu Laboratories Ltd. |  | Japan |  | +81 447 5426 |  | +81 447 5423 |  | anaka@flab.fujitsu.co.jp |  | X |  |  |  |  |
| R |  | Q15 | Y | Toshisa Nakai |  | OKI |  | Japan |  | +81 6 949 5105 |  | +81 6 949 5108 |  | nakai@kansai.oki.co.jp |  | X |  |  |  |  |
| R |  | Q15 | Y | Yuichiro Nakaya |  | Hitachi Ltd |  | Japan |  | +81 423 231111 |  | +81 423 27 776 |  | y-nakaya@crlhitachi.co.jp |  | X |  |  |  |  |
| R |  | Q11 | Y | Mickey Nasiri |  | Ericsson |  | Sweden |  | +46 8 726 2125 |  | +46 8 18 7620 |  | mickey.nasiri@clab.ericsson.se |  |  |  |  |  | X |
| R |  | Q11Q15 | Y | Kevin O'Connell |  | Motorola |  | USA |  | +1 847 576 1956 |  | +1 847 538 4593 |  | oconnell@ccrl.mot.com |  | X |  |  |  | X |
| R |  | Q11 | Y | Dong-Seek Park |  | Samsung Electronics |  | USA |  | +1 310 825 4090 |  | +1 310 825 7928 |  | dspark@icsl.ucla.edu |  | X |  | X | X | X |
| R |  | Q15 | Y | Jeong Hoon Park |  | Samsung Electronics |  | Korea |  | +82 331 200 3746 |  | +82 331 200 3716 |  | celp@mmrnd.sec.samsung.co.kr |  | X |  | X |  |  |
| A |  | Q11Q15 | Y | Saurel Quettan |  | Bellsouth Cellular Corporation |  | USA |  | +1 404 249 4518 |  | +1 404 249 4343 |  | squettan@bellsouth.net |  |  |  |  | X |  |
| R |  | Q11Q15 | Y | Cor Quist |  | KPN Research |  | Netherlands |  | +31 70 332 4005 |  | +31 70 332 6477 |  | C.P.Quist@research.kpn.com |  | X |  |  |  | X |
| R |  | Q15 | Y | Jordi Ribas |  | Sharp Laboratories |  | USA |  | +1 360 817 8487 |  | +1 360 817 8436 |  | jordi@sharplabs.com |  | X |  |  |  |  |
| R |  | Q11 | Y | Jouni Salonen |  | Nokia Research Center |  | Finland |  | +358 3 272 5482 |  | +358 3 272 5322 |  | jouni.salonen@research.nokia.com |  |  |  |  |  | X |
| R |  | Q11Q15 | Y | Richard Schaphorst |  | Delta InformationSystems |  | USA |  | +1 215 657 5270ext. 11 |  | +1 215 657 5273 |  | rschaphorst@delta-info.com |  | X |  | X | X | X |
| R |  | Q11Q15 | Y | Istvan Sebestyen |  | Siemens |  | Germany |  | +49 89 72 24 72 30 |  | +49 89 72 24 7713 |  | istvan.sevestyen@mch.pr.siemens.de |  | X |  | X | X | X |
| R |  | Q11 | Y | Michael Severa |  | UCLA Electrical Engineering |  | USA |  | +1 310 206 3428 |  | +1 310 825 7928 |  | severa@icsl.ucla.edu |  |  |  |  | X | X |
| R |  | Q15 | Y | John Sievers |  | PictureTel |  | USA |  | +1 508 258 4573 |  |  |  | johns@pictel.com |  | X |  |  |  |  |
| R |  | Q15 | Y | Rickard Sjöberg |  | Ericsson Telecom AB |  | Sweden |  | +46 87 19 00 92 |  | +46 87 19 04 60 |  | rick@clab.ericsson.se |  | X |  |  |  |  |
| R |  | Q15 | Y | Gary Sullivan |  | PictureTel |  | USA |  | +1 508 623 4324 |  | +1 508 749 2804 |  | garys@pictel.com |  | X |  | X | X | X |
| R |  | Q15 | Y | Kai Sun |  | Sharp Digital Info |  | USA |  | +1 714 903 3072 |  | +1 714 261 9321 |  | sun@sdismtp.sharpwa.com |  | X |  |  |  |  |
| R |  | Q11 | Y | Hirokazu Tanaka |  | Toshiba Corporation |  | Japan |  | +81 44 548 5350 |  | +81 44 520 5864 |  | horikaza@cns.clab.toshiba.co.jp |  | X |  | X | X | X |
| R |  | Q11 | Y | Yasuhiro Tomita |  | NTT |  | Japan |  | +81 468 59 3749 |  | +81 468 59 2829 |  | tomita@nttvdt.hil.ntt.co.jp |  | X |  |  |  | X |
|  |  |  | Y | Eyal Trachtman |  | INMARSAT |  | UK |  | +44 171 728 1112 |  | +44 171 728 1689 |  | Eyal_Trachtman@Inmarsat.org |  |  |  |  |  | X |
| R |  | Q11 | Y | John Villasenor |  | University of California, Los Angeles |  | USA |  | +1 310 825 0228 |  | +1 310 825 7928 |  | villa@icsl.ucla.edu |  | X |  |  |  | X |
| R |  | Q11 | Y | Peter Vogel |  | Robert Bosch GmbH |  | Germany |  | +49 5121 49 3119 |  | +49 5121 49 2520 |  | Peter.Vogel@fr.bosch.de |  | X |  |  | X | X |
| R |  | Q15 | Y | Tzu-Hsin Wang |  | CCL/Industrial Tech. Research Institute |  | Tiawan |  | +886 35 91 79 49 |  | +886 35 82 00 27 |  | jswang@m2sun3.ccl.itri.org.tw |  | X |  |  |  |  |
| R |  | Q15 | Y | Stephan Wenger |  | Teles AG |  | Germany |  | +49 31 314 73160 |  | +49 30 314 25156 |  | steve@cs.tu-berlin.de |  | X |  | X |  |  |
| R |  | Q11Q15 | Y | Mike Whybray |  | BT Labs |  | UK |  | +44 1 473 643180 |  | +44 1 473 645011 |  | mike.whybray@bt-sys.bt.co.uk |  | X |  |  |  |  |
| R |  | Q11 | Y | Bernhard G. Wimmer |  | Siemens |  | Germany |  | +49 89 636 50417 |  | +49 85 636 52393 |  | Bernhard.G.Wimmer@MchP.Siemens.De |  | X |  |  |  | X |
| R |  | Q15 | Y | Yiu Fai Wong |  | Vxtreme, Inc. |  | USA |  | +1 408 617 2369 |  | +1 408 245 9770 |  | iwong@vxtreme.com |  |  |  |  |  |  |
| R |  | Q11 | Y | Osamu Yamagishi |  | Toshiba Corporation |  | Japan |  | +81 44 5485344 |  | +81 44 5205864 |  | yama@cns.clab.toshiba.co.jp |  |  |  |  | X | X |
| R |  | Q15 | Y | Michael Zeug |  | Iterated Systems |  | USA |  | +1 404 264 8094 |  | +1 404 264 8313 |  | mzeug@iterated.com |  | X |  | X | X | X |

# ANNEX C - Document List [Q15a00]

LIST OF DOCUMENTS FOR THE FIRST MEETING OF THE ITU-T SG16 Q.15

VIDEO CODING EXPERTS GROUP

| DOCUMENT NO. | SOURCE |  |  | P U R P O S E | TITLE | A D M I N | H. 2 6 3 + | H. 2 6 3 + + | H. 2 6 3 L | P R I O R | O T H E R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | COUNTRY | ORGANIZATION/ EXPERT |  |  |  |  |  |  |  |  |
| q15aTD-0 | Q15/16 Rapporteur, Gary Sullivan |  |  | I | Invitation to the First Meeting of Experts for Advanced Video Coding | X |  |  |  |  |  |
| q15aTD-1 | Q.15/16 Rapporteur, Gary Sullivan |  |  | I | Persons Attending the First Meeting of Experts for Advanced Video Coding | X |  |  |  |  |  |
| q15aTD-2 | Q.15/16 Rapporteur, Gary Sullivan |  |  | I | Plan of the First Meeting | X |  |  |  |  |  |
| q15a00 | Q.15/16 Rapporteur, Gary Sullivan |  |  | I | List of Documents for the First ITU-T SG16 Q.15 Meeting of Experts for Advanced Video Coding | X |  |  |  |  |  |
| q15a01 | Q.15/16 Rapporteur, Gary Sullivan |  |  | I | Report on Video Activities at the SG16 meeting in Geneva, CH, March 17-27, 1997 | X |  |  |  |  |  |
| q15a02 | M. Zeug, Chairperson H.263L Group |  |  | I | Report of H.263L Activity |  |  |  | X |  |  |
| q15a03 | G. Sullivan, Chairperson H.263+ Group |  |  | I | Report of H.263+ Activity |  | X |  |  |  |  |
| q15a04 | K. Hibi, Chairperson TMN7 Enhancement Group |  |  | I | Report of Test Model Enhancement Activity |  | X |  |  |  |  |
| q15a05 | - Sullivan, Editor H.263+ Draft Recommendation |  |  | D | Draft Text of H.263+ |  | X |  |  |  |  |
| q15a06 | M. Zeug, Chairperson H.263L Group |  | I | Revised Common Conditions for H.263L Experiments |  |  |  | X |  |  |
| q15a07 | M. Zeug, Chairperson H.263L Group |  | I | Workplan and Objectives for H.263L |  |  |  | X |  |  |
| q15a08 | Sweden | Omnitor, Gunnar Hellström | I | Analysis of the relationship Coding Scheme-Bitrate in a disabled people environment |  | X |  |  |  | X |
| q15a09 | Sweden | Omnitor, Gunnar Hellström | I | Research on requirements of videotelephony for sign language communication at Telefonica I&D |  | X |  |  |  | X |
| q15a10 | Sweden | Omnitor, Gunnar Hellström | P | Proposed contents and workplan for sign language and lip-reading appendix to H.263+ |  | X |  |  |  | X |
| q15a11 | Sweden | Omnitor, Gunnar Hellström | P | Control requirements for sign language use of videophones |  | X |  |  |  | X |
| q15a12 | Norway | Telenor International, Gisle Bjontegaard | P | A simple modification to the loop filter in Annex J |  | X |  |  |  |  |
| q15a13 | Norway | Telenor International, Gisle Bjontegaard | I | A post filter that performs well combined with the modified loop filter in Annex J |  | X |  |  |  |  |
| q15a14 | Norway | Telenor International, Gisle Bjontegaard | P | Modification of Improved PB-frames mode to allow backward prediction |  | X |  |  |  |  |
| q15a15 | Scotland | University of Strathclyde, Richard Fryer | I | Codec delay model for H.263.L novel algorithm evaluation – a proposal for adoption into the common conditions |  |  |  | X |  |  |
| q15a16 (q11a16) | ITU-T SG12 |  |  | L | Liaison re Audiovisual Quality Measurement | X |  |  |  |  | X |
| q15a17 | USA | VTEL, Yi Tong Tse | P | Support of Interlaced Scan Format in H.263+ |  | X |  |  |  |  |
| q15a18 | British Columbia | University of British Columbia, Guy Côté | P | Semi-fixed Length Motion Vector Coding for Noisy Environment |  |  | X |  |  |  |
| q15a19 | USA | Iterated Systems, John Muller | I | Report of MPEG-4 Coding Efficiency Activities at the Bristol Meeting |  | X | X | X |  |  |
| q15a20 | USA | Sharp Labs, Jordi Ribas | I | Rate Control for low-delay Video Communications |  | X |  |  |  |  |
| q15a21 | Germany | Deutsche Telekom Berkom GmbH, Peter List | P | Proposal for modified MBTYPE-table |  | X |  |  |  |  |
| q15a22 | Germany | Deutsche Telekom Berkom GmbH, Peter List | I | Verification of advanced deblocking filter |  | X |  |  |  |  |
| q15a23 | Sweden | Torbjorn Einarsson, Ph.D. | P | Proposal for Simultaneous 4MV and DQUANT |  | X |  |  |  |  |
| q15a24 | Sweden | Torbjorn Einarsson, Ph.D. | P | Changes in Text Related to Multiple Layers |  | X |  |  |  |  |
| q15a25 | Q.15/16 Rapporteur, Gary Sullivan |  |  | I | List of Experts for Q.15/16 Advanced Video Coding | X |  |  |  |  |  |
| q15a26 | Japan | Sharp Corporation, Keiichi Hibi | P | Definition of profiles for H.263+ options |  | X |  |  |  |  |
| q15a27 | Japan | Sharp Corporation, Keiichi Hibi | I | Compatibility between ITU-T Recommendation and MPEG-4 |  | X | X | X |  |  |
| q15a28 | Japan | Oki Electronic, Toshihisa Nakai | P | Proposed Amendments to Annex N of H.263+ |  | X |  |  |  |  |
| q15a29 (q11a55) | Japan | Oki Electronic, Toshihisa Nakai | I | H.263 Annex N (reference picture selection) support in H.324 |  | X |  |  |  |  |
| q15a30 (q11a56) | Japan | NTT, Yasuhiro Tomita | P | Proposed revision of H.245 syntax and semantics for H.263 Annex N |  | X |  |  |  |  |
| q15a31 | Japan | Fujitsu Laboratories Ltd., Akira Nakagawa | I | Comparison of Annex Q and MRR |  | X |  |  |  |  |
| q15a32 | Japan | Fujitsu Laboratories Ltd., Akira Nakagawa | I | Results of performance verification of Annex Q by two independently implemented simulations |  | X |  |  |  |  |
| q15a33 | Japan | Sharp Corporation, Keiichi Hibi | P | Multimode warping prediction of coding efficiency improvement |  |  | X | X |  |  |
| q15a34 | Japan | Hitachi, Yuichiro Nakaya, et al | P | Reduction of computational complexity in bilinear warping |  | X |  |  |  |  |
| q15a35 | Japan | Hitachi, Yuichiro Nakaya, et al | P | Avoidance of error accumulation in motion compensation |  | X |  |  |  |  |
| q15a36 | Germany | Deutsche Telekom Berkom GmbH, Peter List | P | Proposal for motion vector prediction in Annex-M and Annex-O |  | X |  |  |  |  |
| q15a37 | Germany | Deutsche Telekom Berkom GmbH, Peter List | I | Announcement: Free H263+ Codec-software available from Deutsche Telekom |  | X |  |  |  |  |
| q15a38 (q11a31) | USA | VTEL, Sen-ching Samson Chueng | P | Support of H.263+ on H.320 |  | X |  |  |  |  |
| q15a39 | USA | Intel Corp, Tom Gardos | I | Report on H.263+ bit stream exchanges |  | X |  |  |  |  |
| q15a40 | Norway | Telenor International, Gisle Bjontegaard | P | A modified ‘Altered quantization step for chroma coefficients’ (Annex T) |  | X |  |  |  |  |
| q15a41 | Germany | Dr. Stephan Wenger | P | Preferred Mode combinations for use of H.263+ in H.323 systems |  | X |  |  |  |  |
| q15a42 | Q.13/16 Rapporteur Dale Skran |  | L | Liaison to Q.15/16 re Profiles for H.263+ modes in H.323 systems | X | X |  |  |  |  |
| q15a43 | USA | PictureTel Corp., Matt Bace | I | Demonstration of automatic global motion compensation with H.263+ RPR mode |  | X |  |  |  |  |
| q15a44 | USA | PictureTel Corp., Matt Bace & Gary Sullivan | P | Proposal for preferred mode of operation for real-time reliable-channel communications |  | X |  |  |  |  |
| q15a45 | Canada | University of British Columbia, Guy Coté | I | Computation constrained fast-search motion estimation algorithm for TMN7 |  | X |  |  |  |  |
| q15a46 (q11a57) | Norway | Tandberg Telecom AS, Tom-Ivar Johansen | P | Proposed change to Continuous Presence Multipoint mode in Recommendation H.263 section C.4 |  | X | X |  |  |  |
| q15a47 | USA | Intel Corp., Tom Gardos | P | A Proposal for Scalability in one-dimension |  | X |  |  |  |  |
| q15a48 | USA | Intel, Corp., Fernando Martins, Ph.D. | I | Performance Evaluation of H.263+ Advanced Intra Coding |  | X |  |  |  |  |
| LATE REGISTRATIONS: |  |  |  |  |  |  |  |  |  |  |
| q15a49 | USA | Rockwell Semiconductor Systems, Anurag Bist | P | An Adaptive Quantization Scheme for H.263++ |  |  | X |  |  |  |
| q15a50 | Finland | Nokia Research, Marta Karczewicz | I | Improved Deblocking Filter |  | X |  |  |  |  |
| q15a51 | Finland | Nokia Research, Marta Karczewicz | P | New video codec for H.263L |  |  |  | X |  |  |
| q15a52 | UK | Orange Telecom, John Kelliher | I | User Requirements for H.263L |  |  |  | X |  |  |
| q15a53 (q11a59) | Japan | GCL, Sakae Okubo | I | Adoption of H.263+ into H.320, compatibility, and profiles |  | X |  |  |  |  |
| q15a54 | USA | Motorola, Kevin O’Connell | P | Start Code Emulation Problems in H.263+ (and proposed solutions) |  | X |  |  |  |  |
| q15a55 | Keiichi Hibi, chairperson H.263L Activity |  | I | H.263L Activity Report |  |  |  | X |  |  |
| q15a56 | Gary Sullivan, chairperson H.263+/H.263++ Activity |  | I | H.263+/H.263++ Enhancement Activity Report |  | X | X |  |  |  |
| q15a57 | Matt Bace, editor H.263+ Preferred mode specification |  | D | Draft for H.263+ Preferred Mode Combination Specification |  | X |  |  |  |  |
| q15a58 (q11a68) | Q.11/16 Rapporteur, Tom Geary, and Q.15/16 Rapporteur, Gary Sullivan |  |  | L | Liaison to Study Group 12 | X |  |  | X |  |  |
| q15a59 | Tom Gardos, editor Test Model Specification |  |  | I | H.263+ Video Codec Test Model Number 8, as agreed to at the Portland meeting |  | X |  |  |  |  |
| q15a60 | Gary Sullivan, editor H.263+ draft text |  |  | D | Draft Text of H.263+, as agreed to at the Portland meeting |  | X |  |  |  |  |
| q15a61 | Gunnar Hellström, chairperson Sign Language & Lip Reading Activity |  |  | I | Sign Language and Lip Reading Activity Report |  |  |  |  |  | X |
| q15a62 | Michael Zeug, editor H.263L Common Conditions Specification |  |  | I | Revised Common Conditions for H.263L Experiments, as agreed to at the Portland meeting |  |  |  | X |  |  |
| q15a63 | Q15/16 Rapporteur, Gary Sullivan |  |  | I | Report of the First ITU-T SG16 Q.15 Experts meeting (Meeting A) Portland, Oregon USA (Study Period 1997-2000) | X | X | X | X | X | X |

Annex D - Meeting Agenda [Q15aTD-2]

FIRST MEETING OF ITU EXPERTS

FOR Q.15/16 ADVANCED VIDEO CODING

JUNE 24-27, 1997

MEETING AGENDA

1.0	Opening Plenary

1.1	Review of the Q.15 Meeting Report
		from the SG16 meeting, Geneva March 17-27, 1997			Q15a01

1.2	Report of Ad Hoc Committees

- H.263L (M. Zeug)								Q15a02

- H.263+ (G. Sullivan)								Q15a03

- TMN7 Enhancement (K. Hibi)						Q15a04

1.3	Organizational Items

- Attendee List									Q15aTD-1

- Review Experts List								Q15a25

- Meeting Invitation for this meeting						Q15aTD-0

1.4	Liaison Statements and Collaborative Letters

- Study Group 12								Q15a16

- Study Group 16 Question 13						Q15a42

1.5	Document Review								Q15a00

1.6	Review of the Meeting Plan							Q15aTD-2

- Room Assignments

- Joint Meetings

1.7	Future Meeting Dates

- Q.15/16 Experts September 8-11, 1997 Bend OR (co-located with Q.11)

- Q.15/16 Experts December 2-5, 1997 Looking for a host (co-located Q.11)

2.0	H.263, H.263+, and H.263++				Q15a03, Q15a05, Q15a56

- Proposed revisions								Q15a03
Custom Source Formats								Q15a17
Annex J Deblocking Filter						Q15a12, Q15a22
Annex M Improved PB frames					Q15a14, Q15a36
Annex N Reference Picture Selection						Q15a28 
Annex O Temporal, SNR, and Spatial Salability
						Q15a21, Q15a24, Q15a36, Q15a47
Annex P Reference Picture Resampling						Q15a34
Annex Q Reduced-Resolution Update						Q15a31
Annex T Modified Quantization							Q15a40
4MV with DQUANT								Q15a23
Motion Compensation Error Accumulation					Q15a35
Continuous-Presence Multipoint							Q15a46
Start-Code Emulation								Q15a54

- Review and verification of drafted content					Q15a05
Bitstream Exchanges								Q15a39
Annex I Advanced Intra Coding							Q15a48
Annex P Reference Picture Resampling				Q15a34, Q15a43
Annex Q Reduced Resolution Update						Q15a32
TMN7 Enhancement Efforts							Q15a04
Post-processing							Q15a13, Q15a50
Rate Control									Q15a20
Motion Estimation									Q15a45
Software availability				Q15a37, Q15a43, Q15a45

- Coordination with Q.11/16 through Q.14/16
				Q15a01, Q15a29, Q15a30, Q15a38, Q15a42, Q15a53

- Preferred mode combinations for H.263/H.263+	
					Q15a26, Q15a41, Q15a42, Q15a44, Q15a53

- Relationship with ISO/IEC JTC1 SC29/ WG11 (MPEG)		Q15a19, Q15a27

- Proposals for H.263++ extensions
Custom Source Formats								Q15a17
Semi-Fixed length motion vectors							Q15a18
4MV with DQUANT								Q15a23
Multi-mode warping prediction							Q15a33
Motion Compensation Error Accumulation					Q15a35
Change to Continuous Presence Multipoint Operation				Q15a46
Adaptive Quantization								Q15a49

- H.263L							Q15a02, Q15a07, Q15a55

- Conditions and requirements for H.263L proposals
					Q15a06, Q15a15, Q15a16, Q15a52, Q15a62

- Proposals for H.263L content
Multi-mode warping prediction							Q15a33

- Relationship with ISO/IEC JTC1 SC29/ WG11 (MPEG)		Q15a19, Q15a27

- Coordination, Sign Language, and Other business				Q15a61

- Coordination of video coding needs for Q.11/16 through Q.14/16 standards							Q15a01, Q15a29, Q15a30, Q15a38, Q15a42

- Coordination with other Organizations
SG12									Q15a16
ISO/IEC SC29 WG11						Q15a19, Q15a27

- Consideration of video coding needs for users of sign-language and lip-reading
						Q15a08, Q15a09, Q15a10, Q15a11

5.0	CLOSING PLENARY

5.1	Presentation and Review of Results of Meeting Sessions
								Q15a55, Q15a56, Q15a61

5.2	Liaison Statements and Collaborative Letters to be Written		Q15a58

5.3	Plans for Future Work

- Ad Hoc committees

5.4	Future Meetings

# ANNEX E - Liaison to SG 12 [Q11a68/Q15a58]

| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Circuit-Switched Networks Experts Group / Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q11-A-68 / Q15-A-58 Filename: q11a68.doc / q15a58.doc |
| --- | --- |

| Questions: | Q.10/12, Q.11/12, Q.??/12 |  |
| --- | --- | --- |
| Source: | Q.11/16 Rapporteur Tom Geary Rockwell International 4311 Jamboree Road Newport Beach, CA, USA Email: tom.geary@nb.rockwell.com Phone: +1 714 221 4092 Fax: +1 714 221 6511 | Q.15/16 Rapporteur Gary Sullivan PictureTel Corp. M/S 635 100 Minuteman Rd. Andover, MA 01810 USA Email: garys@pictel.com Tel: +1.508.623.4324 Fax: +1.508.749.2804 |
| Title: | Liaison to ITU-T SG12 |  |
| Approval: | Approved at the Q.11&Q.15/16 Rapporteur Group Meetings 6/97 |  |
| Purpose: | Liaison |  |

_____________________________

Q.11/16 and Q.15/16 wish to thank SG 12 for their liaison letter, which was sent in response to our request for help in your area of expertise.

Regarding item 1 of your liaison letter, on the topic of characterization of H.324 (Q.11/16) multimedia terminals:

- Thank you for the information which you have provided regarding the status of your work.

- We have found your comments regarding the tolerances and usefulness of synchronization for intelligibility to be valuable, and we will consider them in our future efforts.

- We have also forwarded some of the information that you have provided to those working on other relevant Questions in our Study Group (Q.4, Q.7, Q.23 / SG 16).

Regarding item 2 of your liaison letter, on the topic of low bit-rate video coding (Q.15/16) common experimental conditions (LBC-96-139):

- The number of sequences which we now recommend for testing at each bit rate in our agreed common conditions has been increased to be four in every case.

- We agree that delay information is essential in the evaluation of video codecs. As a result of your comments, we have made delay information mandatory on all submissions.

- We have included some of the information regarding delay which you provided to us in our revised common conditions description document.

Regarding item 3 of your liaison letter, on the topic of the development of an objective video quality measurement recommendation (P.OVQ):

- Thank you for the information which you have provided regarding the status of your work.

- We hope that this effort can succeed in developing a reliable video quality measurement tool.

# ANNEX F - Proposed Draft Text for Reduced-Complexity Reference Picture Resampling (Annex P of H.263+)

Description of virtual frame warping

Rewrite section P.3 as follows:

P.3.	Resampling Algorithm

The method described in this section shall be mathematically equal in result to that used to generate the samples of the resampled reference picture. Using the integer warping parameters wx0,wy0,wxx,wyx,wxy,wyy,wxxy,and wyxy, integer parameters vx00,vy00,vxH0,vyH0,vx0V,vy0V,vxHV, and vyHV  which denote the x- and y-displacements at the corners of the picture in 32-pixel accuracy (the actual displacements are obtained by dividing these values by 32) are defined as

vx00=16wx0vy00=16wy0vxH0=16wx0+wxx+2(HR−H)vyH0=16wy0+wyxvx0V=16wx0+wxyvy0V=16wy0+wyy+2(VR−V)vxHV=16wx0+wxx+wxy+wxxy+2(HR−H)    vyHV=16wy0+wyx+wyy+wyxy+2(VR−V)

By applying bilinear extrapolation to these vectors, the integer parametersvxLT,vyLT,vxRT,vyRT,vxLB,vyLB,vxLB,andvyRB which denote the x- and y-displacements at (0, 0), (H’, 0), (0, V’), and (H’, V’) in 1/32 pel accuracy (the actual displacements are obtained by dividing these values by 32) are defined as

vxLT=vx00vyLT=vy00vxRT=(H−H')vx00+H'vxH0H     vyRT=(H−H')vy00+H'vyH0HvxLB=(V−V')vx00+V'vx0VVvyLB=(V−V')vy00+V'vy0VVvxRB=(V−V')(H−H')vx00+H'uvxH0+V'(H−H')vx0V+H'vxHV(HV)vyRB=(V−V')(H−H')vy00+H'vyH0+V'(H−H')vy00+H'vyHV(HV)

where “//” denotes integer division that rounds the quotient to the nearest integer, and rounds half integer values away from 0. H’ and V’ are defined as the smallest integers that satisfy the following condition

H'³H,V'³V,V'=2m,H'=2n,m and n are non-negative integers.

Next, the integer parameters vxL(j),vyL(j),vxR(j),and vyR(j)which denote the x- and y-displacements at (0, j+1/2) and (H’, j+1/2) in 1/32 pel accuracy (the actual displacements are obtained by dividing these values by 32) are defined using one dimensional linear interpolation as

vxL(j)=(2V'−2j−1)vxLT+(2j+1)vxLB(2V©)        vyL(j)=(2V'−2j−1)vyRT+(2j+1)vyRB(2V©)vxR(j)=(2V'−2j−1)vxLT+(2j+1)vxLB(2V©)vyR(j)=(2V'−2j−1)vyRT+(2j+1)vyRB(2V©)

Finally, the parameters that specify the transformed position in the reference picture becomes

IR(i,j)=Pi+(2H©−2i−1)vxL(j)+(2i+1)vxR(j)+16SH©P(32SH©P)JR(i,j)=Pj+(2H©−2i−1)vyL(j)+(2i+1)vyR(j)+16SH©P(32SH©P)iR(i,j)=IR(i,j)P    jR(i,j)=JR(i,j)PÆx=IR(i,j)−(IR(i,j)P)P   Æy=JR(i,j)−(JR(i,j)P)P

where

“///”: 	integer division with rounding towards the negative infinity,

P:	accuracy of x- and y-displacements (P=2 when DA=“0” and P=16 when DA=“1”, see section P.2.1. for the definition of DA),

(IR(i, j)+1/2, JR(i, j)+1/2):	location of the transformed position in 1/P pixel accuracy (both IR(i, j) and JR(i, j)  are integers),

(iR(i, j)+1/2, jR(i, j)+1/2):	location of the sampling point near the transformed position (both iR(i, j)  and jR(i, j)  are integers),

(Æx,Æy): 	bilinear interpolation coefficients of the transformed position.

The computation of this equation can be simplified by replacing the divisions by shift operations, since 32SH’/P =2m+2 and P =24 (when P=4 and S=2).

Using these parameters, the sample value, EP(i, j) of the pixel located at (i+1/2, j+1/2) in the resampled picture is obtained by

EP(i,j)=((P-Æy)(P-Æx)ER(iR,jR)+ÆxER(iR+1,jR)                  +Æy(P-Æx)ER(iR,jR+1)+ÆxER(iR+1,jR+1)+P22)P2

where “/” denotes division by truncation. iR and jR are simplified notations for iR(i, j) and jR(i, j), and ER(iR, jR) denotes the sample value of the pixel located at (iR+1/2, jR+1/2) in the reference picture.

The resampling algorithm described above can be rewritten as a psudo-code as follows. To simplify the description of the resampling algorithm, a function prior_sample is defined.  Its purpose is to generate a pixel value for any integer location (m,n) relative to the prior reference picture sampling grid:

clip(xmin, x, xmax) {
  if (x < xmin) {
    return xmin;
  } else if (x > xmax) {
    return xmax;
  } else {
    return x;
  }
}

prior_sample(iR,jR)  if (FILL_MODE_CLIP)    ip=clip(0,iR,HR−1);    jp=clip(0,jR,VR−1);    return prior_ref[ip][jp];   else     if ((iR<0) OR (iR>HR-1) OR (jR<0) OR (jR>VR-1))      return fill_value;     else       return prior_ref[jR][iR];

Next, a filter function that implements the bilinear interpolation is defined.  It is assumed that the bilinear interpolation coefficients, Øx and Øy, are quantized in the range 0,¼,P-1 (inclusive).

filterx0,y0,Æx,Æy

Finally, the method for warping the reference picture to generate a prediction for the current picture can be specified in terms of these functions.  The pixels of the prediction picture can be generated in a raster scan order.  It is assumed that the values vxL(j),vyL(j),vxR(j),and vyR(j)are already calculated and loaded into variables vxL,vyL,vxR, and vyR. The sample values of the pixels in the j th line of the resampled image is obtained by the following psuedo-code:

D = 32 * S * 2m  / P

axi = D * P + 2 * (vxR - vxL);

ayi = 2 * (vyR- vyL);

ax = vxL * 2m+1 + (vxR - vxL)+ D / 2;

ay = j * D * P + vyL * 2m+1 + (vyR- vyL)+ D / 2;

for (j  =  0; j  <  V; j++) {

IR = ax /// D;

JR = ay /// D;

iR = IR /// P;

jR = JR /// P;

Øx = IR - (iR  * P);

Øy = JR - (jR  * P);

new_ref[j][i] = filter(iR, jR, Øx, Øy);

ax += axi;

ay += ayi;

}

where all the variables used in this code are integer variables. According to the definition of the parameters, all the divisions and multiplications in this code can be replaced by binary shift operations.