| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q15-A-11 Filename: q15a11.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Gunnar Hellström, National Swedish Post and Telecom Authority Tel: +46 701 100 501 Fax +46 8 556 002 06 E-mail: gunnar.hellstrom@omnitor.se |
| Title: | Control requirements for sign language use of video telephony |
| Purpose: | Proposal |

_____________________________

Sign language users and people who need lip-reading supported speech perception expect videophones to carry their preferred way of communication.

On low bitrates, the parameters and options in the coders may need different settings when used for these purposes than what is preferred in other applications.

This is a proposal to document such needs and include mechanisms for convenient adjustment of videophone performance for sign language, lip-reading and general purpose usage.

The work should be conducted in connection to the creation of an appendix to H.263+ on sign language optimisation.

It is expected that the work will influence H.245 and H.263+ recommendations.

# Needs in brief

## Optimisation for temporal resolution

Both sign language and lip reading need good reproduction of movements.

- Usability for sign language is reported to start at 12 fps, with improvements at least up to 20.[Hellström, Delvert, Revelius 1996]

- Usability for lip reading is reported to start at 15 fps. [Frowein].

In some low bitrate environments such framerates are only achievable when sacrificing sharpness during movements. Some regular users may regard it too blurry and therefore want another optimisation.

It is the receiving user that experience the quality. Therefore the controlling mechanism should.

- Allow the receiver to control optimisation, influencing also the coding at the sending side. .

- Introduce such control over optimisation that can guarantee or nearly guarantee a minimum frame rate.

## Spatial resolution requirements

For spatial resolution it is reported that for person-to-person calls, the following is needed: [Kamata], [Frowein], [Hellström,Delvert]

- QCIF is useful.

- CIF is good and the increase from CIF is felt to give higher usability.

- SQCIF is too coarse for reliable perception.

- If different resolution is used in different parts, it is the hands and the face that needs highest resolution.

In order to optimise for temporal resolution, one usable approach that would suit the users is:

- Display in CIF as long as it is possible taking the temporal requirement in account.

- Drop down to QCIF when required.

- Allow further blur if needed during motion.

The control of this mode could possibly be combined with the temporal optimisation into a defined way of handling the TemporalSpatialTradeoff parameter in H.245. It should be controllable from the receiver.

## Blur during motion

VHS video is reported to be sufficient for good perception of sign language and lip reading. In VHS video, rapidly moving objects are often shown with considerable blur because shutter speeds are normally 1/50 to 1/60 of a second. This indicates that blur is acceptable on rapidly moving objects involved in big movements. No quantitative data is known on the acceptance levels.

The control of the allowable blur should be allowed the receiver. It may be combined in one parameter together with spatial and temporal tradeoff.

## Delay

Picture delay from sending camera to receiving screen is critical. Values over 1.2 seconds are easily felt hindering a good conversation. [Hellström and Delvert]. Values below 0.5 seconds are preferred.

There may be a need for the user to be able to control the delay. In certain networks a delaying buffer is used to cover variations in transfer rates. Users may be prepared to accept occational hickups in the transmission if they get a low mean delay. No values are known.

If possible, control over delay should be granted the receiver.

## Synchronism

For hearing supported lip-reading, the synchronism between sound and video is essential. Time differences of up to 100 ms is said to be acceptible.  For general videoconferencing it is sometimes claimed that it is better to transmit sound with no delay and accept bad synchronism.

It is important to still support a mode wich supports good synchronism.

If this is left open for control, the receiver should be allowed to control the synchronism in the session.

## H.263+ options

The numerous options in H.263+ have different importance for signing. In order to control them to an optimal combination, the control should be given to the receiver, but mostly used in some default combination not available for detailed user control.

## Channel allocation

Bandwith allocation may be possible in some networks and multiplexer environmnets. Control over the bandwidth means also control over perception quality.

## Stored Preferences

It is important for the users to be able to store preferences so that the likelihood of starrting a session in a mode that must be adjusted by modifying parameters should be made low.

# Bibliography

[Kamata] 1996. Sign language Conversation and Picture Quality of 64Kb/s ISDN Videophones.

[Frowein] 1990, Improved speech reception through Videotelephony experiments with the Hard of Hearing.

[Hellström, Delvert] 1996. Quality measurement of video communication for sign language.