| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q15-A-20 Filename: q15a20.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Jordi Ribas-Corbera and Shawmin Lei Sharp Labs of America jordi@sharplabs.com, shawmin@sharplabs.com |
| Title: | Rate Control for Low-Delay Video Communications |

_____________________________

# 1. Introduction:  rate control and low-delay

In video communications, the end-to-end delay for transmitting the video data needs to be very low, especially in two-way, real-time video applications such as video phone or video conferencing. Typically, the major component of the delay is the time that the video frames require to go through the fixed-rate communication channel, or the channel delay, as described in [5].

In practice, the video source is captured and encoded at a given frame rate F (in frames per second) and the coded bits are sent through a communication channel of  rate R (in bits per second). Hence, we can send up to B=R/F bits during each frame interval of 1/F seconds. If the first frame to be encoded occupies B’ bits, and B’ is larger than B, the (B’-B) bits that cannot be sent during its frame interval remain in the encoder buffer and are sent during the next interval(s). Clearly, the decoder will have to wait an additional (B’-B)/R seconds after that frame’s interval to receive all the bits. Conversely, if B’ is smaller than B the decoder will receive the bits for the frame within its frame interval, but (B-B’) bits will be wasted. Therefore, an efficient rate control method for low-delay communications should encode the video frames with approximately B bits [4].

In this contribution, we will show that the rate control technique in TMN7 does not achieve the target number of bits per frame B accurately, and consequently TMN7’s rate control often suffers from the following two important drawbacks:

- Frame skipping:

When several successive frames are encoded using more bits than the target B, the number of bits in the encoder buffer increases and hence the channel delay increases [4]. If the number of bits in the buffer exceeds a threshold M, the encoder skips encoding one or more video frames until the buffer level is below M. This ensures that the buffer delay for the next frame to be encoded will be below M/R seconds and protects the buffer from overflowing. Frame skipping is reasonable if a scene change occurs and one or several frames with many I macroblocks must be transmitted. But frame skipping should be avoided otherwise, because it produces lip-sync problems and undesirable jerkiness in the motion. In particular, skipping frames in sign-language and lip-reading video applications can drastically reduce the quality of the communication.

- Wasting channel bandwidth:

If several successive frames are encoded with fewer bits than the target B, the encoder buffer may soon run out of bits and there will be periods of time in which no bit will be sent through the channel. This problem is also referred to as buffer underflow. As a result, the video sequence will be encoded at a lower bit rate than that of the channel and the image quality of the encoded frames will decrease accordingly.

In this contribution, we present a new rate control technique for low-delay video communications and compare its performance to TMN7’s rate control. Our method is based on adjusting quantization step sizes down to macroblock level according to a Lagrangian optimized bit allocation strategy. This technique is computationally simple, since it only requires the values of the variances (or MADs) of the macroblocks and encodes them only once. Our rate control achieves the target number of bits per frame very accurately and, as a result, it rarely skips frames at the encoder and makes use of the complete bandwidth of the channel. In comparison to TMN7’s rate control, our method improves the temporal resolution by dropping up to 9 percent fewer frames and increases the image quality up to 1.2 dB in PSNR. Additionally, our rate control technique has the option of assigning a weight to each macroblock according to its quality requirement, which allows the designer to perform perceptual or object-based coding by simply adjusting these weights.

2. The Rate Control Technique

In this section, we describe our new rate control method for an H.263 codec. We use a frame-layer rate control technique very similar to TMN7’s rate control. Our main contribution is in the macroblock-layer, which we describe next, where we use a novel procedure for adapting the quantization parameter (QP) for the macroblocks within a frame.

- Macroblock-Layer Rate Control

Problem Description and Modeling

In H.263+ and other video coding standars, the current video frame to be encoded is decomposed into macroblocks of 16x16 pixels per block. The pixel values of each of the four 8x8 blocks in a macroblock are transformed into a set of coefficients using DCT and these coefficients are quantized and encoded. The encoder for a block in the ith macroblock of an image is illustrated in the diagram of Figure 1.

The quantization of the transformed coefficients is a key procedure since it determines the quality with which the image block will be encoded. As shown in Figure 1, the quantization of a block in the ith macroblock is controlled by only one parameter, QPi.

QPi

Coder

Block

Quantization

DCT Block

Transform

image

block

bits

Figure 1:  Encoding an image block

In H.263 codecs, QPi is known as the quantization parameter QP for the ith macroblock and its value corresponds to half the step size used for quantizing the transformed coefficients. The number of bits produced when encoding the ith macroblock, Bi , is a function of the value of the quantization parameter QPi and the statistics of the macroblock.

Let N be the number of macroblocks in a frame, the total number of bits B for encoding the image is:

B=B1+B2+B3+L+BN.		        (1)

We provide a method to choose the values of the QPi’s to minimize coding distortion under the constraint of (1), where the total number of bits B is selected in advance.

Encoder Model

We use the following model for the number of bits invested in the i-th macroblock :

Bi=A( Ksi2Qi2+C ),				       (2)

where Qi  is the quantization step for the macroblock (here Qi = 2 QP), A is the number of pixels in a macroblock (i.e., A=162 pixels), K and C are constants, and si is the empirical standard deviation of the luminance and chrominance values in the macroblock,

si=116⋅16+8⋅16j=116⋅16(PL,i(j)−Pi)2+j=18⋅16(PC,i(j)−Pi)2      (3)

where PL,i(j) is the value of the luminance for the j_th pixel in the i_th macroblock, PC,i (j) is the value of the j_th chrominance component in the macroblock, and Pi is the average of the luminance and chrominace values:

Pi=116⋅16+8⋅16j=116⋅16PL,i(j)+j=18⋅16PC,i(j).		        (4)

In order to reduce the computational complexity, the MAD of the luminance values for each macroblock can be used instead of the standard deviations. Our experimental results indicate that the rate control performance is only a little bit worse (the PSNR is typically within 0.2 dB’s) .

The model (2) was found using the low bit-rate model for scalar quantization and entropy coding derived in [3] and assuming that the pixel values in the macroblocks are approximately uncorrelated and. The value of K in (2) depends on the statistics of the macro-blocks. For example, it can be shown that if the DCT coefficients are Laplacian distributed, then K=e/ln2 [3]. The constant C in (2) models the bits per pixel to encode the motion vectors and the coder’s overhead (i.e., header and syntax information) for the frame.  In any case, since the values of K and C are often not known, we estimate them with our method.

Distortion Model

We consider the following distortion measure for the N encoded macroblocks:

D=1Ni=1Nαi2Qi212,			        (5)

where the αi’s are weights that can be chosen to incorporate the importance or weight of the macroblock distortions. For example, we could choose larger αi’s for macroblocks in which artifacts are more visible to the human eye or for macroblocks that belong to more important objects in the scene. If α1 =α2 = … =αN = 1, the distortion (5) is approximately the mean squared error (MSE) between the original and encoded macroblocks [6].

Optimization

Our method will select the quantization parameters,Q1,Q2,L,QN, that minimize the distortion in (5), subject to the constraint that the total number of bits must be equal to B in (1):

Q1,Q2,...,QN=argminQ1,Q2,...,QN:i=1NBi=B1Ni=1NQi212.                       (6)

We can formulate the constrained-optimization problem in (6) using the method of Lagrange multipliers:

Q1,Q2,...,QN=argminQ1,Q2,...,QN1Ni=1NQi212+λi=1NAKsi2Qi2+C,            (7)

withλ the Lagrange multiplier. By setting partial derivatives to zero, we obtain the optimized quantization parameters:

Qi=AKB−ANCsiαik=1Nαksk,         i= 1,2, …,N                      (8)

Moreover, if  i-1  macroblocks have already been quantized and encoded, the optimized quantization parameter for the ith macroblock is:

Qi=AKBi−ANiCsiαik=iNαksk,			        (9)

where Ni = N−i+1 is the number of image macroblocks that remain to be encoded and Bi is the number of bits available to encode them,

Bi=B−j=1i−1B©j=Bi−1−B©i−1,		                               (10)

where B’j  here is the actual number of bits used to encode the DCT coefficients of the j-th macroblock.   Equation (9) is the key for our new quantizer control tool.

Reducing the Quantization Overhead

The values of the optimized quantization steps Q1,Q2,L,QN need to be encoded and sent to the decoder. For example, in H.263+ the steps are differentially encoded using the parameter DQUANT, which is the difference between the current and the previous QP (recall that QP= Qi/2) in a raster-scan order.  If  DQUANT=0, nothing is sent, but if DQUANT is -1, -2, 1, or 2, a two-bit code is sent to indicate the change of quantizer. (DQUANT cannot be larger than 2 or smaller than -2.) Additionally, three bits must be sent to indicate that DQUANT is present. Hence, there is a five-bit penalty for changing the quantization step.

At high bit rates, the bits for the quantization overhead are negligible, and hence our previous optimization is effective. But at low or very-low bit rates, the penalty for changing the quantization step is significant. Unfortunately, an optimization procedure that would take the overhead into account is mathematically intractable. In previous work, some researchers have proposed using dynamic programming for that purpose [7], but these techniques have very high computational complexity and are not suitable for real-time video coding.

We propose a simple, heuristic method to reduce the quantization overhead at lower bit rates. Our method decreases the range of the values of the Q’s by choosing the following linear formula for the values of the distortion weights in (9):

αi=min2BAN(1−si)+si,1,

Observe that B/(AN) is the bit rate in bits per pixel for the current frame. If the bit rate is above 0.5 bits per pixel, the αi’s are all equal to 1 (they have no effect). At lower bit rates, theαi’s will linearly approach the respectivesi’s. In fact, if the bit rate is approximately 0, then αi»si and all the quantization steps are all approximately equal to:

Q1»Q2»...»QN»AKB−ANCk=1Nsk2,

and hence the DQUANT overhead would be approximately zero.

The Quantizer Control Tool

Here, we explain in detail our technique for selecting the value of QP for each macroblock within the frame.

Step 1.      Initialization.

Let   i=1         First macroblock.

B1=B    Number of available bits for encoding the frame.

N1=N   Number of macroblocks in a frame.

j = 0           Counter for number of macroblocks withKused in model estimation.

Let   S1=k=1Nαksk,

whereαk andsk are the distortion weight and standard deviation for the k_th macroblock as given in (11) and (3), respectively.

Finally, if this is the first frame to be encoded, set K1 = 0.5 and C 1= 0. Otherwise, set K1 and C1 to the values of K and C, respectively, obtained after encoding the previous frame.

Step 2.  Compute Optimized Q for i_th macroblock

If  L=(Bi−162NiC)≤0 or  Bi≤0 (running out of bits), set Qi=62.

Otherwise, compute:

Qi=162KLsiαiSi.

Step 3.  Find QP and Encode Macroblock

QP= round Qi2 to nearest integer in set 1,2, …, 31.

DQUANT = QP - QP_prev.

If  DQUANT > 2, set  DQUANT = 2.  If  DQUANT < -2,  set  DQUANT = -2;

Set  QP = QP_prev + DQUANT.

DCT encode macroblock with quantization parameter QP, and set QP_prev = QP.

Step 4.  Update Counters

Let Bi' be the number of bits used to encode the i_th macroblock, compute:

Bi+1=Bi−Bi',  S=Si−αisi,  and Ni+1=Ni−1 .

Step 5.  Update Model Parameters K  and C

The model parameters for the i-th macroblock are

K=BLC,i'(2 QP)2162si2 ,  and  C=Bi'−BLC,i'162,

where BLC,i'  is the number of bits spent for the luminance and chrominance of the macroblock.

Next, we measure the average of the K’s and C’s computed so far in the frame.

If  (K>0 and K≤plog2e),  set j= j+1 and compute  Kj=Kj−1(j−1)j+Kj.

Compute Ci=Ci−1(i−1)i+Ci.

Finally, the updates are a weighted average of the initial estimates and the current average:

K=Kj(i/M)+K1(M−i)/M,      C=Ci(i/M)+C1(M−i)/M.

Step  6.   If  i = N,  stop  (all macroblocks are encoded).

Otherwise,  let  i = i+1, and go to Step 2.

3.  Selection of the αi weights for perceptual or object-based coding

In the previous section, we explained that theαi values can be chosen to incorporate the importance or weight of macroblock distortions and we chose them to reduce the quantization overhead at lower bit rates. Observe from (9) that the quantization parameter Qi* decreases in macroblocks with larger αi’s and increases where the αi’s are smaller. This nice property can be used for perceptual or object-based coding. Here we describe two examples on how to choose the αi  weights for the latter purpose:

a) The interesting region:

We can assign a larger value of αi  to a selected region (e.g., a rectangular window).  Thus, the interesting region will be coded with better quality.

b)  Pyramidal weighting:

Knowing that people usually pay more attention to the central region of the picture, we can assign larger values ofαi to the regions near the center of the picture.   We can use a formula such as that of a pyramid to assign larger values ofαi to macroblocks closer to the center of the frame. Specifically, let BX and BY be the number of macroblocks along the horizontal and vertical coordinates, respectively. We can compute the weight for the i-th macroblock as follows:

αi=a11−bX−BX22BX1−bY−BY22BY+a2,

where (a1 + a2) and a2 are the height and offset of the pyramid, respectively, and bX  and bY are the horizontal and vertical position of the ith macroblock in the frame. For example, we can choose a1=15 and a2=1 such that theαi value of the center macroblock would be 16 times of those of the boundary macroblocks.

- Frame-Layer Rate Control

- The frame-layer rate control is very similar to that of TMN7’s rate control. We use the following formula for the target number of bits per frame:

- B=RF−D,				      (13)

- where R is the channel or target bit rate in bits per second, F is the frame rate in frames per second, and D is typically a small value that we define as follows:

- D=WF,W>0.1RFW−0.1RF,Otherwise,			      (14)

- with W the current number of bits in the encoder buffer. Observe that if W is larger than 10 percent of the average number of bits per frame, R/F, Ddecreases the target a little bit. Otherwise, D increases the target to prevent from buffer underflow. In TMN7’s rate control, the target number of bits per frame is R/F. But since there the number of bits used per frame fluctuates significantly, increasing and reducing the target with even a small D may have the encoder skip more frames and underflow the buffer, respectively.

As in TMN7, our method skips encoding the next frame/s when the number of bits in the buffer, W, is larger than a threshold M=R/F. This guarantees that the buffer delay for both methods will not be larger than M/R=1/F seconds, which is the time of a frame interval. (However, note that the channel delay, i.e., the sum of the throughput delay plus the buffer delay, can be larger than M/R [5]). In practice, this threshold M can be increased or decreased if we want to allow larger or smaller buffer delay, respectively, but it needs to be the same for both TMN7 and our method for a fair comparison of their performances.

4.  Experimental results

We used Telenor’s implementation of H.263 (version 2.0) with our new rate control scheme and TMN7’s rate control to encode frames of real video sequences. In the following experiments, we present results for the two methods in a wide variety of sequences and bit rates (22 tests) and compare their performance using the following criteria:

- Average bit rate

- Temporal resolution

- Image quality and average PSNR

- Channel delay

- Target accuracy

In Table 1, we describe the video data sources, target bit rates and target frame rates used in our 22 experiments.

Average Bit Rate

In Table 1, we also show the actual bit rates achieved by the two rate control strategies. In a few of the 22 cases, the bit rate for the two methods was very close, but in all of the others our new technique achieved a bit rate significantly closer to the target than that of TMN7’s. In fact, TMN7’s rate control wastes up to 20 per cent of the bits because the encoder often suffers from buffer underflow. (See “Channel Delay” below.)

Temporal Resolution

In Table 2, we show how many frames the two methods skipped in the steady state after the first P frame is encoded. The new rate control method skipped frames only in three of the tests. In all of them, the sequences were of CIF format, the bit rate (in bits per pixel) was very low, and there was always a significant scene change when the frames were skipped. Conversely, TMN7’s rate control skipped frames in 13 of of the 22 experiments, and it skipped up to 9 percent of the frames in cases where our method did not skip any.

In some cases, frame skipping can have drastic consequences. For example, in “silent” a young lady is talking using the sign-language and the skipped frames may contain an important sign for the conversation. In other cases, the frames skipped by TMN7’s rate control can generate lip-sync problems (e.g., in tests “mad24”, “mad48”, “fmn48”, “fmn68”, “fmn112”, etc.). Finally, a rough break of the motion continuity can be observed in the “coastguard” sequence (test “coast112_c”). Our new method also skipped a few frames there, but the motion in our encoded sequence was fairly smoother.

Image Quality

Table 2 also shows the averaged PSNR of the encoded video sequences (for the luminance component). When frames were skipped, the respective repeated frames were used in the computation. The new method always achieves similar or higher PSNR than TMN7’s. The gain in PSNR is often about 0.5 dB and its sometimes above 1 dB. We observed that the visual image quality was similar or slightly better. Remarkably, we did not observe a quality loss in the 13 video sequences where our rate control encoded more frames than TMN7’s.

Channel delay

Fig. 1-22 show the number of bits left in the buffer after encoding each frame. The channel delay for each frame is the number of bits in the buffer at that time instant divided by the channel rate (i.e., W/B)  [5]. The dotted line indicates the value of the threshold M (recall that M=B/F). When there are more than M bits in the buffer, both rate control schemes skip frames until the buffer fullness is below M. For example, observe that in Fig.3 the number of bits in the buffer of TMN7’s rate control reaches the dotted line 5 times, which indicates that 5 frames are skipped in the encoder.

Observe that the channel delay produced by our rate control method is always very low since there are typically few bits in the buffer. Also, our buffer rarely underflows. In the scenes where there is more motion and scene changes (e.g., in the tests with “foreman”, “mother and daughter” and “silent), our method keeps the channel delay significantly lower than TMN7’s. In some cases, TMN7 has very few or no bits left in the buffer and hence it generates a channel delay a little lower than ours (e.g., in the tests with “hall” and “container”). But observe that there the buffer underflows (i.e., it has no bits) almost constantly and hence this small reduction in delay comes at the expense of wasting bits, in other words, TMN7 rate control uses fewer bits than the target.

Target Accuracy

Finally, Fig. 23-29  show the number of bits per frame used by both rate control methods for a few of the tests. These results are somehow redundant since they can be guessed from the respective previous plots on the buffer occupancy, but they are also interesting to check. Observe that our new rate control achieves a nearly constant number of bits per frame, which cannot be achieved by the rate control of TMN7. In fact, this is the reason why our method can maintain a low buffer occupancy (low channel delay) without underflowing the buffer.

5. Conclusions

We have presented a new rate control method for an H.263 codec and have compared its performance to the rate control in TMN7. We demonstrated that our method skips many fewer frames than TMN7’s rate control. In fact, our technique only skipped frames when needed,  at drastic scene changes. Our method is also superior in meeting the target bit rate, encodes the sequences with higher PSNR, and achieves low channel delay without underflowing the buffer.

This new rate control technology, with some minor modifications, has also been presented and cross-checked by three independent companies in MPEG-4 [8-9]. The results presented here have been cross-checked bit-by-bit at the decoder’s site.

6. References

[1]  MPEG-4 video verification model V.5.0 ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Associated Audio MPEG96/ Maceio Nov. 96.

[2]   J. Ribas-Corbera and Shawmin Lei, “Contribution to Rate Control Q2 Experiment: A quantizer control tool for achieving target bit rates accurately, “ ISO/IEC Coding of Moving Pictures and Associated Audio JTC1/SC29/WG11 MPEG96/M1812, Sevilla, Spain, Feb. 97.

[3]   J. Ribas-Corbera and D. Neuhoff, “On the optimal motion vector accuracy for block-based motion-compensated video coders,” SPIE Digital Video Comp.: Alg. And Tech., pp. 302-314, San Jose, Jan-Feb. 96.

[4]   Michael Zeug, “Report of Ad Hoc Group on MPEG4 Low Delay Evaluations,” ISO/IEC Coding of Moving Pictures and Associated Audio JTC1/SC29/WG11 MPEG96/M1450, Maceio, Brazil, Nov. 96.

[5]   Richard Fryer, “Delay Computation for H.263 Novel Algorithms Proposals,” RJF/ITU/Del/970416, April 16, 97.

[6]   A. Gersho and R.M. Gray, “Vector Quantization and Signal Compression,” Kluwer Academic Publishers, 1992.

[7]  K. Ramchandran, A. Ortega, and M. Vetterli, “Bit allocation for dependent quantization with applications to multiresolution and MPEG video coders,” IEEE Trans. on Circuits and Systems for Video Technology, Vol. 6, N. 1, pp. 12-10, Feb. 96.

[8]  S. Ryoo, J. Shin, and Y. Seo “Results of Core Experiment Q2: Improved Rate Control (Part A, B, and C),” ISO/IEC Coding of Moving Pictures and Associated Audio JTC1/SC29/WG11 MPEG96/M1997, Bristol, UK, Apr. 97.

[9]  A. Vetro and H. Sun, “Core Experiment Q2: Verification of Quantizer Control Tool,” ISO/IEC Coding of Moving Pictures and Associated Audio JTC1/SC29/WG11 MPEG96/M2111, Bristol, UK, Apr. 97.

| Test Name | Video Sequence | Target Bit Rate (Kbps) | TMN7 rate control | New rate control |
| --- | --- | --- | --- | --- |
| coast112_c | “coastguard”, CIF, 15 fps | 112 | 112.2 | 112.1 |
| coast48 | “coastguard”,QCIF, 10 fps | 48 | 46.7 | 48.0 |
| cont10 | “container”, QCIF, 7.5 fps | 10 | 8.1 | 9.1 |
| cont24 | “container”, QCIF, 10 fps | 24 | 19.1 | 23.9 |
| cont48 | “container”, QCIF, 10 fps | 48 | 40.4 | 48.1 |
| fmn112 | “foreman”, QCIF, 10 fps | 112 | 111.0 | 112.1 |
| fmn112_c | “foreman”, CIF, 15 fps | 112 | 110.9 | 112.1 |
| fmn48 | “foreman”, QCIF, 10 fps | 48 | 47.8 | 48.0 |
| fmn64 | “foreman”, QCIF, 10 fps | 64 | 64.1 | 64.1 |
| hall10 | “hall”, QCIF, 7.5 fps | 10 | 9.6 | 9.9 |
| hall24 | “hall”, QCIF, 10 fps | 24 | 22.2 | 24.0 |
| hall48 | “hall”, QCIF, 10 fps | 48 | 44.5 | 48.0 |
| mad10 | “mother &”, QCIF, 7.5 fps | 10 | 10.0 | 10.0 |
| mad24 | “mother &”, QCIF, 10 fps | 24 | 24.1 | 24.0 |
| mad48 | “mother &”, QCIF, 10 fps | 48 | 48.3 | 48.0 |
| news112 | “news”, QCIF, 10 fps | 112 | 105.2 | 112.1 |
| news112_c | “news”, CIF, 15 fps | 112 | 108.1 | 112.1 |
| news48 | “news”, QCIF, 10 fps | 48 | 44.7 | 48.0 |
| news48_c | “news”, CIF, 7.5 fps | 48 | 45.2 | 48.1 |
| sil24 | “silent”, QCIF, 10 fps | 24 | 23.8 | 24.0 |
| sil48 | “silent”, QCIF, 10 fps | 48 | 47.7 | 48.0 |
| sil112 | “silent”, QCIF, 10 fps | 112 | 109.9 | 112.1 |

Table 1

| Test Name | TMN7 r.c Frames skipped | New r.c. Frames skipped | TMN7 r.c PSNR_Y | New r.c. PSNR_Y |
| --- | --- | --- | --- | --- |
| coast112_c | 9 | 5 | 26.39 | 26.36 |
| coast48 | 0 | 0 | 29.15 | 29.24 |
| cont10 | 1 | 0 | 28.94 | 29.32 |
| cont24 | 0 | 0 | 31.69 | 32.40 |
| cont48 | 0 | 0 | 34.79 | 35.53 |
| fmn112 | 5 | 0 | 34.57 | 35.10 |
| fmn112_c | 25 | 22 | 27.98 | 27.97 |
| fmn48 | 5 | 0 | 30.79 | 31.13 |
| fmn64 | 5 | 0 | 32.11 | 32.48 |
| hall10 | 0 | 0 | 30.61 | 30.67 |
| hall24 | 0 | 0 | 32.70 | 33.00 |
| hall48 | 0 | 0 | 36.14 | 36.76 |
| mad10 | 6 | 0 | 32.24 | 32.28 |
| mad24 | 8 | 0 | 34.83 | 35.02 |
| mad48 | 8 | 0 | 37.47 | 37.83 |
| news112 | 0 | 0 | 38.05 | 39.26 |
| news112_c | 0 | 0 | 33.64 | 34.12 |
| news48 | 0 | 0 | 33.12 | 34.14 |
| news48_c | 1 | 1 | 31.24 | 31.68 |
| sil24 | 4 | 0 | 30.84 | 30.89 |
| sil48 | 4 | 0 | 33.88 | 34.10 |
| sil112 | 5 | 0 | 38.38 | 39.30 |

Table 2

FIG. 1-29 will be incorporated in a later version of this paper. I apologize for the inconveniences.