| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q15-A-22 Filename: q15a22.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Deutsche Telekom |
| Title: | Verification of advanced deblocking filter |
| Purpose: | Information, Proposal |

_____________________________

A deblocking filter is already part of H.263+.    Recently, a proposal was made to slightly modify the filter to get visibly better performance.

# This paper tries to verify the claimed improvements. Basically it is only accompanying a tape demo for subjective assessment which compares:

Sequence without Filter

Sequence with current Filter

Sequence with new Filter

Sequence with Nokia‘s

Filter proposal

Nokias filter proposal is included, because at previous compares it seemed to perform best. It serves here as sort of upper bound.

Results:

Although the additional complexity is negligible a visible improvement of picture quality can be observed. In general the filter proposed by Nokia seems to provide still higher performance,    but the difference has become pretty small although its complexity is considerably .

Data rate and SNR are not completely unaffected. The data rate tends to grow marginally. In the worst case (Froeman CIF) 1.8%.    SNR-values can not be presented here, because unfortunately the values before filtering were measured.

| Seq. Name | Format | SNR current | SNR new | Rate current | Rate new |
| --- | --- | --- | --- | --- | --- |
| Foreman | QCIF |  |  | 11702 / 2479 | 11702 / 2536 |
| Foreman | CIF |  |  | 35682 / 7879 | 35682 / 8024 |
| Silent | QCIF |  |  | 9981 / 1229 | 9981 / 1265 |
| Silent | CIF |  |  | 34157 / 4012 | 34157 / 4005 |
| Coastg. | QCIF |  |  | 9451 / 2940 | 9451 / 2918 |
| Mother | QCIF |  |  | 8571 / 697 | 8571 / 699 |
| Hall monitor | QCIF |  |  | 12404 / 988 | 12404 / 979 |
|  | CIF |  |  | 42868 / 3063 | 42868 / 3079 |

Proposal:

It is therefore proposed to modify Annex-I of Rec-H.263 according to the new proposal.