| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q15-A-27 Filename: q15a27.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Japan Contact: Keiichi Hibi Sharp corporation Tel: +81 43 299 8613 Fax: +81 43 299 8608 Email: hibi@trl.mkhar.sharp.co.jp |
| Title: | Compatibility between ITU-T Recommendation and MPEG-4 |
| Purpose: | Discussion |

_____________________________

1. Introduction

@MPEG-4 and H.263+/H.263L have made its own progress without clear relationship for their harmonization, such as common text and/or joint meeting. Until now, only things they did are; experts were encouraged to attend both meetings, and they exchanged information of their study status each other.

Taking the standardization schedule into account, H.263+ has already determined and will be approved in next January with its final text in this September for voting. MPEG-4 CD, in which technical specifications will be finalized, will be ready in this November.

This document provide materials for discussion and lists up important discussion points on the relation between MPEG-4 and ITU-T to draw clear conclusion.

2. How to realize compatibility

- The target of MPEG-4 includes object-based coding, hybrid coding of natural and synthetic image (SNHC). To achieve these goals, MPEG-4 applies object-based syntax. Therefore, bit stream level compatibility with ITU-T H.263+ can not be expected.

- It is not a good solution that each standard define compatibility mode, because the compatibility mode will be a sub-set composed of limited coding tools and not have enough useful functionality.

- Implementation scheme of codec is changing from specific hardware to software-based. It is not difficult to realize dual mode codec according to this trend.

- It is a fact that an implementation becomes a little easier, if both standards are technically aligned by applying common tools.

3. Items for discussion

- Should H.263+ and MPEG-4 not be the same (or sub-set) ?

- Do the tools which each standard selected different solution for the same objective (ex. AC/DC prediction, deblocking filter, slice structure, scalability, etc.) cause any problem ?

- How is a relation between MPEG-4 and H.263L ? MPEG-4 has not been provided coding tools to meet the low delay requirement which ITU-T was requested.

- If MPEG-4 standard will apply different tools from ITU-T, will it be an independent standard of what ITU-T studies ?

4. Conclusion

This document have discussed harmonization between MPEG-4 and ITU-T providing important items for discussion. The bit stream compatibility between MPEG-4 and ITU-T can not already be expected. However, it needs clear answer for the technically aligned standards whether MPEG-4 and H.263+/H.263+L should be the same.

END