| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group Portland, 24-27 June 1997 | Document Q15-A-32 Filename: q15a32.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Japan |
| Title: | Results of performance verification of Annex Q by the two independently implemented simulations |
| Purpose | Information |

# Introduction

This contribution reports the two independent simulation results concerning on Annex Q “Reduced Resolution Update Mode” of  H.263+ Draft[1].

# Simulation Conditions

## Encoder Option (except Annex Q)

- Advanced Prediction		: On

- UMV			: On

- Other Options		: Off

## Other Conditions

The other detailed conditions are described in the document which was distributed via e-mail reflector on 97/02/05.   This is also attached in this  document as Appendix.

## Summary of Simulation Conditions

Table  1 Summary of Simulation Conditions

| Sequence | Bitrate | Rate Control | coding method |
| --- | --- | --- | --- |
| Foreman | 112kbps | TMN6 and TMN6 | without Annex Q |
| (1-300) |  | without FRR | with Annex Q |
| Mother &Daughter | 40kbps | TMN6 and TMN6 | without Annex Q |
| (600-899) |  | without FRR | with Annex Q |
| Silent | 40kbps | TMN6 and TMN6 | without Annex Q |
| (1-300) |  | without FRR | with Annex Q |

As for the rate control, we applied two methods, TMN6 and TMN6 without Frame Rate Regulations(FRR).

# Results

Table 2 to Table 7 show the simulation results obtained by independent implementers A and B.  These tables include the average Bits/Frame, the average PSNR of each component, and the number of the coded frame.

## TMN6 Rate control

Table  2  Foreman

| Implement | Coding mode | Bits / Frame | PSNR Y | PSNR Cb | PSNR Cr | Coded Frame |
| --- | --- | --- | --- | --- | --- | --- |
| A | with Annex Q | 17056.00 | 31.5305 | 37.7274 | 38.6227 | 68 |
|  | without Annex Q | 18589.42 | 31.8245 | 37.7444 | 38.7732 | 62 |
| B | with Annex Q | 16810.90 | 31.5333 | 37.7098 | 38.6457 | 69 |
|  | without Annex Q | 18392.89 | 31.7899 | 37.7385 | 38.7334 | 63 |

Table  3  Mother & Daughter

| Implement | Coding mode | Bits / Frame | PSNR Y | PSNR Cb | PSNR Cr | Coded Frame |
| --- | --- | --- | --- | --- | --- | --- |
| A | with Annex Q | 6447.41 | 32.3966 | 38.4491 | 38.2178 | 68 |
|  | without Annex Q | 7562.07 | 33.0136 | 38.6256 | 38.3802 | 58 |
| B | with Annex Q | 6288.00 | 32.2973 | 38.2480 | 37.9531 | 69 |
|  | without Annex Q | 7544.69 | 33.0246 | 38.6421 | 38.3413 | 58 |

Table  4  Silent

| Implement | Coding mode | Bits / Frame | PSNR Y | PSNR Cb | PSNR Cr | Coded Frame |
| --- | --- | --- | --- | --- | --- | --- |
| A | with Annex Q | 6270.42 | 30.624 | 36.367 | 37.7887 | 71 |
|  | without Annex Q | 7921.57 | 30.970 | 36.2758 | 37.7198 | 56 |
| B | with Annex Q | 6295.66 | 30.6189 | 36.1927 | 37.6160 | 70 |
|  | without Annex Q | 7902.43 | 30.9688 | 36.3222 | 37.6771 | 56 |

## TMN6 Rate control without FRR

Table 5  Foreman

| Implement | Coding mode | Bits / Frame | PSNR Y | PSNR Cb | PSNR Cr | Coded Frame |
| --- | --- | --- | --- | --- | --- | --- |
| A | with Annex Q | 12242.55 | 30.8223 | 37.0960 | 37.9696 | 94 |
|  | without Annex Q | 12992.27 | 30.7978 | 37.0669 | 37.9160 | 89 |
| B | with Annex Q | 12154.27 | 30.8467 | 37.0918 | 37.8621 | 95 |
|  | without Annex Q | 13009.26 | 30.842 | 37.0626 | 37.8901 | 89 |

Table  6  Mother & Daughter

| Implement | Coding mode | Bits / Frame | PSNR Y | PSNR Cb | PSNR Cr | Coded Frame |
| --- | --- | --- | --- | --- | --- | --- |
| A | with Annex Q | 4768.53 | 32.0066 | 38.0883 | 37.8562 | 91 |
|  | without Annex Q | 4904.36 | 31.9707 | 37.7752 | 37.6568 | 89 |
| B | with Annex Q | 4653.02 | 32.0539 | 37.8390 | 37.6352 | 94 |
|  | without Annex Q | 4897.71 | 32.0641 | 37.7694 | 37.6198 | 89 |

Table  7  Silent

| Implement | Coding mode | Bits / Frame | PSNR Y | PSNR Cb | PSNR Cr | Coded Frame |
| --- | --- | --- | --- | --- | --- | --- |
| A | with Annex Q | 4792.78 | 30.2535 | 35.8926 | 37.3542 | 92 |
|  | without Annex Q | 4966.29 | 30.0348 | 35.4247 | 37.0018 | 89 |
| B | with Annex Q | 4739.53 | 30.2483 | 35.6802 | 37.1943 | 93 |
|  | without Annex Q | 4900.80 | 30.1256 | 35.5351 | 37.0183 | 90 |

# Discussion

As shown in Table 2 to 7 , the two independently implemented simulation results showed good consistency.  As for the subjective assessment of the decoded sequences,  the following coherent observations are also reported from both implementers.

- With TMN6 rate control, coded frame rate are clearly  increased by Annex Q.  Though the area of moving object, such as the face of  Mother and Silent, becomes somewhat blurred compared to the default mode(without Annex Q), this is observed only for the part of sequence in which smoothness of the motion is improved and even in that part of sequence, the high resolution of static background area is maintained.

- In the case of TMN6 without FRR control, block artifact of the default mode is annoying.  We can also observe the blurring in Annex Q, however, the total subjective quality with Annex Q is better than the default mode.   The gain in the coding frame rate by Annex Q is smaller than the TMN6 case.  For some sequence like Silent, this improvement in motion smoothness is difficult to recognize.  But the overall subjective quality with Annex Q is better than the default mode for these sequences because block artifact is not much annoying for Annex Q.

# Conclusion

The performance of Annex Q “Reduced Resolution Update Mode” of H.263+ draft was evaluated by the two independently  implemented simulations.  The results of the two simulations showed good consistency.  The simulation results also confirmed the improvement of total subjective quality by the Annex Q for two kind of rate control cases.

References

[1] H.263+ Draft / Annex Q

[2] “Video Coder Test Model, TMN6”, LBC-96-141

Appendix

(distributed by e-mail reflector on 97/02/05)

Simulation Model for Reduced-Resolution Update mode

# Motion estimation and mode selection

In Reduced-Resolution Update mode, motion estimation is performed on the luminance 32x32 macroblock instead of 16x16 macroblock. SAD (Sum of Absolute Difference) is used as error measure.  In this mode, each component of the macroblock motion vector or four motion vectors is restricted to be half-integer or zero value in order to widen the search range with the same MVD table.

## Motion estimation in baseline mode (no options)

### Integer pixel motion estimation

The search is made with integer pixel displacement in the Y component.  The comparisons are made between the incoming macroblock and the displaced macroblock in the previous reconstructed  picture.  A full search is used, and the search area is up to ±30 pixels in horizontal and vertical direction around the original macro block position.

SAD(x,y)=i=1,j=132,32original−decodedprevious,x,y=
          up +- 30

For the zero vector,  SAD(0,0) is reduced by 400 to favor the zero vector when there is no significant difference.

SAD(0,0)=SAD(0,0)−400

The (x,y) pair resulting in the lowest SAD is chosen as the integer pixel motion vector, MV0.  The corresponding SAD is SAD(x,y).

### INTRA/INTER mode decision

After the integer pixel motion estimation the coder makes a decision on whether to use INTRA or INTER prediction in the coding.  The following parameters are calculated to make the INTRA/INTER decision:

MBmean=(i=1,j=132,32original)1024

A=i=1,j=132,32original−MBmean

INTRA mode is chosen if:   A<(SAD(x,y)−2000)

Notice that if SAD(0,0) is used, this is the value that is already reduced by 400 above.

If INTRA mode is chosen, no further operations are necessary for the motion search. If INTER mode is chosen the motion search continues with half-pixel search around the MV0 position.

### Half-pixel search

The half-pixel search is done using the previous reconstructed frame. The search is performed on the Y-component of the macroblock.  The search area is ±1 half-pixel around the 32x32 target matrix pointed to by MV0, complying the condition that each component of the candidate vector for the half-pixel search is half-integer or zero value.  For the zero vector (0,0), SAD(0,0) is reduced by 400 as for the integer search.

The half pixel values are calculated as described in ITU-T Recommendation H.263, Section 6.1.2.

The vector resulting in the best match during the half-pixel search is named MV. MV consists of horizontal and vertical components (MVx, MVy), both measured in half pixel units.

## Motion estimation in advanced prediction (AP) mode

This section applies only if advanced prediction mode is selected.

### Integer pixel motion estimation

No integer pixel search is performed for the 16x16 block motion vectors. The 16x16 motion vectors are assigned the same value as the 32x32 MB integer motion vector.

### Half-pixel search

The half-pixel search is performed for each of the blocks around the 16x16 integer vector.  The search area is ±2.5 pixel around the 16x16 integer vector of the corresponding block, complying the condition that each component of the candidate vector for the half-pixel search is half-integer or zero value and within [-31.5, 30.5].

### One vs. Four MV Decision in AP

This section applies only if advanced prediction mode is selected.

SAD for the best half pixel 32x32 MB vector (including subtraction of 400 if the vector is (0,0)):

SAD32(x,y)

SAD for the whole macroblock for the best half pixel 16x16 block vectors:

SAD4x16=14SAD16(x,y)

The following rule applies:

If:	SAD4x16<SAD32−800,	choose 16x16 block prediction

otherwise:				choose 32x32 MB prediction

## Motion estimation in the unrestricted motion vector (UMV) mode

This section applies only if the extended motion vector range in the UMV mode is selected.

### Search window limitation

Since the window with legal motion vectors in this mode is centered around the motion vector predictor for the current macroblock, some restrictions on the integer motion vector search is applied, to make sure the motion vectors found will be transmittable.

With these restrictions, both the 32x32 MB vector and the 16x16 block vectors found with the procedure described below, will be transmittable, no matter what the actual half-pixel accuracy motion vector predictor for the macroblock, or each of the four blocks, turns out to be.

### Integer pixel search

First, the motion vector predictor for the 32x32 MB vector based on integer motion vectors only, is found. The 32x32 MB search is then centered around this predictor, with a somewhat limited search window. The 32x32 MB search window is limited to the range 29 - (2*16x16_block_search_window+1).   Since in this model the 16x16_block search window is 2.5, the default search window in the UMV mode turns out to be ±23 integer positions. The 16x16_block searches are centered around the best 32x32 MB vector.

### Half-pixel search

Half-pixel searches are performed as in the other modes, around the best 32x32 MB vector.    The search area is ±2.5 pixel around the 16x16 integer vector of the corresponding block, complying the condition that each component of the candidate vector for the half-pixel search is half-integer or zero value.

# Down-sampling of the prediction error

After motion compensation on 16*16 block basis, the 16*16 prediction error block is down-sampled to the 8*8 reduced-resolution prediction error block.  In order to realize a simple implementation, filtering is closed within a block which enables to perform an individual up-sampling on block basis.  Fig. 1 shows the positioning of samples.  The down-sampling procedure for the luminance and chrominance pixels is defined Fig.2.  Filtering is performed regardless of the block boundary.   “/” in Fig.2 indicates division by truncation.

Fig.  1

Positioning of samples in 8*8 reduced-resolution prediction error block and

16*16 prediction error block

Fig.  2

Creation of reduced-resolution prediction error

for pixels inside block

# Transform and Quantization

A separable 2-dimensional Discrete Cosine Transform (DCT) is applied to the 8*8 reduced-resolution prediction error block in the same way as described in TMN6/7.  Then Quantization is performed in the same way as described in TMN6/7.

# Switching

(1) Resolution Decision Algorithm

A simple decision algorithm is used based on the following two parameters which are already used in TMN6 rate control algorithm.

QPi−1 :  the mean QP of previous encoded frame

Bi−1    :  the number of bits used in the previous encoded frame.

Assuming that the relation between QPi−1and Bi−1 is close to inverse proportion, the product of QPi−1 and Bi−1 can be regarded as an index of the approximate complexity of the coded frame.

In the case the current frame is default mode, switching to reduced-resolution update mode is done if the product of QPi−1 and Bi−1 is larger than the threshold TH1. Then, QPi−1 is divided by the constant value C1 and this QPi−1 is used to the rate control of the current frame.

In the case the current frame is reduced-resolution update, switching to default mode is done if this product of QPi−1 and Bi−1 is smaller than the threshold TH2.  Then, QPi−1 is multiplied by the constant value C2 and this QPi−1 is used to the rate control of the current frame.

- from default mode to reduced-resolution update mode

if(QPi−1* Bi−1 > TH1){

Switch to

reduced-resolution update mode;

QPi−1 = QPi−1 / C1;

}

- from reduced-resolution update mode to default mode

- if(QPi−1* Bi−1 < TH2){

Switch to default mode;

QPi−1 = QPi−1 * C2;

}

TH1 is determined in the following equation, where QP1 and FR1 represents  the lowest subjective quality which we allow to encode in default mode.

TH1 = QP1 * (Target_Bitrate / FR1)

In the same way, TH2 is determined in the following equation,  where QP2 and FR2 represents the highest subjective quality which we allow to encode in reduced-resolution update mode.

TH2 = QP2 * (Target_Bitrate / FR2)

If Source Format indicates CIF, then QP1 = 16, FR1 = 6, C1 = 2.5, QP2 = 7, FR2 = 8, C2 = 2.5, respectively.

- Restriction of DCT coefficients in switching from reduced-resolution to normal-resolution

Once the reduced-resolution mode is selected, the detail of the image is likely to be lost.  If the mode goes back to the default mode again, the detail of image must be reproduced, which consumes a large amount of bits. This sudden increase of coding bits often causes an unintentional frame skips.  Furthermore,  because the resolution-decision algorithm described above uses the product of mean QP and the amounts of bits, this sudden increase of the bits cause to switch back to reduced-resolution update mode, and the oscillation between both modes often occurs.  In order to avoid this degradation, the restriction of DCT coefficients to be sent is introduced to the several frames after switching from reduced-resolution update mode to default mode.  In the first frame after switching to the default mode, the coefficients only within 4x4 low frequency can be sent,  then in the same way, 5x5  in the second, 6x6 in the third, and 7x7 in the forth.  This “smooth-landing” algorithm can suppress the unintentional frame skip and the oscillation of the modes effectively.

Contact Person:

Akira NAKAGAWA			Hiroyuki KATATA

FUJITSU LABORATORIES LTD.		SHARP CORPORATION

Phone  : +81 44 754 2679			Phone  :  +81 43 229 8540

Fax      : +81 44 754 2347			Fax      :  +81 43 299 8509

E-mail : anaka@flab.fujitsu.co.jp		E-Mail : katata@imgsl.mkhar.sharp.co.jp