| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q15-A-40 Filename: q15a40.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Telenor Satellite services |
| Title: | A modified ‘Altered quantization step size for chroma coefficients’ (Annex T) |
| Purpose: | Discussion/Proposal |

_____________________________

Introduction

Altered quantization step size for chrominance coefficients was introduced in Nice.  In doc. q15a03 Gary mentioned some items that need verification.  One of them:

‘half-step-size chroma quantization in Annex T (specifically for chroma-rich sequences such as Paris, according to one verbal comment to the editor)’.

I agree that verification is needed.  I have made some quick tests.  As expected, the subjective benefit from reduced QUANT_C for chroma is largest for high QUANT (say 24-31).  For such QUANT values the added bitrate is moderate even for ‘chroma-rich’ sequences like PARIS and MISS AMERICA.  For lower QUANT the subjective benefit is smaller and for a sequence like MISS AMERICA the increase in bitrate may reach 35%.

A modified relation between QUANT and QUANT_C

Instead of defining  QUANT_C=QUANT//2,  I therefore tried with the following relation between QUANT and QUANT_C:

| QUANT | QUANT_C | QUANT | QUANT_C | QUANT | QUANT_C |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 12 | 10 | 23 | 14 |
| 2 | 2 | 13 | 10 | 24 | 14 |
| 3 | 3 | 14 | 11 | 25 | 14 |
| 4 | 4 | 15 | 11 | 26 | 14 |
| 5 | 5 | 16 | 12 | 27 | 15 |
| 6 | 6 | 17 | 12 | 28 | 15 |
| 7 | 6 | 18 | 12 | 29 | 15 |
| 8 | 7 | 19 | 13 | 30 | 15 |
| 9 | 8 | 20 | 13 | 31 | 15 |
| 10 | 9 | 21 | 13 |  |  |
| 11 | 9 | 22 | 14 |  |  |

Some simulation result

I used the QCIF sequence COASTGUARD due to its low chroma content.  The QCIF sequence MISS AMERICA and the CIF sequence PARIS represent ‘rich chroma’ sequences.  Three values of QUANT: 7,14,28 were used.  The table below summarize the increased chroma SNR and bitrate with different choices of QUANT_C.  The numbers are for INTER frames, but the tendency for INTRA frames seemed to be similar.

|  | QUANT=7 |  |  |  | QUANT=14 |  |  |  |  |  | QUANT=28 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QUANT_C=7 |  | QUANT_C=4 |  | QUANT_C=14 |  | QUANT_C=11 |  | QUANT_C=7 |  | QUANT_C=28 |  | QUANT_C=15 |  |
| Sequence | Dsnrc | %bit | Dsnrc | %bit | Dsnrc | %bit | Dsnrc | %bit | Dsnrc | %bit | Dsnrc | %bit | Dsnrc | %bit |
| Coastguard | 0 | 0 | +1.5 | +1.4 | 0 | 0 | +0.8 | +0.2 | +1.8 | +1.4 | 0 | 0 | +0.6 | +0.3 |
| Miss America | 0 | 0 | +1.7 | +37 | 0 | 0 | +0.8 | +5.6 | +2.0 | +36 | 0 | 0 | +1.5 | +8.0 |
| Paris | 0 | 0 | +2.3 | +8.7 | 0 | 0 | +0.9 | +2.4 | +2.8 | +11 | 0 | 0 | +1.8 | +7.0 |

Subjective assessment of the sequences

COASTGUARD

The main subjective improvement with lower QUANT_C is for QUANT=28.  The increase in bitrate is small.  For this type of sequence the best solution is: QUANT_C=QUANT//2 (as in Annex T).  The proposed modified relation between QUANT and QUANT_C is a good compromise.

Miss America

The chroma is ‘over saturated’.  This sequence is therefore not typical.  There was very little difference in subjective quality with the different choices of QUANT_C.  On the other hand the increase in bitrate is large for QUANT_C=QUANT//2.  The best choice for this sequence is QUANT_C=QUANT. ).  The proposed modified relation between QUANT and QUANT_C is a good compromise.

Paris

The main subjective improvement with lower QUANT_C is for QUANT=28. The proposed modified relation between QUANT and QUANT_C is the best solution.

Conclusion

From this limited test it seems that the proposed modified relation between QUANT and QUANT_C is an overall better solution than the present definition of: QUANT_C=QUANT//2  in Annex T.