| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q15-A-42 Filename: q15a42.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |

ITU – Telecommunications Standardization Sector			Document TD-37_FINAL

STUDY GROUP 16 Contribution

Experts Group for Video Coding and Systems in			June 10-13, 1997
ATM and Other Network Environments				Hertzliya Beach

Title:		Liaison to Q15/16 re Profiles for H.263+ modes in H.323 systems

Source:	 	D. Skran, Rapporteur for Q13/16

Purpose: 	A Contribution to Q15/16 from Q13/16.  Not a Formal Liaison.

Contact:	Stephan Wenger			Voice:	+49 30 31473160

Secr. FR 6-3				Fax:	+49 30 31425156

TU Berlin				Email:	stewe@cs.tu-berlin.de

Franklinstr. 28-29

D-10587 Berlin, Germany

Q13/16 feels that the large number of possible permutations of H.263+ options (approximately 2^15, or 32678 possible mode combinations) will make selection of a common set of enhanced modes beyond the H.263 baseline, difficult to agree upon between H.323 systems.

Therefore, we would like to request that Q15/16 provide a small set of H.263+ profiles which are each optimized for various specified applications.  We suggest that these should be based on the following operational parameters (this is only a partial list, there may be other relevant parameters we have not considered here):

- Bitrates: 10 kbps to 10 Mbps

- Frame/Block/Macroblock/Slice/GOB loss rate: 0 to 0.30 (possibly higher)

- With and without backchannel available

- Computational complexity: Low to High (encoder and decoder separately)

- Acceptable latency and jitter (low to unlimited)

We realize that beyond any set of profiles, individual H.263+ modes can additionally be turned on or off, but the profiles are needed to simplify mode negotiation.  It is important for Q13/16 to understand the design criteria that went into H.263+, to enable optimal use of H.263+ in systems.

Also, we would like some guidance on which H.263+ modes should and should not be used at the same time.

[end]