| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q15-A-47 Filename: q15a47.doc |
| --- | --- |

| Question: | Q.15/16 |  |
| --- | --- | --- |
| Source: | Thomas Gardos Intel Corporation, M/S JF2-78 2111 NE 25th Ave. Hillsboro, OR 97124 | e-mail: trgardos@ibeam.jf.intel.com Tel: +1 503 264-6459 Fax: +1 503 264-9372 |
| Title: | A proposal for scalability in one dimension in H.263+ |  |
| Purpose: | Proposal |  |

_____________________________

Summary

It is possible to increase the functionality of Annex O (Temporal, SNR, and Spatial Scalability Mode) with no change to the H.263+ syntax. The modification is to allow subsets of spatial scalability by interpolation in one dimension only. This allows finer granularity in the creation of enhancement layers, and also permits more efficient implementations of scalability. The method by which this can be achieved is described herein.

Introduction

Currently, Annex O of H.263+ specifies that a particular base layer frame can be enhanced by either an SNR enhancement frame (same dimensions, same pixel aspect ratio) or a spatial enhancement frame (2X the size in both dimensions, same pixel aspect ratio).  As it turns out there is nothing in the syntax that prevents the use of an intermediate enhancement layer, one that is twice the size in one dimension and also twice the pixel aspect ratio. Consider the following example.

Example:

Say we have a base layer of video with picture size 176x144 pixels (QCIF) and, for sake of simplicity let’s assume that the pixel aspect ratio is indicated to be 1:1. An enhancement layer can be created exactly as for SNR or spatial enhancement, except we would now allow the enhancement layer picture size to be 2X in neither, one or both directions. So for this example, we assume that there is spatial enhancement in the horizontal direction only. In that case the enhancement layer would have ELNUM = 1, RLNUM = 0, a picture size of 352x144 (indicated by custom picture format), and a pixel aspect ratio of 1:2 (needs to be stretched vertically by 2 before display).

Note that another 1-D spatial enhancement layer can be added, with interpolation in the vertical direction this time. The result is that we can get two layers of spatial enhancement between, say, QCIF and CIF video, rather than only one layer as currently defined.

The only item remaining to be defined are the 1-D interpolation filters. They are defined next.

1-D Interpolation Filters

To define the 1-D filters, we first recast the 2-D interpolation filtering operation into the equivalent multirate operation. The picture is first up-sampled by two in both directions. Put another way, zero-valued pixels are inserted between every pixel horizontally, and then zero-valued pixels are subsequently inserted between every pixel vertically. The up-sampled frame is then filtered with the 2-D filter defined as

f(m,n)=1331399339931331.

The origin of the filter is indicated by the smaller brackets. The interpolation process is not exactly linear, since the last step in calculating each filtered pixel value is to add 8 and divide by 16 and truncate.

This filter, f(m,n), is separable, and the 1-D version is defined as

f1(n)=1331.

Again, the origin of the filter is indicated by the inner square brackets. As before there is an additional step of adding 4 to the filtered value, and then dividing by 8 and truncating.

The 1-D interpolation at picture boundaries can be similarly defined.

Conclusion

As indicated in this document, a slightly more flexible method of scalability can be defined with no change to the H.263+ syntax and only small changes to the Draft Recommendation. Spatial scalability in one dimension can be viewed as a simple case of the 2-D spatial scalability currently defined in the Draft Recommendation. Hence, it is our recommendation that this feature, which is almost supported already be explicitly permitted.