| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Portland, 24-27 June 1997 | Document Q15-A-54 Filename: q15a54.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Kevin O’Connell (Motorola) Samson Chueng (VTel) Toshihisa Nakai (Oki) Rickard Sjoberg (Ericsson Telecom) |
| Title: | Some Start Code Emulation Problems in H.263+ |
| Purpose: | Proposal |

_____________________________

During the course of the Portland SG16-Q.15 meeting, several potential start code emulations have been noted.  This contribution proposes solutions to these emulation problems.

# 1.	Picture Header CPFMT+EPAR

The current H.263+ syntax allows CPFMT Bits 15-23 = 0000 0000 0 followed by EPAR Bits 1-8 = 0000 0001, resulting in 16 consecutive zeros and thus a start code emulation.

Solution:

Modify section 5.1.5, by replacing the definition of CPFMT Bits 15-23, with:

Bit 15-23 Picture Height Indication: Range [1,...,288];  Number of lines = PHI*4.

# 2.	Annex K MBA + SQUANT

The current H.263+ Annex K syntax allows MBA field width up to 14 bits, e.g. a MBA = 1000 0000 0000 00 when the maximum custom picture format is in use, followed by SQUANT = 0000 1, resulting in 17 consecutive zeros and thus a start code emulation.

Solution:

- Modify FIGURE K.1/H.263, by renaming EPB to EPB1 and inserting EPB2 between the MBA and SQUANT.

- Modify the title of K.5 to “Emulation Prevention Bit 1 (EPB1) (1 bit)”.

- Add a section between K.6 and K.7:

- K.#	Emulation Prevention Bit 2 (EPB2) (1 bit)

- If the MBA Field Width is greater than 12 bits, then a single bit whose value is always 1 is included to prevent PSC emulation.

- Annex N must be modified in a similar way.

# 3.	Picture Header + MCBPC Stuffing

When Annex M and Custom Source Format are both indicated in the picture header, the current H.263+ syntax allows TRB = 1000 0, DBQUANT = 00, PEI = 0, COD = 0, and MBA = 0000 0000 1 (Stuffing), resulting in 16 consecutive zeros and thus a start code emulation.

Solution:

Add a sentence to section 5.3.2:

If Improved PB-frame is indicated by MPPTYPE Bits 1-3 and Custom Source Format is indicated OPPTYPE Bits 1-3, then MBA shall not indicate Stuffing before the first macroblock of the picture.

Alternatively, this sentence could read:

If PLUSPTYPE is present, then MBA shall not indicate Stuffing before the first macroblock of the picture.

# 4.	Proposed MCBPC Extension for INTER4V+Q

A proposal to extend the MCBPC table to allow INTER4V+Q is made in q15a23.  The proposal includes some codes that began with 10 consecutive zeros, which when preceded by 6 zeros (which is possible in various ways), would result in a start code emulation.  By modifying the proposal, start code emulation can be avoided.

Solution:

- Add four entries in TABLE 8/H.263:

| Index | Mbtype | CBPC | bits | Code |
| --- | --- | --- | --- | --- |
| 21 | 5 | 00 | 11 | 0000 0000 010 |
| 22 | 5 | 01 | 12 | 0000 0000 0110 |
| 23 | 5 | 10 | 13 | 0000 0000 0111 0 |
| 24 | 5 | 11 | 13 | 0000 0000 0111 1 |

If an open code table is desired, the code length for index 22 or 24 could be increased by 1 bit.

- Add a sentence to section 5.3.2:

If PLUSPTYPE is present, then TABLE 8 indices 21-24 are also allowed, except in the first macroblock of the picture.

- Modify section T.4 as follows:

Change “cyclicly rotating them to the right by 4 bit positions” to “cyclicly rotating them to the right by 5 bit positions”.

Modify FIGURE T.1/H.263 to reflect the change from 4-bit to 5-bit cyclic rotation, i.e. replace the figure by:

Bits of LEVEL field

| b10 | b9 | b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Bits of EXTENDED-LEVEL field

| b4 | b3 | b2 | b1 | b0 | b10 | b9 | b8 | b7 | b6 | b5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FIGURE T.1/H.263: Cyclic rotation of coefficient representation