| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fifth Meeting: Whistler, British Columbia, CA, 21-24 July, 1998 | Document Q15-E-04 Filename: q15e04.doc Generated: 10 July ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Thomas R. Gardos, Gim L. Deisher Intel Corporation 5200 NE Elam Young Parkway Hillsboro, OR 97124 | Tel: Fax: Email: | (503) 264-6459 (503) 264-9372 Thomas.R.Gardos@intel.com |
| Title: | Report of the Ad Hoc Committee on H.263+ Packetization |  |  |
| Purpose: | Information |  |  |

_____________________________

The following is a brief summary of events pertaining to the H.263+ RTP payload format since the report to the Tampere Q.15 meeting 0.

# H.263+ RTP Payload Format

As reported in 0, the “AVT Working Group last call for comments” period ended for the payload format on May 5, 1998.

On May 7, 1998, a third version of the payload draft2 was distributed on the IETF AVT and Advanced Video Reflectors.  A brief summary of changes from version 21 is as follows:

- Extended the first paragraph to describe the relationship of this payload draft and RFC2190. (Section 1)

- Mentioned P-picture temporal scalability (sections 1 & 4.2).

- Made a correction in the RTP timestamp description. (Section 2.1)

- Clarified GOB boundary packetization. (Section 2.2)

- Adjusted diagrams.  No change in payload format. (Sections 5.1.1, 5.1.2, 5.1.3, 5.2)

- Added/deleted references. (Section 8)

- A few other minor editorial changes.

# On July 10, 1998 the chair of the AVT Working Group requested that IESG publish 2 as a Proposed Standard.

# H.263+ RTP Interoperability

Preliminary H.263+ RTP payload interoperability tests were done in late May between UBC/TU-Berlin and Intel.  Not all the functionality was tested: the V bit and PLEN field in the payload header were always set to zero and none of the H.263+ optional modes were used.  Nonetheless, real-time video streaming sessions (QCIF video) over the Intranet were successfully established.

# References

- Report of the Ad Hoc Committee on H.263+ Packetization, q15d05.doc

- RTP Payload Format for the 1998 Version of ITU-T Rec. H.263 Video (H.263+), draft-ietf-avt-rtp-h263-video-01.txt

- RTP Payload Format for the 1998 Version of ITU-T Rec. H.263 Video (H.263+), draft-ietf-avt-rtp-h263-video-02.txt, May 7, 1998.