| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) Fifth Meeting: Whistler, Canada, 21-24 July, 1998 | Document Q15-E-12 Filename: q15e12.doc Generated: 20 July ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Stephan Wenger TU Berlin, Sekr. FR6-3 Franklinstr. 28-29 D-10587 Berlin | Tel: Fax: Email: | +1 415 297-2063 +1 604 736-7853 stewe@cs.tu-berlin.de |
| Title: | Proposal for an H.263+ Test model extension for H.324M- mobile Video |  |  |
| Purpose: | Proposal |  |  |

_____________________________

## 0. Document Structure

This document can be considered as an updated version of Q15-D64r2, which was an output of the Tampere meeting.    It contains several new aspects relative to D64r2 with respect to both the channel characteristics and the video coding itself.    The main changes are as follows:

- Q15-D-64r2 distinguished between two different types of errors within delivered H.223 SDUs.    It was made clear in Cannes, that this distinction (between packets with MUX errors and packet with CRC errors) is not practical.    Therefore, this test model specification contains only one type of packets with errors, and distinguishes only between three (and not four) cases in general: SDUs delivered without errors, SDU delivered with errors, and lost SDUs.

- Q15-D-64r2 proposed to discard any packets containing errors and deal with the resulting artifacts by using VRC on individual GOBs.    Recent research at UBC showed a more efficient method to code error resilient video (by adding a sufficient high number of INTRA MBs).    This document reflects this research.    Simulations results are presented in Q15-E-xxx.

The document structure remains unchanged as follows:

- It introduces the video-oriented Q.15 experts to the error-prone environment of H.324 mobile applications.    More specific, it describes the possible situations, which can happen when a H.223 AL3 (video-) SDU is delivered to a receiver using a mobile channel.

- It describes a ‘packetization’ scheme, similar to the one defined for the Internet/RTP environment (as outlined in Q15-D20) for interfacing a H.263+ codec to H.223.    The author feels that it might be necessary to make this part normative for similar reasons as on the Internet payload specifications have normative status.    There are, however, no changes in the data structures necessary; the only necessary change is a slight restriction in the way in which the AL3 SDUs are used.

- It outlines a test model for H.263+ error resilience mechanisms for the mobile case (based on the ‘packetization’ scheme as defined in 2).    This includes some restrictions regarding the use of the packetization scheme, and information about the applicable optional modes of H.263+.

- It briefly outlines some ideas on common test conditions for this environment.

## 1. H.223’s AL3: background information

H.324 terminals rely on H.223 transport.    H.223 includes three “adaptation layers” (Alx) for various data types.

AL3 is used for video data, although it is unaware of many video specific data structures like GOBs or synchronization points, except that H.324 states that the start of H.263 pictures should be byte aligned and start an AL3 SDU.    It features 16 bit CRC over its SDUs, and a re-transmission mechanism for damaged SDUs.    The re-transmission algorithm is mandatory; sending terminals are, however, free to use or not to use it based on factors like round-trip-delay.    It is not allowed to re-transmit SDU more than one time, leading to a well defined delivery time but also to a non-guarantied quality of delivery: SDUs can be lost or incorrectly transmitted even if the re-transmission algorithm is used.    The maximum length of a (variable length) AL3 SDU is 2048 bytes and it is always byte-oriented.    AL3 is part of the network-independent sections of H.223, and thus used over both wireline and wireless networks.

Hierarchically “below” AL3’s mechanisms, AL3 PDUs are delivered to H.223’s multiplexing mechanism.    This layer achieves a low latency multiplexing of various ALx PDUs according to their importance.    ALx PDUs might be broken up into smaller multiplexing entities, and the link is time-multiplexed between those smaller Mux PDUs.    This mechanism allows sending large (e.g. video) ALx PDUs without hurting critical latency requirements of other data types (e. g. audio).

All the described mechanisms are common for all H.223 systems, including wireline and wireless.    Mechanisms below the multiplex layer are defined in both the main body of H.223 and its Annexes A, B, and C to provide a scalable error protection mechanism for mobile networks of various types.    H.223 “baseline” (without the use of any of the Annexes) is intended for wireline networks and has shown acceptable performance using today’s modem technology.    For the much more error-prone mobile channels, the three Annexes provide scalable error-protection at the cost of additional computational complexity, bit-rate overhead, and latency.    It is beyond the scope of this document to describe the mechanisms of those Annexes in detail, please see [H.223] for more information.

The adaptation layer service interface is not defined in H.223.    A hypothetical interface might use the following data structures:

- The packet itself (payload),

- An Error-Flag indicating the results of a CRC check and/or any error reported by the MUX layer,

- A length field.

Note, that for this document it will be assumed that both length and payload information were transmitted error-free, whenever the Error-Flag indicates no error.    It is understood that H.223 cannot guaranty this condition, but that the probability of a packet to pass the CRC check (and the other multiplex-level error-control mechanisms in H.223) without reporting the error is insignificantly low.    Decoders should, however, have the capability to re-synchronize to broken data streams and not crash in such a case.    The probability of packets with reported errors depends on the bit error rate and the distribution of those bit errors as well as on the size of the AL3 SDUs.

## 2. A packetization scheme for H.263+ into AL3 SDUs

This section describes a general packetization scheme for H.263+ coded video and H.223’s AL3 SDUs.    The author feels that it might be valuable to mandate some of the characteristics as normative content, similar to the H.263+ on RTP packetization RFC for H.323.    Procedural questions, however, are outside the scope of this document.

Based on the current H.223 and H.263 recommendations, it is possible (and implementation dependent) to split H.263 video data at any byte-boundary.    For this packetization scheme, however, more limited mechanisms are proposed to achieve a better re-synchronization capability without employing special VLC-resynchronization mechanisms in video decoders.    The purpose of this section is to provide ideas for normative content that could to be added to future versions of H.223, H.324, or to a H.324 implementers guide.

H.263 video coding works with variable length codes (VLCs), which make re-synchronization to the data stream difficult if the synchronization was lost due to bit errors or to packet losses.    To facilitate this re-synchronization, H.263 (as many other video coding standard including H.261 and H.262) provides synchronization points in the form of picture, GOB and slice headers, which can be easily found in case of lost synchronization.    H.263 is designed that information following a synchronization point can be decoded independently from previous information.    That is, none of the prediction mechanisms within a single coded picture (such as motion vector prediction or advanced INTRA prediction) is used across synchronization points.    (There are also mechanisms available that isolate any decoding error to the spatial region they occurred in, by not allowing motion compensation across those spatial boundaries.    It is not proposed to use those mechanisms (ISD) because of the lower coding efficiency).

We propose that each AL3 SDU shall start with either a picture or a GOB header, or a Slice header.    We do not propose the use of slices in the test model of section 3 below, because they make the use of error concealment techniques much more difficult.    Implementers, however, might discover better ways to perform error concealment, and should not be restricted in using slices, which also might be helpflu for interoperability within gateways.    Since AL3 SDUs can be up to 2048 bytes long, the bit-rate of the proposed mobile channels is comparably low, and a coded picture can contain a lot of synchronization point, it is unlikely that any application needs to send AL3 packets not starting with a synchronization point.

### 2.1. Encoding and packetization

Due to the low overhead of the H.223 packetization, which is in the order of 5 bytes per AL3-SDU (including a 16 bit CRC), it is reasonable to use much smaller packet sizes than for example on the Internet (where the overhead is around 40 bytes per packet).    It is therefore recommended to break up the coded picture into several packets.    For now, the author proposes to use the GOB mechanism, because scan-order slices are not really compatible with the Reference Picture Selection mode (as to be outlined outlined in Q15-E-xx) and not too much experience with rectangular slices is yet available to the author.

Each GOB containing macroblock data (there is at least one macroblock in the GOB with COD=0) should be coded with a header.    If one or more preceding GOBs do not contain any coded data, they should be included into this GOB (and the GOB address should be adjusted accordingly).    At the start of a picture, a picture header (which is the first GOB header) is coded.    After, all empty GOBs are coded and the first non-empty GOB is coded as well in the same packet.    The second packet contains the GOB-header of the first GOB after the previously coded non-empty GOB, all subsequent empty GOBs and the first non-empty GOB.    This scheme should be repeated until the end of the picture is reached.

The reason for not coding the headers of empty GOBs is the incurred overhead, which is in the order of 10 bytes (5 bytes packetization overhead, plus 40 bits for the GOB header; the 11 COD bits do not count as overhead because they have to be coded anyway).    The reason why the empty GOBs are coded before the full GOB is coded is that the spatially ‘higher’ GOBs are more likely to be empty than the spatially ‘lower’ GOBs.    For many video sequences, this will lead to one less AL3 SDU per picture to be sent.    A coded picture with GOBs 1, 5, 7 and 8 empty would be packetized as follows:

| Packet | Content |
| --- | --- |
| 1 | Picture header, 11 COD bits indicating empty GOB 1, GOB 2 without header |
| 2 | Header and data of GOB 3h |
| 3 | Header and data of GOB 4 |
| 4 | Header of GOB 5, 11 COD bits indicating empty GOB 5, GOB 6 |
| 5 | Header of GOB 7, 22 COD bits indicating empty GOB 7 and 8, GOB 9 |

Table 2: example for packetizing a QCIF picture with empty GOBs

Please note that this scheme needs additional investigation to ensure that it does not pretend decoders to use GOB-based error concealment.

### 2.2. Decoding

Since the packetization scheme operates on GOB level, and each packet starts with GOB headers, those packets are essentially self-contained if the picture header of the picture the GOB belongs to is available.    The mechanism of error control beyond the requirement of starting a packet at a synchronization point of H.263+ is outside the scope of this set of rules.    Section 3 provides one idea of a mechanism about which the author feels that it might produce good results.

Generally, all packets containing payload have to be collected and combined to one coded picture.    If complete packets are lost, error concealment techniques can be used to achieve a higher picture quality.    Packets containing bit errors might be discarded or (partially) decoded depending on the decoders capabilities.

## 3. Test model description

This section outlines a description of the Q15 test model for the mobile communication case.    The test model assumes an implementation of the payload specification as described in section 2, along with some additional restrictions.

The idea of the test-model is to send GOBs in AL3 packets, with AL3’s CRC mechanism used and AL3’s re-transmission mechanism not used (because of the incurred delay and difficulties in implementing a sufficient simulation model).    All packets that are received along with an evidence of possible damage (by one of the various error-checking means as AL3-CRC, and MUX-EI flag, see section 1) are discarded.    That is, only AL3 SDUs with no reported errors will be decoded.    This simplified mechanism will possibly lead to a lower performance than to allow some form of a decoding attempt of a possibly damaged AL3 SDU (to recover at least parts of the damaged data).    Future enhancements might be necessary.    Descriptions of possible mechanisms along with simulation results are solicited.

Since some wireless applications do allow the use of a feedback-channel and others do not, two cases are distinguished in the following:

- Uni-directional transmission is applicable for multicast or broadcast channels and protocol hierarchies without means to convey a back channel message, and

- Bi-directional transmission, applicable for all other applications.

Please note that H.324 mobile terminals likely belong to the second category, if the codecs implement Annex N.

### 3.1. Uni-directional transmission

To simulate the erroneous channel, the sequence is first coded using the H.263+ specific coding tools.    After, the data is packetized as described in section 2.    The packet loss/damage is simulated by deleting or patching packets.    Document Q11-F-06 (identical with Q15-E-xx) describes Q15’s requirement to such a tool.    Last, the remaining data is decoded by a compliant decoder employing error control and error concealment techniques.

### 3.1.1. Encoding

H.263 encoders should use a higher INTRA update frequency to achieve error resilience.    Document Q15-E-xx provides an outline of the necessary number of I-updates.    As a start, we propose here to send an I-MB latest after the spatially corresponding MB was sent at most 10 times with coefficient information.    The initial I-MB update pattern should be pseudo-random to avoid bursts in the GOB size, as outlined in Q15-E-xx.

### 3.1.2. Packetization

The packetization process should follow the guidelines described in section 2.

### 3.1.3. Erroneous channel simulation

To simulate the error prone channel, the sequence of packets resulting out of the packetization process should be processed through a damage simulator.    This simulator applies bit errors by patching individual bits in a packet and multiplex errors (which can have the several forms).    The output of the damage simulator is again a sequence of packets (not necessarily the same number/size of the initial packets), along with a sequence of bits indicating the correctness of each packet and a packet length information.

### 3.1.4. De-packetization, reconstruction of the coded data

The de-packetization process discards all damaged data packets (as reported by the corresponding error flag) and reconstructs a data stream by concatenating data packets identified as correct.    Please note that the resulting bit-stream is not fully H.263 compliant, because GOBs might be missing.    Since each packet starts with a GOB header (as mandated by the packetization procedure of section 2), missing GOBs can be identified by the decoder and appropriate error-concealment action might be performed.

[It might also be possible to reconstruct a correct data stream be adding pseudo-GOBs (with uncoded blocks) and/or by adding an external signal to identify those GOBs (this can also be done using non-standardized supplementary information).    Suggestions are solicited.]

Please note that the described procedure might lead to non-optimal results because packets with single bit errors are completely discarded, resulting in the loss of partially correct information.    This part of the test model is likely to change in the future to incorporate more sophisticated mechanisms.

### 3.1.5. Decoding

The decoding process is straightforward, if no error concealment techniques are used.    There are, however, error concealment techniques known which greatly improve the quality of the reproduced image sequence, for example the one in TMN10.    The test model mandates the use of this error concealment technique.

### 3.2. Bi-directional transmission

If the scope of the communication relationship is limited to the point-to-point case (as it is true for most H.324/M systems), then most system environments including H.324/M allow to employ the powerful mechanisms using a feedback channel.    The feedback channel informs the encoder about the decoder’s status regarding the successful decoding of pictures or picture segments (e. g GOBs).    This information can be used by the encoder to choose an earlier reference picture (or reference picture segment) to avoid erroneous or not-decoded reference material.

H.263+ features two different ways of conveying such back-channel information: the information can be multiplexed in the video bitstream of the opposite direction (VideoMux back channel) or it can be conveyed by other (external) means.    Since it would be very difficult to simulate the VideoMux back channel (because the characteristics of the video information of the opposite direction are not known and would have to be simulated simultaneously) the test model assumes an external back channel.    The only standardized way to convey such external back channel messages in H.324M terminals is through an independent virtual H.223 channel, using either AL2 or AL3 depending on the nature of communication (see H.324 section xxx for details).    This transport is also subject to eventual losses and is still difficult to simulate, although not that difficult as the VideoMux option.    There is, as today, no means to convey a back channel message over a reliable channel such as H.245 messages.

Since both mechanisms available in current standards are difficult to simulate, the author proposes to assume a reliable back channel.    This will gain a highly simplified model, which is comparably easy to implement.    The author feels, that even such a simplified solutions will gain a good estimation of the performance of the back channel mechanisms as compared to other methods.    Later versions of this test model might suggest more sophisticated means to simulate the erroneous nature of the back channel data.

Encode a picture (possibly using RPS back channel information)

H.263+ coded picture

Packetization

Each packet contains one non-empty GOB

Back channel message delay simulation, provided by Q.11 experts

AL3 SDUs (packets)

Loss/Damage Simulation,

provided by Q.11 experts

Back channel message

(possibly damaged) AL3 SDUs, CRC, Mux EI-Flag

Back channel message generator, CRC check, de-packetizer

H.263+ picture (Containing per GOB RPS info)

RPS capable H.263+ decoder, w/ optional error concealment

### 3.2.1. Encoder

The encoder processes one picture at a given time, using the (simulated) back channel information regarding lost GOBs.    This is done by not coding an early temporal reference in Annex N’s forward channel information if the GOB was not reported to be lost.    Otherwise, an appropriate temporal reference is coded.

### 3.2.2. Packetization

The packetization process should follow the procedures described in section 2.

### 3.2.3. Loss/Damage simulation

See 3.1.3.

### 3.2.4. Back channel generation, De-packetization

See 3.1.4.    Additionally, all known to be lost GOBs are reported to the Back channel delay simulation.    Please note, that this might include empty GOBs, since it is impossible to identify empty/non-empty GOBs after a sequential loss of more than two packets.    If only one, or two packets are lost sequentially, only those GOBs known to contain information should be included in the back-channel message.

### 3.2.5. Decoder

See 3.1.5.

### 3.2.6. Back channel message delay

The back channel message delay module should delay any back-channel message in an appropriate way.    The delay should be expressed in intervals corresponding to the picture clock frequency.    For now, Q.15 experts recommend to use a fixed delay, but Q.11 experts are encouraged to investigate the possibility to develop a tool which provides a better simulation of a real-world’s system.    It should be assumed, that the back-channel message is conveyed over H.245, employing the codepoints available in H.245R3 for this purpose.

## 4. Common conditions

This section is still incomplete.    Some ideas are sketched as follows:

- Error patterns @ various error rates.    To be provided by mobile experts.

- Data streams like in Q15-D65, or only QCIF?    QCIF and SQCIF?

- Constant bit-rate?    Would be more realistic.    What bit-rates?    Specify rate control?

- Reasonable delay for back channel message (possibly including distribution)

## 5. To do list

- Integration of Toshi’s ACK/NACK back channel model (VideoMux only???    No, seems not.    Get more info from Toshi about it.)