| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fifth Meeting: Whistler, BC, Canada, 21-24 July, 1998 | Document Q15-E-24 Filename: q15e24.doc Date generated: July 16, 1998 |
| --- | --- |

| Question: | Q.15/16 |  |  |
| --- | --- | --- | --- |
| Source: | Kook-yeol Yoo and Dong-seek Park Multimedia Laboratory Samsung Electronics, Korea | Tel: Fax: Email: | +82-331-200-3746 +82-331-200-3717 kyoo@mmrnd.sec.samsung.co.kr |
| Title: | An adaptive resynchronization method based on Intra-GOB Maker (IGM) for transmission of coded video signals over error-prone channel |  |  |
| Purpose: | Proposal |  |  |

1. Introduction

In this contribution, we propose to use more resync(resynchronization) patterns to minimize the error propogation. Since each GOB and MBs in the GOB have unequal impacts on the quality of reconstructed image, resync patterns should be inserted in an adaptive manner according to importance of each GOB and MBs within the GOB. For the purpose, we propose a modified syntax to achieve adaptive resync patterns.

This contribution is organized as follows: Section 2 gives the theorectical background and necessity of using adaptive resync pattern. A modified syntax for the proposed resync patterns is described in Section 3. The performance of proposed method is compared with original H.263++ codec for several channel errors in Section 4. Finally our conclusions are drawn in Section 5.

2. An adaptive IGM based resynchronization method

2.1 Effect of resynchronization on MB errors

The error is modeled in MB and the error probability for each MB is assumed to be same, i.e., MB-based equiprobable error model is used. Note that if the k-th MB in the i-th GOB is lost due to channel error, then all MBs from k-th MB to the last MB in the GOB is lost due to error propagation before next synchronization patterns such as GBSC or PSC. Also it should be noted that if the COD of lost MB is equal to one, the simple replacement of the MB of the same spatial position in the previous frame doesnot degrade the quality in the reconstructed image. Based on the above assumptions and reasoning, the following cost function is formulated:

(1)

where represents the number of lost MBs with COD of zero for the j-th GOB when the i-th MB in the GOB is corrupted by channel errors, the subscript zero in  represents that there is no resync patterns except GBSC and PSC, and  is defined as follows:

where n represents frame number. Notice that a frame consists of g GOBs and GOB consists of m MBs.

Since the equi-probable MB error model is assumed that all the MBs in the j-th GOB has same error probability, for instance, 1/m for single MB error in the GOB, the mean of the error cost for the j-th GOB is given by

(2)

Therefore, we have the following corollary:

Corollary 1: The quality of reconstructed image is determined by the number of MBs with COD of zero. Therefore in the sense of channel error protection, the GOB with large number of MBs with COD of zero should be more protected than the GOB with small number of MBs with COD of zero.

MB1

MB2

...

IGM

GH

MBk+1

MBm-1

...

MBm

GH: GOB header, IGM: resync pattern

Fig. 1: GOB structure with a single resync pattern

Let us consider the position of resync pattern when inserting one resync pattern for a given GOB, as shown in Fig. 1. When all the MBs in the j-th GOB have CODs of zero, i.e., , the mean error cost is given by

(3)

If there is a resync pattern just after k-th frame, the mean error cost for the GOB can be given by the following equation:

(4)

By differentiating  with respect to k, we can obtain the position of resync pattern where minimum mean error occurs.

(5)

By applying the above results, we can get the following mean error when a resync pattern lies in just after k* -th MB:

(6)

By comparing eqns. (3) and (6), the mean error cost is reduced by m2pe/4. So, the resync pattern can be a powerful tool in reducing the channel error effects.

Let us consider for the GOB with r resync patterns as shown in Fig. 2

MBk

MB1

...

IGM1

GH

MBk2

MBm-1

...

MBm

MBk1

IGM2

...

IGMr

MBkr+1

MBkn

GH: GOB header, IGM: resync pattern

Fig. 2: GOB structure with n resync patterns

By using similar procedure used for the GOB with single resync pattern, the following equation is obtained:

(7)

By differentiating the above equation for ki, i=1,...,r and equating the resulting derivatives into zeros, the following positions for the resync patterns are obtained:

(8)

Let us further consider the situation that several MBs have CODs of ones, for instance  MBs have COD of zero. Since the MB with COD of one does not affect the reconstructed signals, the above results can be used only by changing m into . From these results, the following corollary is formulated:

Corollary 2: If r resync patterns are used for a GOB which has MBs with COD of zero, then the position of the i-th    resync pattern address is given by

(9)

2.2 The proposed adaptive IGM-based resynchronization method

Based on the Corollaries 1 and 2, we propose to insert resync pattern within a GOB, namely IGM(Intra-GOB Maker)-based resynchronization algorithm. The algorthim is described as follows:

Step 1: Initialization and coding the first frame only using intra-frame coding method

, , and

where n is a frame number and is the residual number of IGM resync patterns.

Step 2: Allocation of the number of resync patterns for each GOB

Step 3: Computation of resync pattern positions using (9), i.e.,

Step 4: Coding (n+1)-th frame and updating

Step 5: Updating parameters as follows:

and then

Step 6: Go to Step 1

- Proposed syntax for the adaptive IGM algorithm

To describe the proposed algorithm, the syntax of H.263++ , especially on the GOB layer, is modified as shown in Fig. 2. In Fig. 2, the syntax element, IGM is described as follows:

...

GN

RSF

MBA

GBSC

IGM

GBSC: GOB start code, GN: GOB number,

RSF: resync flag, MBA: absolute MB address in a GOB

Fig. 3: Syntax of resync pattern, IGM

Also, the GOB header denoted as GH in Fig. 1 and Fig. 2 is modified as follows:

=

GSBI

GFID

GQUANT

RSF

GBSC

GN

GH

Fig. 4: Syntax of modified GOB header

If RSF is equal to one, the detected synchronization syntax is the resync pattern, otherwise it is the GOB sync pattern. As shown in Fig. 3, MBA is also inserted to indicate the location of resync pattern.

- Simulation results

- Subject quality : Video demonstration (D1-tape)

- Object quality measure: averaged PSNR for luminance signals

- Simulation environments:

- Test sequence: Mother and Daughter (310 frames), Hall (90 frames), Foreman (120 frames), and News(95 frames) where the numbers of frames in the test sequences are the one of coded frames.

- Error profiles:

- DECT2 and three wideband CDMA error profiles

| error profiles | doppler frequency (Hz) | average bit-error rate | Average burst length (bits) |
| --- | --- | --- | --- |
| w54(wcdma-64kb-005hz-4) | 5.3 | 8.17x10-5 | 11 |
| w74(wcdma-64kb-07hz-4) | 70 | 1.21x10-4 | 13 |
| w21(wcdma-64kb-211hz-4) | 211 | 9.37x10-5 | 11 |

- Coding options: no advanced prediction

- Error concealment: simple replacement of MBs between nearest sync patterns.

The simulation results showed rather low correlation between the two results, objective results shown in Figs. 5, 6, 7, and 8 and the subject results to be shown in video demonstration. The main reason is due to missing several frames in the course of decoding. Thereby, measuring PSNR between the non-corresponding original frames and decoded frames results in very low PSNR, though the subjective quality of decoded frames are very high.

Fig 5: Mother and Daughter sequence

Fig 6: Hall sequence

Fig 7: Foreman sequence

Fig 8: News sequence

5. Conclusion and remarks

From the simulation results, the proposed resync method gives remarkable improvements in subjective quality. We believe that the proposed resynchronization method would be very powerful tool for transmission of coded video signals over error-prone channel.

=