| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fifth Meeting: Whistler, British Columbia, CA, 21-24 July, 1998 | Document Q15-E-31 Filename: q15e31.doc Generated: 15 July ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Detlev Marpe Humboldt-University Berlin Augustenburgerstr. 1 D-13353 Berlin Germany | Tel: Fax: Email: | +49 30 450 57182 +49 30 450 57117 marpe@charite.de |
| Title: | Video Demonstration for PACC-Codec |  |  |
| Purpose: | Information |  |  |

# 1.	Introduction

This document describes the detailed test conditions which were used to produce the material provided for the video demonstration of the PACC-Codec (see document Q15-D-57 [1] for a technical description of our proposed coding method). Following the recommended simulation conditions for H.263v3 we demonstrate a set of sequences decoded at various bit-rates, frame rates and spatial resolutions. In addition, we provide data obtained by objective measurements at the same conditions.

# 2.	Test Conditions

For our experiments we have used 6 test sequences at QCIF resolution and one additional test sequence at CIF resolution. In table 1, we have listed the parameters of each sequence including the quantization step sizes (Q) which were used for encoding with the Test Model. The relevant bitstreams of TMN-coded sequences have been downloaded and decoded to provide the anchor for subjective comparison.

| Target Frame Rate [fps] | 10 | 15 |
| --- | --- | --- |
| Sequences | Hall-Monitor, QCIF,300, Q=10, 30 fps Container, QCIF,300,Q=7, 30 fps News, QCIF,300 Q=15, 30 fps Foreman, QCIF,300, Q=7, 30 fps | Paris, CIF, 900, Q=15, 30 fps Silent Voice, QCIF, 300, Q=7, 30 fps Tour of Glasgow, QCIF, 750, Q=10, 15 fps |

Table 1

### 2.1.	I-Frame Coding

For coding of the first frame in intra mode we have used approximately the same number of bits as for the first frame of the anchor bitstream. The exact bit-numbers and PSNR-values obtained for each of the test sequences are given in table 2.

### 2.2.	P-Frame Coding

Given the overall bit-rate produced by TMN10 we have adjusted the quantization parameter for coding of P-frames accordingly to meet this specific bit-rate. Throughout one run of P-frame coding the quantization value was kept constant. The PSNR values given in table 2 are computed as the arithmetic mean of the measured PSNR values of each frame including the first I-frame. The bit-rate (in kbit/sec) is given as the average of all bits per P-frame multiplied by the target frame rate which was obtained by temporally subsampling the original sequence with an appropriate factor.

# 3.	Test Results

All presented results have been derived from decoded bitstreams. Objective measurements are documented in table 2 according to the test conditions given in section 2.

|  | I-frame data |  |  |  | P-frame data |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequences | Y-PSNR | U-PSNR | V-PSNR | Bits | Y-PSNR | U-PSNR | V-PSNR | Bit-Rate |
| Container Hall-Monitor News Foreman Silent Voice Tour of Glasgow Paris | 38.00 42.67 42.88 31128 36.69 40.07 42.20 21368 32.76 37.56 38.40 18568 38.90 43.58 45.41 30040 37.21 41.18 42.01 31440 34.65 39.41 40.81 32976 32.22 36.85 37.31 90568 |  |  |  | 34.64 40.81 40.68 28.27 33.61 38.40 40.72 16.69 30.22 35.89 36.82 18.67 34.17 40.18 40.70 78.79 34.64 39.30 40.09 50.73 31.12 37.48 39.34 91.76 28.79 34.55 35.18 106.16 |  |  |  |

Table 2

# 4.	Conclusion

The PACC coding method has proven to be competitive or superior to the TMN10 coder both with respect to objective and subjective evaluation. Its underlying motion model is fairly simple and offers a great potential of further increase in coding gain and of visually improved quality. For example, the block-based prediction can be greatly enhanced by integration of

- an ‘advanced prediction mode’ with variable block size

- some kind of block-wise intra mode for local prediction failure

- RD-constrained motion estimation and mode decision

These features are useful and efficient methods of TMN10 and can be easily adapted to the frame-based residual coding of the PACC-method.

Another approach might be to integrate new ideas in motion estimation which break the limitations given by the traditional block-based motion model. A motion estimation technique using an image warping model combined with the PACC residual and intra-frame coding has already shown promising results [2].

Other sophisticated motion models like the region-based Nokia-approach in combination with the PACC-coder might offer increased coding efficiency as well. Moreover, the possibility of spatial scalability and object tracking over time is given by such advanced schemes in a more natural way.

# References

- D. Marpe and H.L. Cycon: Very Low Bit Rate Video Coding Using Wavelet-Based Techniques, to appear in IEEE Transactions on CSVT, Sept. 1998.

- G. Heising, D. Marpe, H.L. Cycon: A Wavelet-Based Video Coding Scheme Using Image Warping Prediction, to be presented on IEEE Int. Conf. on Image Processing, Chicago, Oct. 1998.