| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Sixth Meeting: Seoul, Korea, 3-6 November, 1998 | Document Q15-F-43 Filename: q15f43.doc Generated: 5 Nov ’98 |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Keiichi Hibi, AHC Chairperson Sharp Corporation tel: +81 43 299 8537 fax: +81 43 299 8509 email: hibi@trl.mkhar.sharp.co.jp |
| Title: | Summary of the technical evaluation of H.26L proposals |
| Purpose: | Report |

_____________________________

1. Summary of the features of technical proposals for H.26L

| Q15-F-11 (Telenor) Variable Block Size MC with 4x4 transform |  |  |
| --- | --- | --- |
| Intra Frame Coding |  | 5 prediction mode for each 4x4 block (DC, horizontal, vertical, two diagonals) Prediction mode is coded refering to neighboring blocks Codeword is assigned based on the probability of the mode |
| Inter Frame Prediction |  |  |
|  | M.C. | Prediction from more than one past frame (no B-pictures usage is intended) Different block sizes for prediction 16x16, 8x8, 4x4 |
|  | M.E. | Step search from 16x16 down to 8x8, 4x4 smaller search window for 8x8 and 4x4 Approximate bit usage is taken into account during the search |
| Residual coding |  |  |
|  | Transform | Use of 4x4 transform integer transform, almost identical to DCT |
|  | Attribute | Use of H.261 like CBP MB structure: 16 Y and 8 U, V CBP signals coded or not coded of four 4x4 blocks |
|  | Quantizer | 32 different QPs coresponding to similar range of H.263 No dead zone |
|  | Scan | Double scan of 4x4 block for high QP values, Each 8 coefficients in 4x4 block are scanned separately |
|  | VLC | Only one VLC for coding (VLC-D) Parameters designed to have the same statistics Similar to Annex D, but not reverse decodable Capable of fast recovery of VLC synchronization 1bit EOB, 2D-VLC, no escape coding (max. length=27) |
| Current implementation |  | No long-term memory is used |
| Remarks on the results |  | Improved performance in Intra coding More improvement in higher bitrate |

| Q15-F-18, Q15-F-19 (Univ. Strathclyde) SCT (Strathclyde Compression Transform) - Variable Dimension Vector Quantization |  |  |
| --- | --- | --- |
| Intra Frame Coding |  | Identical to the anchor |
| Inter Frame Prediction |  |  |
|  | M.C. | Decomposition of image Based on quad-tree decomposition of picture But, not restricted by quad-tree structure VQ and form of MC Variable Dimention VQ 32x32, 16x16, 8x8, 4x4 blocks code book size for each dimention is 256 Prediction from previous frame (no B-pictures) Different motion vectors for luma and chroma Zero motion with changing brightness |
|  | M.E. | Best-first rule for selecting block to be coded |
|  | Coding | Index of code book is not entropy coded 8 bits for coding the index 6 bits for mean and 6 bits for gain of the vector Entropy coding for brightness, block positions, etc. |
| Residual coding |  | Any other residual coding means can be combined Not used in the current implementation |
| Current implementation |  | Off-line codec Real-time encoder with different VQ code book |
| Remarks on the results |  | Better than anchor in high QP values (lower bitrate) |

| Q15-F-24 (Nokia) MVC - Affine MC of segmented frame with multiple transform |  |  |  |
| --- | --- | --- | --- |
| Intra Frame Coding |  |  | Pixel prediction (7 directions) and DC prediction Prediction error is coded similar concept of Inter prediction DC transmitted separately Different VLC tables from Inter coding One more 4x4 ECVQ code book for /8 direction Three directional 8x8 KLT Predetermined most probable basis functions |
| Inter Frame Prediction |  |  |  |
|  |  | M.C. | Segmentation of frame Quad-tree like splitting Composed of 8x8 blocks Motion assisted merging for motion field coding Affine Motion Model Motion of all pixels are modeled by affine function Six motion coefficients affine function Subpixel values by cubic spline interpolation (separable 2D filtering with 4x4 window) |
|  |  | M.E. | Motion estimation and split decision Gauss-Newton minimization 5 iteration on subsampled image Motion model adaptation |
| Residual coding |  |  |  |
|  | Transform |  | Multiple transform (4x4) Multishape DCT, directional VQ, extrapolation 7 ECVQ code books, (sizes are 100 to couple of 100) Low/high energy horizontal Low/high energy diagonal Low/high energy texture Flat |
|  | Attribute |  | Classification concept Classified by localizing active area Directionality of activity Calculated on prediction frame Classifier Calculate variance for 4x4 pixels in predicted frame 5 different cluster sizes: 8x8, 4x8, 8x4, 3x8, 4x4 4x4 block is coded one of multiple transform methods Other sizes by DCT |
| Current implementation |  |  |  |
| Remarks on the results |  |  | Improvement in both Inter and Intra Similar bit savings in low to high bitrate range |

| Q15-F-33 (U. Erlangen-Nurem. and 8x8) Long-term frame memory with affine MC |  |  |
| --- | --- | --- |
| Intra Frame Coding |  | Identical to the anchor |
| Inter Frame Prediction |  |  |
|  | M.C. | Long-term frame memory with affine MC 6 parameters affine transform Reference picture selection is signaled MB by MB |
|  | M.E. | Searching affine motion parameters Gradient-based approach |
|  | Coding | Affine parameters for warped version of reference frames Transmitted in picture header |
| Residual coding |  | Identical to the anchor |
| Current implementation |  | Long-term memory of 10 previous coded frames 32 affine warped reference frames |
| Remarks on the results |  |  |

2. Action Items to be focused on for H.26L development

2.1 Key Technical Areas of interest by H.26L proposals

# Inter frame prediction coding

(1) Variable-size block-based segmentation

- Quad-tree like decomposition of frame

- Small block size to 8x8, 4x4

(2) Long-term memory

- increase candidates of reference materials for prediction

- combination with other prediction scheme (e.g., affine warping)

(3) Affine motion model

- adoption of six parameter affine motion model

- coding method and syntax of affine motion parameters

- searching of affine motion parameters

# Inter frame residual coding (also applicable to Intra texture coding)

(4) 16-sampled waveform coding (4x4)

- coding with smaller block size than 8x8 DCT

(5) VQ, EZW with DCT, multiple waveform coding

- adoption of (variable-dimention) VQ scheme

- embedded EZW structured coding

- selection from multiple coding mode

# Intra frame coding

(6) Directional (pixel-prediction) Intra coding

- Improvement of prediction in Intra coding

- Consideration of activity direction of picture

Note: cordination with JPEG-2000 to obtain an information of the state of the art of Intra coding technology is helpful.

2.2 Area of functionalities to be worked on

(a) Simplification “back to basics”

- use of one VLC for all parameters to be coded

- for example, adoption of 2D-VLC, CBP, no OBMC,…

(b) Low delay (no B picture)

- use of more references for prediction (e.g., long-term memory)

- constant bitrate coding of each frame to reduce buffering delay

- to be extendable to different delay environment

(c) Error resilience

- needs more work…

(d) Complexity scalability in encoder

- asynmetry of encoder and decoder processing complexity

- scalability between amount of encoder processing and achivable quality

(e) Full specification of decoding (no mismatch)

- resolve mismatch problem (e.g., integer transform, VQ,…)

(f) High quality application

- performance improvement in higher bitrate

- applicability to broadcasting quality applications

(g) Network friendliness