| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Sixth Meeting: Seoul, Korea, 3-6 November, 1998 | Document Q15-F-44 Filename: q15f44.doc Generated: 11. Nov ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wiegand Telecommunications Laboratory University Erlangen-Nuremberg Germany Jani Lainema Nokia Research Center P.O.Box 100, Sinitaival 6 FIN-33721 Tampere, Finnland | Tel: Fax: Email: Tel: Fax: Email: | (9131) 85-27117 (Germany) (9131) 85-28849 (Germany) wiegand@nt.e-technik.uni-erlangen.de (358) 40 525 8946 (Finnland) (358) 3 272 5322 (Finnland) jani.lainema@research.nokia.com |
| Title: | Core Experiment on Affine Motion Compensation |  |  |
| Purpose: | Proposal |  |  |

_____________________________

Paticipants:

Nokia Research Center (Contact: J. Lainema, jani.lainema@research.nokia.com)

University Erlangen-Nuremberg (Contact: T. Wiegand, wiegand@nt.e-technik.uni-erlangen.de)

The core experiment is based on proposals Q15-F-24 and Q15-F-33. In both documents, motion compensation using affine models is employed resulting in significant coding gains. Although, affine models are used in both proposals, there are differences regarding the syntax and motion estimation. Common to both proposals is the use of orthonormalized basis functions for motion field coding and cubic spline interpolation for sub-pixel position warping. In fact, a trans-coder could be built that maps the information specified using the syntax of one of the proposals into the syntax of the other proposal representing the same motion-compensated prediction signal.

The proposal in Q15-F-24 (Nokia) emphasizes on:

- Inter-connected image segments that are assigned affine motion parameter sets

- An algorithm for motion estimation taking inter-connectivity into account in ist optimization strategy

On the other hand, proposal Q15-F-33 (Uni Erlangen) emphasizes on:

- Disconnected image segments and macroblock based motion compensation similar to H.263

- An optimization strategy using warped reference frames

- Motion compensation using several past decoded frames

The core experiment targets the following issues:

- Investigation of partial benefits of syntax elements in both proposals

- Both sides will test features of the other proposal

- Exchange of test results

- Based on test results, creation of improved syntax

- Common proposal for syntax specifying affine motion compensation

- Verification of results

- Cross checked common syntax and encoding method via independent implementation

- Bit-stream exchange

- Output documents from both sides describing results obtained with improved syntax and encoding method