| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eighth Meeting: Berlin, 03-06 August, 1999 | Document Q15-H-32 Filename: q15h32.doc Generated: 31 July ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Guido Heising Heinrich-Hertz-Institut Berlin Einsteinufer 37 10587 Berlin, Germany Detlev Marpe Heinrich-Hertz-Institut Berlin Einsteinufer 37 10587 Berlin, Germany Hans L. Cycon Fachhochschule für Technik und Wirtschaft Berlin Allee der Kosmonauten 20-22 10315 Berlin, Germany Alexander Petukhov Dept. of Mathematics State Technical University Polytechnicheskaya st., 29 St. Petersburg 195251, Russia | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: Fax: Email: | +49 30 31002 226 +49 30 3927200 heising@hhi.de +49 30 31002 223 +49 30 3927 200 marpe@hhi.de +49 30 54699 363 +49 30 54699 329 hcycon@fhtw-berlin.de +7 812 5343314 petukhov@pdmi.ras.ru |
| Title: | Enhancement to HHI Proposal for ITU-T H.26L |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# 1  Introduction

The proposed new video coding scheme, which was presented at the Monterey meeting (Document Q15-G-36)  has been refined in two directions. First, a variable block/grid size is used in order to improve the quality of the temporal prediction and, second, a more efficient coding of the parameters of the motion model has been realized especially for the case of scenes with partially static background. Coding results demonstrate the benefit of these modifications in comparison to the first version of our proposal. Objectively, a gain of 0.5-1 dB PSNR on the average compared to the Monterey status of our proposal has been obtained, while subjectively the quality appeared to be much approved due to a better prediction and less coding artifacts.

# Fig. 1: Principle of quadtree based control grid segmentation

# 2  Variable-Sized Control Grid

The use of a regular grid of block size 16x16 pels imposes a severe constraint on the motion model in cases where both highly active and (quasi-) stationary regions are given. For a better adaptation to this kind of scenes, we consider an extension of our motion model based on an irregular grid with square blocks of sizes 8x8 and 16x16.

To simplify the discussion, let us first consider a pure image warping prediction. Figure 1 gives an illustration of the warping model based on a grid of variable-sized squares. As usual, the prediction is obtained by relating the grid of the current frame k to a warped grid on the previous frame k-1. However, the irregular grid consists of two kind of vertices. The grid points which are connected to four other grid points are freely moving control points (CPs) as indicated in Fig. 1. The other type of grid points are bound vertices (BVs) forming a T-shaped connection between three other grid points. In contrast to a CP, the motion vector (MV) of a BV is bilinearly interpolated from the MVs of neighboring CPs and hence must not be transmitted. The overall structure of the grid can be described by a quadtree.

The motion estimation process is a hierarchical organized scheme operating on successively refined regular grids of sizes 32x32, 16x16 and 8x8 in a way, which was described in Document Q15-G-36. As a result of this first estimation step, a motion vector field on a fine grid of size 8x8 is obtained. In a second step, CPs on the fine grid which are not grid points on the coarser grid of size 16x16 are examined in order to merge those CPs which are related to homogeneously moving areas. In doing so, we evaluate a costfunction M which, on one hand, measures the local increase in MSE by replacing the actual estimated motion vector mvopt by its bilinearly interpolated motion vector mvip . On the other hand, M accounts for the local variance of the motion vector substitution by means of the squared error of the residual motion vector. If the cost M of a CP is below a certain threshold Tmerge, its motion vector is substituted by its interpolated MV. The choice of the critical merging threshold Tmerge controls the amount of side information which is generated by the motion parameters. In order to obain a fixed ratio of side information and information to represent the residual image (at least in the sense of a first-order approximation), we choose Tmerge  to be linearly dependent on the quantization parameter QP. After finishing the merging phase, each remaining CP on the fine grid is examined in order to decide whether it is a unrestricted CP (with its own MV) or a bound vertex.

The result of this first motion estimation process is a segmentation into 8x8 and 16x16 square blocks where the motion parameters, i.e. the motion vectors, the decision on the predicion type (warping or OBMC) and the interpolation type (bilinear or high quality interpolation) have to be refined. This is done in a final step by using the same type of estimation routines as described in Doc. Q15G36.

# 3  Coding of Motion Parameters

Given the variable-sized control grid, a quadtree code has to be generated in order to inform the decoder about the structure of the motion vector field. In addition to the motion vector of each control point, the flags indicating the prediction and interpolation type for each block must also be encoded. Here we introduce a special zero motion vector block symbol in the case of a block where each of the vertex CPs has the zero motion vector. For this type of block, no additional side information must be transmitted.

# 4  Simulation Results

The proposed and enhanced method has been used for coding simulations using the five recommended QCIF test sequences. We compared the results to those obtained by the previous version of our proposal and to those obtained by the anchor. The related rate distortion graphs are given in Section 5. At the meeting , a D-1 tape demonstration will show some reconstructed sequences for subjective comparisons.

# 5  Rate-Distortion Graphs

## 5.1 Combined Intra- and Inter-Coding

## 5.2 Intra-Coding