| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Ninth Meeting: Red Bank, New Jersey, 19-22 October, 1999 | Document Q15-I-48 Filename: q15i48.doc Generated: 13 Oct. ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | B. G. Haskell, A. Puri, B. Paul, M. R. Civanlar AT&T Labs - Research 100 Schultz Drive Red Bank, NJ 07701 | Tel: Fax: Email: | 732-345-3300 732-345-3033 bgh@research.att.com |
| Title: | A Profile for Cable Video Telephony |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# 1.	Introduction

The H.263 standard provides an important basis for high performance real-time video communications. The original H.263 standard is organized as the baseline standard with 5 optional annexes (Annex D through Annex G). The subsequent version 2 has added 13 optional annexes (Annex H through Annex T) and ongoing version 3 expects to add 3 additional optional annexes (Annex U through Annex W). Furthermore, some annexes contain up to three tools. Collectively this large set of tools, although some subsets of it quite useful, poses a challenge for interoperability of H.263 codecs. While it is true that these annexes are optional and can be negotiated by external signaling (e.g. H.245), due to their large number, it is likely that common mode of operation between codecs really ends up being the baseline mode.  This can have the undesirable effect that key, useful tools capable of providing higher efficiency or other functionalities such as high error resilience or scalability, may not be typically used. Realizing this limitation, Appendix II of the Draft H.263 recommendation lists three interoperability “levels” in the form of combinations of Annexes. There are two concerns with this approach, first, that the interoperability points are not mandatory, and further, the recommended interoperability points need to be made consistent with potential key applications of H.263.

In this document we present a structure for improving the current situation of interoperability with H.263 as well as demonstrating its utility by defining a tools subset of H.263 for addressing the application area of video telephony over cable.

# 2.	Profiles and Levels

To allow economical development of video telephony products as well as to promote interoperability between these products, it is necessary to define subsets of the H.263 standard such that each subset addresses specific classes of applications with similar functional requirements. This can be accomplished via the concept of profiles.

A profile is a well defined subset of the entire bitstream syntax of the H.263 standard. The syntax of a profile corresponds to the tools it supports. Further, since a profile may cover a large range of processing complexity, each profile is partitioned into a number of levels. Each level within a profile refers to constraints on values of parameters in the bitstream. Therefore, a profile and level combination specifies the tools needed for decoding a bitstream as well as an estimate of its decoding complexity.

# Cable  Video Telephony (VT) Profile

Videotelephony over cable is expected to provide a rich set of ‘live’ 2-way video enabled capabilities typically difficult to obtain for home consumers via the internet. The primary difference is in bitrates since home consumers typically connecting to internet via regular modems run into bitrates less than or equal to 56 kbit/s. On the other hand, Cable modems, either, standalone in PC’s or via the set top box promise much higher bitrates for two-way communication.

## Some Key Issues

The H.263 Version 2 Draft  Standard addresses in its scope a whole range of operating

conditions for video codecs, ranging from very low bit rates in the order of tens of kilobits per second to relatively high bit rates of hundreds of kilobits per second with low to stringent error-rates. Moreover, the standard specifies an extensive set of scalability features that can be applied in many different scenarios and several modes of motion compensation with various levels of complexity.  While Cable Video Telephony shares most of the issues that led to the development of the set of Annexes and Tools which form an integral part of the H.263 Draft Recommendation, it nevertheless has its own set of requirements to be met.  It is then desirable to formulate an operating point for Video Telephony over cable that is tailored to the characteristics of IP over DOCSIS(Data Over Cable System Interface Specifications) and use the tools and sub-annexes that are immediately relevant to this application.

More specifically, terminals for Video Telephony over Cable Systems will normally use several hundred kilobits per second in the near term and reach megabits per second in the long term. So we need not address issues related to lower bit rates in this context. The type of errors expected when transmitting over this medium is likely to be mostly packet losses. Hence, we need to include specific error resilience tools to cope with such eventualities. We should expect that some form of quality of service will be implemented in a near future. Exploiting such features requires some of the scalability sub-modes already described in the annexes or currently in discussion. In addition, some or all of the video telephony functionalities may be implemented in future digital set-top boxes with various levels of computing power.

## Tools

#### Necessary Tools

| Annex | Section | Details |
| --- | --- | --- |
| D | D.1 | Motion vectors over picture boundaries |
|  | D.2 | Extension of the motion vector range |
| F | F.2 | Four motion vectors per macroblock |
| I | All | DC and AC prediction and scan patterns, modified quantizer oddification, Intra VLC |
| J | J.3 | Deblocking edge filter |
| K | Submode 1 | Rectangular slice submode |
| N | N.3.2 | Reference picture selection with backchannel as per the VideoMux mode |
| R | Subset of R.2 | Independent segment decoding using slice based video picture segment |
| T | All | Modified quantization mode – modified DQUANT update, altered chrominance quant, modified coefficient range |

Additional Desirable Tools

| Annex | Section | Details |
| --- | --- | --- |
| L | L.4 | Supplemental enhancement information – full picture freeze |
|  | L.14 | Supplemental enhancement information – chromakey information |
| O | O.1.1 | Temporal scalability and its syntax and semantics (spread over several sections) |

## Justification

In the light of the issues raised in Section 3.1, our target is to specify a critical set of tools from H.263 Annexes to achieve optimal performance in video quality, error resilience, implementation complexity and simplified capability exchange. In doing so we need to decouple each useful tool from the Annex where it is specified in the interest of incremental performance-complexity trade-off. By selecting sub-modes having high performance improvement but lower incremental computational cost we reduce overall implementation complexity and increase the likelihood that these beneficial modes would actually be used.

In this context, the selected sections from Annexes D, F, I, J and T address the need for improved picture quality beyond baseline H.263 video coding.  However, the overlapped motion compensation section of annexes is not included in this specification because of its potentially high implementation cost. Similarly,

selected sections from Annexes K, N, R and O allow some level of error resilience, while others are not included on the ground that the incremental performance improvement that would be achieved does not warrant the cost in implementation complexity. Sections L.4 and L.14 from Annex L are included for introducing additional functionalities and O.1.1 introduces some level of scalability.

The use of profiles and levels in this context extends some issues already addressed in the Appendix II of H.263 Version 2 Draft Recommendation to include consideration for simplified capability exchange. This will ensure easier integration with lightweight multimedia communication protocols likely to be used on DOCSIS networks whether they are H.245-like, Media Gateway Control or PacketCable.

# 4.	Potential Solution

We now propose a structure for adding profile/level information to the H.263 standard. A new annex, called Annex X is defined for this purpose. The proposed annex (Annex X) is attached to this document.

Annex X
Specification of Profiles and Levels

(This annex forms an integral part of this Recommendation.)

## X.1	Introduction

This annex describes a means to specify the profile and level information in H.263. The use of this annex is indicated in the PLUSPTYPE field of the picture header. The profile information indicates the combination of tools needed to decode bitstreams compliant to that profile while the level information indicates the complexity of the bitstream.

## X.2	Syntax and Semantics

### X.2.1	Plus PTYPE (PLUSPTYPE) (Variable Length)

A codeword of 12, 20 or 30 bits which is present only if the presence of extended PTYPE is indicated in bits 6-8 of PTYPE.

PLUSPTYPE is comprised of up to three fields from among four fields: UFEP, OPPTYPE, OPPLID and MPPTYPE. There is a semantic restriction that only one out of the two optional fields, OPPTYPE and OPPLID, is allowed at a time. OPPTYPE is present only if UFEP has one particular value. Likewise, OPPLID is only present if UFEP has another particular value.

#### X.2.1.1	Update Full Extended PTYPE (Variable Length)

This section modifies the semantics of Update Full Extended PTYPE (UFEP) to define a reserved code, UFEP=”100” to signal the presence of an 8 bit optional ProfileLevelID (OPPLID). Thus, including the existing definition of UFEP, (H.263 draft, section 5.1.4.1) all values allowed are as follows.

TABLE X.1/H.263

##### Extended definition of UFEP field

| UFEP Code | #### Meaning | #### Total length of PLUSPTYPE |
| --- | --- | --- |
| 000 | Mandatory part (MPPTYPE) of PLUSPTYPE | 12 |
| 001 | Optional part (OPPTYPE) and the Mandatory part (MPPTYPE) of PLUSPTYPE | 30 |
| 010 | Optional Profile and Level ID (OPPLID) and the Mandatory part (MPPTYPE) of PLUSPTYPE | 20 |
| 011 | Reserved | - |
| : | Reserved | - |
| 111 | Reserved | - |

#### X.2.1.2	Optional Profile and Level ID (8-bit)

Further, the 8-bit OPPLID field is subdivided into three subfields,  Escape Structured Profiles (ESP), Profile ID (PID) and  Level ID (LID). The structure of OPPLID is given as follows.

| ESP | PID | LID |
| --- | --- | --- |

FIGURE X.1/H.263

Structure of optional profile and level ID

#### X.2.1.3	Escape Structured Profiles (1-bit)

This is a 1-bit code which when set to ‘1’ indicates use of nonhierarchical profiles. The default value of this bit is ‘0’, indicating the use of hierarchical structured profiles.

#### X.2.1.4	Profile ID (4-bit)

This is a 4-bit code that specifies the profile ID of the bitstream; Table X.2 shows the profiles in a hierarchical structure of profiles that can be signaled by this PID code.

TABLE X.2/H.263

##### Profiles ID Table

| PID Code | #### Meaning |
| --- | --- |
| 0000 | reserved |
| : | reserved |
| 0010 | Cable VT profile |
| : | reserved |
| 1111 | reserved |

#### X.2.1.5	Level ID (3-bit)

This is a 3-bit code that specifies the level ID of the bitstream; Table X.3 shows the levels of a hierarchical structure of levels that can be signaled by this LID code.

TABLE X.3/H.263

##### Level ID Table

| LID Code | #### Level Indicated | #### Level Definition (upper limits) |
| --- | --- | --- |
| 000 | reserved | - |
| 001 | reserved | - |
| 010 | Low | 176 pels/line, 144 lines/frame, 30 frames/s, 352 kbit/s |
| 011 | Main | 352 pels/line, 288 lines/frame, 30 frames/s, 1,408 kbit/s |
| 100 | High | 704 pels/line, 576 lines/frame, 30 frames/s, 5,632 kbit/s |
| 101 | reserved | - |
| 110 | reserved | - |
| 111 | reserved | - |

## X.3	Cable VT Profile

A list of mandatory tools (subject to minor changes) for this profile is shown in Table X.4.

TABLE X.4/H.263

##### Tools (in addition to baseline H.263) in Cable VT Profile

| Annex | Section | Details |
| --- | --- | --- |
| D | D.1 | Motion vectors over picture boundaries |
|  | D.2 | Extension of the motion vector range |
| F | F.2 | Four motion vectors per macroblock |
| I | All | DC and AC prediction and scan patterns, modified quantizer oddification, Intra VLC |
| J | J.3 | Deblocking edge filter |
| K | Submode 1 | Rectangular slice submode |
| N | N.3.2 | Reference picture selection with backchannel as per the VideoMux mode |
| R | Subset of R.2 | Independent segment decoding using slice based video picture segment |
| T | All | Modified quantization mode – modified DQUANT update, altered chrominance quant, modified coefficient range |

In this profile we support two levels only, the Low and the Main level of Table X.3.