| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Ninth Meeting: Red Bank, NJ, 18-22 October, 1999 | Document Q15-I-61 Filename: q15i61.doc Generated: 22 October, 1999 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Stephan Wenger TU Berlin, Sekr. FR 6-3 Franklinstr. 28-29 D-10587 Berlin, Germany | Tel: Fax: Email: | +49-172-300-0813 +49-30-314-25156 stewe@cs.tu-berlin.de |
| Title: | Common conditions for the Internet/H.323 case |  |  |
| Purpose: | Information |  |  |

_____________________________

## Summary

The following set of common conditions was agreed on for the low-latency H.323/Internet communication case during the Monterey meeting, and revised during the Red Bank meeting of Q.15.  The networking environment for systems of this category can be summarized as follows: IP/UDP/RTP/RFC2429 environment for video transport, need for low latency, possibly multicast transport characteristics, and substantial packet loss rates.  The document follows in it’s structure, sequences, and bitrates the common conditions for mobile (Q15-I-60 and predecessors).  The main difference lies in the network oriented parts, specifically in the use of a different network simulator and different error patterns.

In addition to mandating the network simulation software, as introduced in Q15-I-09, several error patterns, bit-rates, frame-rates, and sequences are mandated.  Proponents of new video coding schemes applicable for H.323/Internet communication are to present both subjective and objective results for all the permutations presented below.

## Simulation software

Network simulation software corresponding to the below specification is available in document Q15-I-09.  This software package consists of the following modules:

- Packet loss simulator,

- Code for RFC2429 packetization, and

- Code for RFC2429 de-packetization.

The packet loss simulator accepts an input file with RFC2429 packets (IP/UDP/RTP RFC2429 packets) and an error pattern file.  It then applies packet losses according to the contents of the error pattern file and writes an output file with the resulting RFC2429 packets.

The packetization tools accepts as input H.263+ coded video that was coded according to the specification in TMN11.  The de-packetization tool generated coded video and is able to use redundant picture header information (as defined in RFC2429) and the generation of the TR based on the RTP timestamp.

Since these common conditions cover multicast communication, and to minimize the simulation and testing effort, only uni-directional mechanisms without any feedback are allowed.  For the same reason video is to be coded assuming a fixed average error rate, that is part of the error pattern file.  Later versions of these common conditions might allow the use of updated error information by simulating RTCP receiver reports, or the use of video or protocol oriented feedback mechanisms, such as ARQ, or reference picture selection.

## Sequences

For all sequences below, 4000 coded frames shall be used to avoid the influence of distribution errors in the error patterns (see below).  For smaller sequences, those sequences shall be looped.  It is up to the proponent to ensure that the large content changes resulting from the difference between the last and the first frame of a sequence is coded efficiently.

All sequences shall be coded with a fixed frame-rate and bitrate as indicated below.  Note that the bitrates are chosen conservatively.  The quality measurement scheme (described below) will add some penalty for non-coded frames, or frames that got lost during transmission.  The first frame (which has to be coded as INTRA information) shall be coded in a quality comparable an H.263+ coded picture at a fixed QP of 20.  The transmission of the first frame is subject to the same transmission errors as any other frame.

The bit-rates given in the table below includes the overhead of IP/UDP/RTP/RFC2429 headers.  This has to be taken into account when setting the rate control parameters for the video coding.  In particular will a higher number of packets per picture lead to higher overhead and thus to a lower available bitrate for video.  The overhead per packet results out of the size of the IP/UDP/RTP/RFC2429 headers and is minimum 40 bytes (if no redundant picture header is coded in the RFC2429 header).  To ensure low latency communication it is not allowed to code more than one picture in a single packet.

| Sequence | Size | Frame Rate | Channel Bitrate |
| --- | --- | --- | --- |
| Hall Monitor | QCIF | 10 fps | 32 kbit/s |
| Foreman | QCIF | 7.5 fps | 64 kbit/s |
| Foreman | QCIF | 7.5 fps | 144 kbit/s |
| Paris | CIF | 15 fps | 144 kbit/s |
| Paris | CIF | 15 fps | 384 kbit/s |
| Irene | CIF | 30 fps | 384 kbit/s |

## Error rates and error patterns

Error patterns are available in Q15-I-16r1.  For all sequences, the error free case and 3%, 5%, 10%, and 20% average packet loss rate ahs to be tested, using the respective error pattern files.  Please note, that the 5% packet loss rate pattern in Q15-I-16r1 is different than the one proposed in the initial revision of Q15-I-16, in which a large error burst at the end of the error pattern sequence could be observed.  Details about the generation, file format, and usage of the error patterns are also available in Q15-I-16r1.

## Anchors

Anchor bit-streams will be provided and their availability will be announced on the itu-adv-video reflector.  Details about their generation are provided in TMN11.  They can be summarized as follows: using the high complexity Internet/H.323 mode of TMN11 (including packet error aware rate-distortion optimization, error concealment based on TCON, and Annexes D, F, I, J, and T).

## Quality measurement

Objective quality measurement is performed similarly to the current common conditions for mobile, as defined in Q15-I-60.  In addition, subjective test data is to be provided.  See Q15-I-60 for details.