| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 16-nnn-E October 1999 Original: English |

Question: 15/16

STUDY GROUP 16 – CONTRIBUTION nnn

SOURCE*:	RAPPORTEUR FOR Q.15/16 (Gary SULLIVAN)

TITLE:	DRAFT FOR AMENDMENT 6 TO ITU-T REC. H.262|ISO/IEC 13818-2

________

This contribution proposes a draft for Amendment 6 to the common text ITU-T Recommendation H.262 | ISO/IEC 13818-2 “Information technology - generic coding of moving pictures and associated audio information: video.”

This amendment provides new upper bounds on the number of vertical lines in all profiles at High and High-1440 levels to reflect the agreement of the square pixel common image format for HDTV systems in ITU-R BT.709-3 PART II. The value is changed from 1152 to 1088. In ISO/IEC JTC 1/ SC 29, this is currently in the status of Final Draft Amendment (FDAM) being balloted and will be elevated to Amendment (AMD) in December 1999.

It is proposed that this amendment be decided at the SG 16 meeting in February 2000 to achieve synchronized approval between ITU-T and ISO/IEC JTC 1. It is noted that no change has been made from the text determined in May 1999.

__________________

INTERNATIONAL STANDARD

ITU-T RECOMMENDATION

INFORMATION TECHNOLOGY -- GENERIC CODING OF MOVING PICTURES AND ASSOCIATED AUDIO INFORMATION: VIDEO

AMENDMENT 6

1) Replace table 8-11 in clause 8:

"

Table 8-11 - Upper bounds for sampling density

|  |  | Spatial resolution |  |  |  | Profile |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Level | layer |  |  |  | Simple |  | Main | SNR | Spatial | High | 4:2:2 | Multi |
| High | Enhancement |  | Samples/line Lines/frame Frames/sec |  |  |  | 1920 1088 60 |  |  | 1920 1088 60 | 1920 1088 60 | 1920 1088 60 |
|  | Lower |  | Samples/line Lines/frame Frames/sec |  |  |  | - |  |  | 960 576 30 | - | 1920 1088 60 |
| High-1440 | Enhancement |  | Samples/line Lines/frame Frames/sec |  |  |  | 1440 1088 60 |  | 1440 1088 60 | 1440 1088 60 | - | 1440 1088 60 |
|  | Lower |  | Samples/line Lines/frame Frames/sec |  |  |  | - |  | 720 576 30 | 720 576 30 | - | 1440 1088 60 |
| Main | Enhancement |  | Samples/line Lines/frame Frames/sec |  | 720 576 30 |  | 720 576 30 | 720 576 30 |  | 720 576 30 | 720 608 a) 30 | 720 576 30 |
|  | Lower |  | Samples/line Lines/frame Frames/sec |  | - |  | - | - |  | 352 288 30 | - | 720 576 30 |
| Low | Enhancement |  | Samples/line Lines/frame Frames/sec |  |  |  | 352 288 30 | 352 288 30 |  |  | - | 352 288 30 |
|  | Lower |  | Samples/line Lines/frame Frames/sec |  |  |  | - | - |  |  | - | 352 288 30 |
| In the case of single layer or SNR scaled coding, the limits specified by ‘Enhancement layer’ apply. a) 512 lines/frame for 525/60, 608 lines/frame for 625/50 |  |  |  |  |  |  |  |  |  |  |  |  |

"

__________________